Ref. No.: 276-9/4-2016

Date:  26. 9. 2016

TECHNICAL SPECIFICATION AND REQUIREMENTS

“Tower simulator”

Prepared by ATS automation department

© 2016 Slovenia Control. All rights reserved. No part of this document may be reproduced, transmitted, stored in a retrieval system or translated into any language, in any form, or by any means, without the prior written consent of Slovenia Control.

Table of Contents

1	Scope	2

2	General	2

3	Introduction	2

3.1	Current situation	2

3.2	Scope of work	2

4	Requirements	3

4.1	General requirements	3

4.2	General visualisation requirements (environment)	4

4.2.1	Specific visualisation requirements (airports)	5

4.3	Model behaviour	5

4.4	Exercise creation	6

4.5	Meteorological functionality	8

4.6	Pseudo pilot interface and control of objects	8

4.7	Exercise execution	10

4.8	Storage and maintenance of database, exercises, objects and airspace	10

4.9	Instructor interface and communication with pseudo pilots	11

4.10	Airport lights control	11

4.11	Interaction with FDPS	11

4.12	Interaction with RDPS	11

4.13	Interaction and import of data with/from other instances	12

4.14	Configuration/Customization	12

4.15	Training and documentation	12

4.16	FAT, SAT and acceptance	12

5	Compliance column	13

6	APPENDICES	14

6.1	APPENDIX A	14

6.2	APPENDIX B	16

6.3	APPENDIX C	16

6.4	APPENDIX D	17

6.5	APPENDIX E	18

6.6	APPENDIX F	19

6.7	APPENDIX G	20

6.8	APPENIDX H	21

# Scope

This specification covers the requirements for the tower simulator for Slovenia Control ltd (Contracting Authority).

# General

The following practice has been followed in writing this specification:

- For mandatory elements the operative verb shall is used,

- For recommended elements the operative verb should is used.

Mandatory elements are required, recommended are desired.

NOTES are to be consider as additional explanation of the requirement.

Nomenclature used: supplied system is synonym for TWR simulator (as software) and vice versa.

# Introduction

## Current situation

Slovenia Control uses platform called CATS, originally developed by Swedish company C-ITS, later absorbed by BAE and renamed to DATS. TWR and RDR part are unified in one common piece of software. There is no separate TWR or RDR part. It is up to user to start corresponding configuration, which calls for plug-ins and processes needed for certain type of exercise. Configurations can be defined in admin console. All the pilot positions are equal; each exercise leader (pilot1a, 2a, 3a) can start any configuration defined as long as exercise contains required data. Vega Prime is taking care of the visual part (visualisation) for the TWR configuration. Vega Prime uses Open Flight format, which is not very common and not many people are capable of delivering high definition models or sceneries. DATS has been customised to feed our real equipment and software (FDPS and RDPS) with data in proper format, which is then correctly represented in both instances. DATS also controls time in FDPS, making it possible to start and stop exercise, it sends flight plans in ICAO format, it sends ACT (activation) messages and FDPS forefront (called JETI) is used on pseudo pilot side as activation point for VFR flights and some other action, simulating FDPS activities. DATS sends track data in ASTERIX CAT 62, and sensor status via ASTERIX CAT 63 and 65. Latest upgrade introduced mode S data broadcast from the individual tracks.

Slovenia Control has put together a 270 degree display suite, consisting of 18 55’ narrow bezel LCD monitors installed in letter mode, centrally controlled and paired with video wall control unit, which distributes video signal from computers visualising the view from the tower position.

Real VCS equipment is integral part of the simulator premises. No simulated VCS suite is therefore not part of the requirements nor procurement.

## Scope of work

Slovenia Control ltd is looking for TWR simulator with modern high definition graphics engine, capable of detailed representation of airport environment, immediate surroundings and area within visual range from tower, aircraft and other objects, capable of rendering different weather situations and phenomena, capable of communicating and data exchange with FDPS and RDPS and with realistic model behaviour.

# Requirements

## General requirements

| ID |  | compliant |
| --- | --- | --- |
| GEN0010 | Supplied system shall be TWR simulator with visualisation |  |
| GEN0020 | Supplied system shall be capable of displaying field of view of 270 degrees minimum |  |
| GEN0030 | Supplied system shall have some features already installed, which will be used in RDR simulator. NOTE: Required features are described in paragraph 4.13 |  |
| GEN0040 | Simulator shall have following functionalities and/or positions: - scenario preparation, - environment preparation, - simulator management (can be joined with positions 1 and 2), - pseudo pilot, - instructor, - visualisation NOTE: if joined, positions 1, 2 and 3 must be on separate computer from the rest of the suite. |  |
| GEN0050 | Exercise control shall be centralised NOTE: preferably on pseudo pilot or instructor position |  |
| GEN0060 | Voice Communication System (VCS) shall not be part of system. NOTE: Slovenia Control has own operational VCS in the simulator |  |
| GEN0070 | Exercises shall be database driven. NOTE: exercise are not allowed to be a binary. See para 4.8 NOTE: binary – file containing all needed data. Exercise should access central database (routes, waypoints etc). |  |
| GEN0071 | Any change in central database shall be reflected in exercise. |  |
| GEN0080 | It shall be possible to pause or stop the exercise in any moment, and to resume from that moment |  |
| GEN0081 | It shall be possible to fast forward, stop and pause the exercise at any given moment. |  |
| GEN0090 | It shall be possible to create objects with own performances on ground and in the air |  |
| GEN0100 | It shall be possible to import dataset from external source. See para 4.13 INT0010 NOTE: by import any kind of import (even copy/paste of text) is considered as valid. NOTE2: Slovenia Control can provide data in any desired format. |  |
| GEN0110 | It shall be possible to create own airspace database, containing: - waypoints, - navigational aids, - routes, - maps, - runways and airports, - holding patterns, - missed approaches, - SIDs and STARs |  |
| GEN0120 | It should be possible to create touchdown zones on the runways, where aircraft would randomly touch down for any given runway. |  |
| GEN0130 | There shall be collision detection mechanism installed for objects. |  |
| GEN0131 | Pseudo pilots should be able to switch detection mechanism on or off for individual objects during exercise. |  |
| GEN0132 | Objects with collision detection on shall automatically stop when they enter other object buffer zone. |  |
| GEN0140 | TWR simulator shall have audio effects. NOTE: sound of engines, both jet and internal combustion, sounds of propellers and rotors, sound of vehicles. |  |
| GEN0150 | Simulator shall feature recording of an exercise along with sound effects. |  |
| GEN0151 | Recording shall be saved as separate file. |  |
| GEN0160 | It shall be possible to create custom maps for pseudo pilots. NOTE: custom maps are visible only on pseudo pilot position. |  |
| GEN0161 | It should be possible to add different layers of maps, including bitmaps. |  |
| GEN0170 | It shall be possible to setup individual radar station with radar shadows, range, SIC/SAC address, type of radar. (see Para 4.12 for details) |  |
| GEN0171 | It shall be possible to group radar stations, in order to be added per exercise as a group, rather as individual radar station (unless other solution is in place like radars being part of environment from the start). |  |
| GEN0180 | Radar stations shall have possibility of malfunction control during exercise running |  |
| GEN0190 | Objects on ground shall follow the vector map logic – issuing taxing/driving instructions via waypoints/taxiways must be enough for object to follow the order. Example: XXX123, taxi via A, B, N, HS of RWY30 (ABC123, taxi via taxiway Alpha, Bravo and November, hold short of runway 30). |  |
| GEN0200 | It shall be possible to define the parameters for intercepting ILS, VOR, for Long Visual Approach, Normal Visual Approach, Short Visual Approach |  |

## General visualisation requirements (environment)

| ID |  | compliant |
| --- | --- | --- |
| VIS0010 | Digital Elevation Model (DEM) shall be used as basis for creating the environment |  |
| VIS0020 | High resolution textures shall be applied on top of DEM for areas within visual range of view from tower |  |
| VIS0030 | Visualisation engine shall be capable of at least following visual effects: - Volumetric lighting of environment and objects (real lighting effects on objects), - singular light objects producing light (runway and taxiway lights, object lights including lights on aircraft and flood lights on the airport), - volumetric clouds, fog and smoke - precipitation - strobe and navigational lights on airplanes, - models displayed with big level of detail (LOD) regardless of distance from viewer, - propeller and rotor animation from static to rotating, - realistic undercarriage animation, - realistic ground behaviour of the objects (vehicles on ground, taxiing aircraft), |  |
| VIS0040 | At least following objects shall be affected by the wind force and direction as set in the exercise - windsock - smoke - airplanes - clouds |  |
| VIS0050 | Visualisation engine shall enable rendering of all major cloud families and sub types plus following phaenomena: - shallow fog, - fog, - mist, - thunderstorm with lightning effects, - rain, - snow |  |
| VIS0060 | It shall be possible to create own liveries from the template per any object (including vehicles) |  |
| VIS0070 | It shall be possible to alter viewpoint in order to get proper view from the simulated tower position. |  |
| VIS0080 | Visualisation engine shall enable visualisation of engine fire (smoke, fire or both) |  |
| VIS0090 | Visualisation for the airports shall include high-resolution textures for area at least 6 NM around the airport and all geographical features within visual range around it. |  |
| VIS0100 | Visualisation of the airports should consist of three dimensional objects with realistic high-resolution textures. |  |
| VIS0110 | Tree lines shall include diversity in both type, size and quantity of the trees. |  |
| VIS0120 | It should be possible to setup a group of objects to be part of any exercise. EXAMPLE: group of airplanes on ground, group of vehicles, etc. |  |
| VIS0130 | Strobe lights shall start automatically with engine start |  |
| VIS0140 | There shall be possibility to use simulated binoculars (software or combination of hardware/software). |  |

### Specific visualisation requirements (airports)

| ID |  | compliant |
| --- | --- | --- |
| AVIS0010 | Windsocks shall be interactive (moving according wind speed and direction in the exercise) |  |
| AVIS0020 | Airport bridges should be animated |  |
| AVIS0030 | It shall be possible to ad hoc select pre-set markings or 2D objects and put them on the airport surface per exercise (like closure of taxiway and similar). |  |
| AVIS0040 | Visualisation of following airports shall be provided: LJLJ, LJMB, LJCE, LJPZ |  |
| AVIS0050 | Visualisation of the airports shall include all objects and geographical features within visual range from TWRs in above mentioned airports |  |

## Model behaviour

| ID |  | compliant |
| --- | --- | --- |
| MOD0010 | It shall be possible to automate/script certain actions. EXAMPLE: service vehicles coming to service parked aircraft, buses coming to airplanes. NOTE: Predefined route with certain action sequence can be deemed as automated/scripted action. |  |
| MOD0020 | It should be possible to join more objects into group and to assign them scripted action |  |
| MOD0030 | It should be possible to ad hoc connect follow-me car with any airplane on ground which then leads airplane to the parking stand. |  |
| MOD0040 | It shall be possible to join push-back vehicle with aircraft |  |
| MOD0041 | Joined vehicles manoeuvres should be controlled via tug controls. NOTE: meaning, that direction, speed and turn rates are controlled via tug controls, airplane attached to it follows the tug’s path according own limitations. |  |
| MOD0042 | Individual vehicles joined in a group should retain individual manoeuvring characteristics (for instance towing vehicle and towed aircraft should not move on the as single object, but rather as two separate with two joints – one on tug and the other on the plane). |  |
| MOD0050 | It should be possible to join airplane and glider in a tow. NOTE: same logic as for the ground handling should apply. |  |
| MOD0051 | It should be possible to separate glider from the tow |  |
| MOD0052 | It should be possible to attach a banner to the airplane. |  |
| MOD0060 | It should be possible to join two or more planes into a group, which is controlled as single object, with airplanes still flying individually (pair or squadron) NOTE: same logic as for the ground handling should apply |  |
| MOD0070 | It shall be possible to alter performance of any model (turn rates, speeds etc.) |  |
| MOD0080 | Propellers and rotors shall start turning with engine starts |  |
| MOD0090 | Helicopter landing model shall automatically adjust the speed, level and general behaviour of the helicopter after landing command has been issued and landing spot selected by pilot. |  |
| MOD0100 | Object shall automatically proceed to next waypoint/level as selected from the pre-set route or by pseudo pilot, after take-off or landing command has been issued. EXAMPLE: After take-off turn right to S3, climb to 3500 feet. |  |
| MOD0110 | Gear, strobe and navigational lights, propellers and rotors shall start automatically with single pilot command for start-up or by script command. |  |
| MOD0120 | It shall be possible for pseudo pilots to override gear/engine control by selecting individual failures |  |
| MOD0130 | It should be possible for pseudo pilots to select the following emergency visualisations: fire from individual engine, smoke from individual engine, burning/smoking tires |  |
| MOD0140 | Helicopters shall have capability to start hovering with single command over any point |  |
| MOD0150 | It should be possible to join sling cargo under any helicopter |  |
| MOD0160 | Automatic visual approaches shall calculate minimum distance to fly under pre-set parameters (see GEN0200) in order to automatically land. |  |
| MOD0170 | Runway usage detection should be in place |  |
| MOD0171 | When runway is blocked, landing aircraft should automatically generate report for the pilot about runway blockage. |  |

## Exercise creation

| ID |  | compliant |
| --- | --- | --- |
| EXEC0010 | Exercise shall include simplan (route which object will follow) and flight plan (ICAO flight plan). NOTE: either as separate entities, or simplan with full ICAO flight plan functionality. |  |
| EXEC0020 | Exercise shall enable copy-paste any parts or whole exercise to another exercise |  |
| EXEC0030 | Exercise creation should enable inserting: - notes, - instructions to pilots, |  |
| EXEC0040 | Activation (ACT) messages for IFR flights shall be automatically created in Exercise creation window with following data in accordance with Eurocontrol OLDI standard 4.2. See: https://www.eurocontrol.int/sites/default/files/publication/files/20101216-oldi-spec-v4.2.pdf |  |
| EXEC0050 | It shall be possible to insert pilot messages and instructions for any object in the exercise at any point. |  |
| EXEC0060 | It shall be possible to preview the exercise in exercise creation window at any stage of creation. |  |
| EXEC0070 | It shall be possible to create pre-set scripts for actions, which include objects, their routes and actions |  |
| EXEC0080 | Sim plans shall be visible as full trajectory in the exercise creation window. |  |
| EXEC0090 | It shall be possible to preview the trajectory by manipulating time slider for individual object |  |
| EXEC0100 | It shall be possible to preview whole exercise (trajectory of all objects) by manipulating time slider |  |
| EXEC0110 | Simplan routes shall have the possibility to add events at any given point in trajectory and stored in database |  |
| EXEC0120 | Simplan routes in the exercise shall have the possibility to add events at any given point on trajectory per individual exercise EXAMPLE: STAR with waypoints, on which inserted event is automatically executed: speed 160 kt, descent to, climb to, land [RWY] etc. |  |
| EXEC0121 | List of events shall include at least following items for airborne objects: - climb, - descent, - rate of climb/descent, - intercept radial, - fly heading, - set speed, - land RWY [type of approach] [RWY designator] - enter holding, - orbit (left/right), - joint/brake pair/group, - pause, - 3D effects - Join L/R downwind [RWY] |  |
| EXEC0130 | Trajectories for all objects in exercise shall include waypoints and calculated time over with given speed. NOTE: Unless other way of determining of the position and ETO of track is possible, like moving of the track along the trajectory with time). |  |
| EXEC0140 | It shall be possible to move the object in time along its planned route to see where loss of separation might happen |  |
| EXEC0141 | Whole trajectory of such object shall be automatically adjusted and activation message update. |  |
| EXEC0150 | It shall be possible to select for individual object in the exercise generation if it is mode S capable |  |
| EXEC0160 | Mode S capability shall include following fields: Aircraft identification, Selected FL, vertical rate, speed, heading. |  |
| EXEC0170 | It shall be possible to select individual mode S capabilities for the individual object (see EXEC0160) |  |
| EXEC0171 | By selecting mode S capabilities in exercise creating window, all above mentioned mode S items shall be automatically pre-selected. |  |
| EXEC0180 | It shall be possible to script landing and taxiing on parking position without any need for pseudo pilot intervention |  |
| EXEC0190 | It shall be possible to script start-up, taxiing and take-off from parking position without need for pseudo pilot intervention |  |
| EXEC0200 | When adding aircraft to the exercise, SSR code should be automatically allocated from the bank. |  |
| EXEC0230 | Aircraft performance shall include variations. REMARK: To be reflected in max ROC, speed, service ceiling |  |
| EXEC0231 | Instructor shall be in position to select variation of performance for individual aircraft |  |
| EXEC0240 | It should be possible to create areas of turbulence with severity stages light, light to moderate, moderate and severe. Entering this area pseudo pilots will get notification of encountering turbulence and its severity. NOTE: no actual turbulence needs to be modelled; just volume of airspace is required. |  |
| EXEC0250 | It should be possible to control areas with turbulence from instructor position |  |
| EXEC0260 | It shall be possible to create areas with adverse weather. See MET0100. |  |

## Meteorological functionality

| ID |  | compliant |
| --- | --- | --- |
| MET0010 | TWR simulator shall enable manual creation of weather conditions per exercise |  |
| MET0020 | Weather setup shall be available in order to populate QAM fields accordingly. NOTE: QAM is locally developed software for displaying current weather at particular airport. See APPENDIX C |  |
| MET0030 | Instructors or pseudo pilots shall have possibility to alter pre-set weather conditions during exercise running. |  |
| MET0040 | It shall be possible to manually set the weather conditions or to select dynamic weather pattern. |  |
| MET0050 | It shall be possible to add at least following meteorological phenomena: - fog, - shallow fog, - rain, - snow, - clouds of vertical and horizontal expansion (Cu/Cb and/or Sc or similar). - haze, - thunderstorm, - Wind individually or combined in layers |  |
| MET0060 | It shall be possible to alter weather for the running exercise via instructor console |  |
| MET0080 | It should be possible to add turbulence/icing areas in the exercise including intensity (light, light to moderate, moderate, severe) NOTE: no actual model of turbulence is needed, just notification when pilots enter such area, so they go and report turbulence/icing to the controllers. |  |
| MET0090 | Area with turbulence should be defined by geographical and vertical boundaries for each intensity. NOTE: see above |  |
| MET0100 | It should be possible to create adverse weather areas for RDR exercises when exercise is already running NOTE: only as fixed, flat area for pilot orientation regarding position of Cb clouds, see above. |  |
| MET0110 | Area described in MET0100 should be exported in terms of x,y coordinates to RDD NOTE1: exact method and format are to be discussed among parties. NOTE2: they can be in Asterix 9 format |  |

## Pseudo pilot interface and control of objects

| ID |  | compliant |
| --- | --- | --- |
| PPIL0010 | Pilot view shall be top-down or isometric or both |  |
| PPIL0011 | If both views are available, pilots should be able to switch views during exercise. |  |
| PPIL0020 | Each pilot shall be able to control at least 15 objects or more at any given time |  |
| PPIL0030 | Pilots shall be able to type in commands for particular object NOTE: commands input should follow ATCO’s instructions EXAMPLE: DLH123 TAXI RWY 30 VIA A B C |  |
| PPIL0040 | Internal logic shall be part of text input, which automatically fills in commands for given position of the aircraft (airborne or on ground). |  |
| PPIL0050 | Pilots shall be able to control objects by point and click on waypoints |  |
| PPIL0060 | Pilots shall be able to control objects by assigning them pre-set routes |  |
| PPIL0070 | It shall be possible to direct object to any waypoint in such pre-set route |  |
| PPIL0080 | Pilots shall be able to change pre-programmed trajectory (object route). |  |
| PPIL0090 | Pilots shall be able to select at least following approach types: - Visual approach (short, normal, long) - ILS, - VOR, - NDB L |  |
| PPIL0100 | Pilots shall be able to set next clearance after current one, which will automatically execute after first clearance. EXAMPLE: RWY30 TG RTP (runway 30, touch-and-go with right traffic pattern) |  |
| PPIL0110 | Pilots shall be able to land helicopter on any given spot. |  |
| PPIL0120 | Pilots shall be able to delegate/transfer object under their control to other pseudo pilots which take part of running exercise |  |
| PPIL0130 | Besides minimum set of controls (level, speed, heading, take/off, land…), pseudo pilots shall be able to control objects with following commands as well: - push-back, - switch on/off navigational/anti-collision lights, - raise/lower gear, - Select and follow traffic with subsequent command (land, T/G…) |  |
| PPIL0140 | Pilots shall be able to change transponder code of any aircraft. |  |
| PPIL0150 | Pilots shall be able to select emergency codes. NOTE:7500,7600,7700 |  |
| PPIL0160 | Pilots shall be able to select squawk ident with simulator transmitting identification signal as defined by ICAO. |  |
| PPIL0170 | Pilots shall be able to change Mode S aircraft identification |  |
| PPIL0180 | Pilots shall be able to de-select any of the Mode S transponder data blocks (see EXEC0160) |  |
| PPIL0190 | Pilots shall be able to degrade transponder Mode A, C or parts or complete Mode S data blocks (see EXEC0160) |  |
| PPIL0200 | Pilots should be able to join objects into formations on ground already. NOTE: planes in such formation follow the leader automatically. |  |
| PPIL0210 | Pilots should be able to select pre-set scripts and assign them to any object. Example: script fire vehicles proceed to selected object and extinguish fire assigned object |  |
| PPIL0220 | Messages and instructions for the pilots should be connected to particular aircraft for which the message or instruction is intended. |  |
| PPIL0230 | If message to pseudo pilots is general in nature, it shall be displayed in a way, so it will be difficult for pilots to miss it. |  |
| PPIL0240 | Pilots shall command the starting of the engines (with anti-collision lights automatically switching on/off). |  |
| PPIL0250 | Pilots shall be able to override automatic gear operation by selecting which gears are not fully functional |  |
| PPIL0260 | It should be possible to automate “follow traffic” for any controllable object. NOTE: By selecting which traffic to follow and what action to follow: touch-and-go, land, continue taxiing etc.) |  |
| PPIL0270 | Pseudo pilot’s HMI shall enable view window to be separated from it and displayed on another monitor. |  |
| PPIL0280 | There shall be object list with all objects which are controlled by particular pseudo pilot |  |
| PPIL0290 | Selecting the object in the list shall automatically activate controls for this object. |  |
| PPIL0300 | Pseudo pilots shall be able to see whole object route with waypoints and ETO over each waypoint. |  |
| PPIL0310 | In object control window type performance limits for controlled aircraft shall be visible. NOTE: They shall include max cruise speed in IAS and Mach, max ROD/ROC, service ceiling |  |
| PPIL0320 | When airplane enters pre-defined special, area pilot should be notified (see EXEC0240) |  |
| PPIL0330 | When pseudo pilot selects stand number, aircraft should proceed automatically to that stand and stop according it size. REMARK: regardless if stand is equipped with bridge or not. |  |
| PPIL0340 | When object is no longer under pseudo pilot command (it has been transferred by ATCO), it shall be coasted and continue to fly under last entered commands. |  |
| PPIL0350 | Pseudo pilots shall have the possibility to alter mode S aircraft identification per object |  |

## Exercise execution

| ID |  | compliant |
| --- | --- | --- |
| EXEL0010 | Exercise shall start, pause, stop and rewind from single position |  |
| EXEL0020 | One of the positions in the configuration shall initiate communication with FDPS |  |
| EXEL0030 | Pre-set messages and instructions for the pseudo-pilots should be clearly visible on their HMI. |  |
| EXEL0040 | There shall be possibility to add objects from the database during running exercise |  |
| EXEL0050 | Pilots shall be able to select runway in use other than one set in the exercise |  |
| EXEL0060 | All approach clearances shall automatically adopt for selected runway in use. |  |
| EXEL0070 | Missed approach procedure (MAP) shall automatically execute when selected for current runway in use |  |
| EXEL0080 | It shall be possible to manually change MAP per object |  |

## Storage and maintenance of database, exercises, objects and airspace

| ID |  | compliant |
| --- | --- | --- |
| SOE0010 | The storage subsystem shall be independent from the rest of the simulator. NOTE: Malfunction or disconnection of computers hosting this subsystem will have no influence on the ability of the simulator to execute datasets and exercises. |  |
| SOE0020 | Data shall be stored centrally. |  |
| SOE0030 | Total amount of data shall have no influence on overall performance NOTE: start-up time shouldn’t be affected by database size, CPU load must not be elevated for more than 10 % |  |
| SOE0040 | The amount of items in the exercises and data banks in general (waypoints etc..) shall have no influence on performance, the only limit being only the storage space on disks. |  |
| SOE0050 | It shall be possible to store minimum 20.000 waypoints and 500 routes. |  |
| SOE0060 | Dataset and exercise data shall be in standard format NOTE: preferably in XML |  |
| SOE0070 | Dataset and exercise data shall be protected against modification by running exercise. |  |
| SOE0080 | It shall be possible to install dataset and exercises on other computers outside simulator environment for purpose of exercise creation with no need to be physically present on the simulator premises. |  |

## Instructor interface and communication with pseudo pilots

| ID |  | compliant |
| --- | --- | --- |
| INS0010 | Instructor shall have possibility to communicate and issue orders to pseudo pilots |  |
| INS0020 | Instructor shall have the possibility to alter certain exercise parameters during exercise running (weather, runway lights availability etc.) |  |

## Airport lights control

| ID |  | compliant |
| --- | --- | --- |
| LGT0010 | Runway lights shall be centrally controlled |  |
| LGT0020 | Each airport shall have own customised controls fitting the airport layout. |  |
| LGT0030 | Runway lights control panel shall have own input device (touch panel) |  |
| LGT0040 | It shall be possible to individually select groups of lights (edge, centreline, taxiway centreline, edge etc.) or all on/off |  |

## Interaction with FDPS

| ID |  | compliant |
| --- | --- | --- |
| FDP0010 | TWR simulator shall control the time of the FDPS connected to the simulator NOTE: FDPS functionality is not part of this ORD |  |
| FDP0020 | TWR simulator shall control start/stop/pause/fast forward of the FDPS connected to the simulator NOTE: FDPS functionality is not part of this ORD |  |
| FDP0030 | Simulator shall send following messages to FDPS via AFTN/OLDI gateway or directly to TWR simulator FDPS: ABI, ACT, ICAO flight plan as minimum. NOTE: in accordance with Eurocontrol OLDI standard 4.2 |  |
| FDP0040 | It shall be possible to set parameters for sending ABI and ACT messages (content and time parameters) NOTE: in accordance with Eurocontrol OLDI standard 4.2 |  |

## Interaction with RDPS

| ID |  | compliant |
| --- | --- | --- |
| RDP0010 | All tracks shall be in ASTERIX CAT 62 NOTE: Slovenia Control will provide samples of traffic from tracker to RDPS. |  |
| RDP0020 | Radars in the exercise shall transmit their SIC/SAC to RDPS. |  |
| RDP0030 | Radars in the exercise shall transmit their status to RDPS |  |
| RDP0040 | Simulator tracker gateway shall have the possibility to produce primary and secondary tracks, the latter with A and/or C mode |  |
| RDP0050 | Tracker shall display tracks according aircraft status in the exercise (Mode A/C on/off, mode S on/off) and all possible combinations. |  |
| RDP0060 | Tracks shall be sent via multicast through offline definable port or ip address |  |
| RDP0070 | Tracks shall be able to transmit following mode S data: aircraft identification, FL selected, heading, speed (IAS/M), vertical rate |  |
| RDP0080 | Simulator shall feed the RDPS with simulated weather radar data. NOTE: See MET0100, MET0110 |  |

## Interaction and import of data with/from other instances

| ID |  | compliant |
| --- | --- | --- |
| INT0010 | It should be possible to import offline navigational dataset (waypoints with co-ordinates, routes etc.) from AIXM 4.5 and 5.1 format REMARK: for IAD xml format visit http://www.aixm.aero REMARK: Slovenia Control can prepare data in any form and shape for import (including copy/paste in txt or similar format) |  |
| INT0020 | It should be possible to import flight plans in standard ICAO and/or ADEXP format either in TXT or XML format REMARK: Slovenia Control can prepare data in any form and shape for import (including copy/paste in txt or similar format) |  |
| INT0030 | Simulator shall distribute time over simulator LAN in standard format, recognisable by NTP server. |  |

## Configuration/Customization

| ID |  | compliant |
| --- | --- | --- |
| CONF0010 | It shall be possible to create complete airspace (waypoints, routes etc) |  |
| CONF0020 | It shall be possible to create more than one airspace |  |

## Training and documentation

| ID |  | compliant |
| --- | --- | --- |
| TRA0010 | Training shall include written material – documents (manuals, guiding material) and tuition for users. NOTE: All relevant documentation for approval for use by CAA. |  |
| TRA0020 | Training shall include training for pseudo pilots, instructors, simulator managers and maintenance staff |  |
| TRA0030 | Training shall take place at the Slovenia Control premises. |  |
| TRA0040 | Training shall be organised for maximum 5 Slovenia Control members of the staff |  |
| TRA0050 | Training shall last at least 5 working days. |  |
| TRA0060 | Scope of the training shall be: clean installation of the simulator, management of the simulator software (maintenance, backups), usage of the software (pseudo pilots, exercise preparation, exercise running, object and livery creation). |  |

## FAT, SAT and acceptance

| ID |  | compliant |
| --- | --- | --- |
| ACC0010 | FAT and SAT shall take place at the Slovenia Control premises NOTE: Slovenia Control FDPS and RDPS will be used NOTE: Slovenia Control will be available for intermediate integration testing |  |
| ACC0020 | Supplier shall prepare FAT and SAT protocols and send for the confirmation to Slovenia Control at least 14 days prior to FAT or SAT. |  |
| ACC0030 | FAT and SAT shall be considered as successful if no crashing bugs or errors are discovered |  |
| ACC0040 | If FAT or SAT are not successful, they shall be repeated within 3 weeks after unsuccessful FAT or SAT. |  |

Classification of the bugs and errors:

Crashing: software crashes and/or stops working

Major: software runs, but not according specifications. Work on the simulator is impaired.

Minor: software runs, but not according specifications. Work on the simulator is possible with mitigations.

After successful FAT/SAT, Contracting authority will sign certificate of acceptance prepared by the supplier.

## Warranty period

| ID |  | compliant |
| --- | --- | --- |
| MAI0010 | Supplier shall be available for urgent help during work days between 8 and 16 UTC |  |
| MAI0020 | Response time for urgent help shall be 24 hours after error has been reported |  |
| MAI0030 | Response time for non-urgent help shall be 3 work days after error has been reported |  |
| MAI0040 | During warranty period, all errors and bugs found in the simulator shall be corrected on the expense of the supplier. NOTE: error is malfunctioning of the simulator to perform according to specifications. NOTE: bug is error in code, causing simulator not to perform adequately |  |
| MAI0050 | Slovenia Control shall receive all updates, bug fixes and changes done by the supplier during warranty period, free of charge. |  |

Note: error correction and bug fixes can be installed remotely. Bug and error reports will be stored in Tracker software, owned by Slovenia Control. Supplier will be granted access to the reports.

# Compliance column

The Compliance column in tables shall be filled in properly and shall be enclosed to the bid. The contractor is required to specify for each requirement where in the tender documentation the compliance is described (on the lowest possible sub-chapter level).

Bidders shall designate in the last column of each requirement bloc whether their simulator is compliant or not. Possible entries are:

- Yes

- No

Requirements with should are not mandatory. Requirements with shall are mandatory. Bidders not compliant will be eliminated from the bidding process. Required objects and liveries are not eliminatory elements if missing at the time when bid is submitted. If some have to be modelled, price for modelling should be included in the total price of the bid.

# APPENDICES

## APPENDIX A

List of required objects with liveries

| OBJECT | ICAO TPYE DESIGNATOR | LIVERY (if already created by supplier) |
| --- | --- | --- |
| Airbus A310 | A310 | DHL |
| Airbus A319-100 | A319 | Adria Airways, Air Serbia, Easy Jet |
| Airbus A320-200 | A320 | Adria Airways, Wizzair, Easy Jet, Turkish Airlines, Aegean, Finnair, Tunisair |
| Airbus A330-200 | A332 | Air China |
| ATR 72-500 | AT75 | Air Serbia, |
| Augusta 109 | A109 | Slovenian Police |
| Beechcraft 1900D | B1900D | Blank |
| Beechcraft Baron | B58 | Blank |
| Beechcraft King Air | B20 | blank |
| Bell Textron B412 | B412 | Slovenian Police, Slovenian Air Force |
| Bell Textron Jet Ranger | B06 | Slovenian Police, Slovenian Air Force |
| Boeing 737-300 | B733 | Blank, UPS, TNT |
| Boeing 737-800 | B738 | Turkish Airlines, Aegean |
| Boeing 747-400 | B744 | Air China, JAL |
| Boeing 757-200 | B752 | DHL |
| Boeing 777-300ER | B77W | JAL |
| Boeing Globemaster | C17 | US Air Force |
| Bombardier CRJ200 | CRJ2 | Adria Airways |
| Bombardier CRJ900 | CRJ9 | Adria Airways |
| Bombardier Dash 8-300 | DH8D | LOT |
| British Aerospace 142 | B462 | Blank |
| Cessna 152 | C152 | Blank |
| Cessna 172 | C172 | Blank |
| Cessna 421 | C421 | Blank |
| Cessna Citation XL | C560 | Blank |
| Cirrus SR22 | SR22 | Blank |
| Czech Sport Cruiser | CRUZ | Blank |
| Dassault Falcon 2000 | F2TH | Blank |
| Diamond Katana | DV20 | Blank |
| Diamond Twin Star | DA42 | Blank |
| Embraer ERJ 135 | E135 | Blank |
| Embraer ERJ 190 | E190 | Montenegro Airways, HOP (Air France), blank |
| Eurocopter Couguar | AS32 | Slovenian Air Force |
| Eurocopter AS350 | AS350 | Blank |
| Eurocopter EC120 | EC120 | Blank |
| F-16 | F16 | blank |
| Globemaster | C17 | US Air Force |
| Gulfstream V | GLF5 | Blank |
| Hercules | C130 | Blank |
| Pilatus Porter | PC6 | Slovenian Air Force |
| Pilatus PC9/T6 Texan | PC9 | Slovenian Air Force |
| Piper Super Cub | PA18 | Blank |
| Piper Arrow | PA28R | Blank |
| Piper Chieftan | PA31 | Blank |
| Piper Tomahawk | PA38 | Blank |
| Piper Malibu | PA46 | Blank |
| Pipistrel Virus | PIVI | Blank |
| Robin R-2160 Alpha | R200 | Blank |
| Robinson R22 | R22 | Blank |
| Saab 340 | SF34 | Blank |
| Swearingen Metroliner | SW4A | Blank |
| Turbolet | L410 | Slovenian Air Force |
| Zlin | Z143 | Slovenian Air Force |
| parachute |  |  |
| glider | Blanik |  |
| Ambulance |  |  |
| Bus | Cobus 300 |  |
| Car |  |  |
| Catering truck |  |  |
| Crane |  |  |
| Fire Engine | Rosenbauer Panther |  |
| Fire Engine | generic |  |
| Grader |  |  |
| Police car |  |  |
| Security car |  |  |
| Fuel truck |  |  |
| De-icing truck |  |  |
| Baggage train | Bombelli baggage dolly, Mulag tractor | www.mulag.com |
| Conveyer belt |  | www.tips.si |
| tug |  |  |

## APPENDIX B

Schematics of the current simulator setup

## APPENDIX C

QAM (meteorological information panel used in Slovenia Control)

## APPENDIX D

Overall schematic of the hardware in the simulator on the premises of Slovenia Control

## APPENDIX E

Overall Infrastructure schematic of the current simulator setup at the premises of Slovenia Control

## APPENDIX F

TWR visualisation schematics at the premises of Slovenia Control

## APPENDIX G

Desired Dataflow in the new TWR simulator

## APPENIDX H

Airports to be modelled

LJLJ
http://www.geopedia.si/#T105_x459702.94384765625_y119775.52075195312_s14_b2

https://www.google.com/maps/@46.2293213,14.4556198,1444m/data=!3m1!1e3

LJMB
http://www.geopedia.si/#T105_x553456.0141601562_y148488.2403564453_s15_b2

https://www.google.com/maps/@46.4780556,15.6845694,688m/data=!3m1!1e3

LJPZ
http://www.geopedia.si/#T105_x392138.7813720703_y37350.1748046875_s16_b2

https://www.google.com/maps/@45.4739966,13.6198899,1582m/data=!3m1!1e3

LJCE
https://www.google.com/maps/@45.8951174,15.5286457,1259m/data=!3m1!1e3