|  | REQUEST FOR QUOTATION NO: 49276 |  |
| --- | --- | --- |

| REQUISITION: 8180026 |  |  |
| --- | --- | --- |
|  | END USER | NPP Krško d.o.o. VRBINA 12 8270 KRŠKO SLOVENIA natasa.sagernik@nek.si Attn. of Nataša Sagernik |
|  | DATE: | 13.03.2018 |
|  | PAGES: | 1 |

1. SCOPE Line; NEK Item Number; Part Number; UOM; Quantity; Need by Date; Safety Classification. 1; 415747; EA; 2; 6-Mar-18; SR. VALVE,Type SWING CHECK,Size 2",ASME III, Div.1, Subsection ND,SC3,Seismic Cat. I,Valve ID: 2-C36,End preparation SW,Body Material: CS,Design press: 200 psig,Design temp: 200 deg F,ASME pressure class: 600#,SP-G508A Sh.C36-7,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 2; 415748; EA; 4; 6-Mar-18; SR. VALVE,GATE,Size 2",ASME III, Div.1, Subsection ND, SC3,Seismic Cat. I,Valve ID: 2-G36End preparation: SW,Body Material: CS,Design press: 200 psig,Design temp: 200 degF,ASME pressure class: 600#,SP-G508A Sh.G36-3,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 3; 415749; EA; 5; 6-Mar-18; SR. VALVE,GLOBE,Size 2",ASME III, Div.1, Subsection ND, SC3,Seismic Category I,Valve ID: 2-T36,End preparation: SW,Body Material: CS,Design pressure: 230 psig,Design temperature: 200 deg F,ASME pressure class: 600#,SP-G508A Sh.T36-16,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 4; 415750; EA; 1; 6-Mar-18; SR. VALVE,GLOBE,Size 2",ASME III, Div.1, Subsection ND, SC3,Seismic Cat. I,Valve ID: 2-T36,End prep.: SW,Body Material: CS,Plug Form: Eq. Perc.,Design press: 230 psig,Design temp: 200 deg F,ASME pressure class: 600#,SP-G508A Sh.T36-17,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 5; 415751; EA; 22; 6-Mar-18; SR. VALVE,GLOBE,Size 3/4",ASME III, Div.1, Subsection ND, SC3,Seismic Cat I,Valve ID: 3/4-T36,End preparation: SW,Body Material: CS,Design press: 230 psig,Design temp: 200 deg F,ASME pressure class: 600#,SP-G508A Sh.T36-18,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 6; 415752; EA; 1; 6-Mar-18; SR. VALVE,GLOBE,Size 2",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 2-T76,End preparation: SW,Body Material: 316 SS,Design press: 600 psig,Design temp: 400 deg F,ASME pressure class: 600,SP-G508A Sh.T76-5,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 7; 415753; EA; 53; 6-Mar-18; SR. VALVE,GLOBE,Size 3/4",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 3/4-T76,End preparation: SW,Body Material: 316 SS,Design press: 600 psig,Design temp: 400 deg F,ASME pressure class: 600,SP-G508A Sh.T76-6,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 8; 415754; EA; 1; 6-Mar-18; SR. VALVE,GLOBE,Size 2",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 2-T76,End preparation: SW,Body Material: 316 SS,Design press: 600 psig,Design temp: 400 deg F,ASME pressure class: 600,SP-G508A Sh.T76-7,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 9; 415755; EA; 6; 6-Mar-18; SR. VALVE,GATE,Size 6",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 6-G79,End preparation: BW,Body Material: 316 SS,Design press: 600 psig,Design temp: 400 deg F,ASME pressure class: 400,SP-G508A Sh.G79-1,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 10; 415756; EA; 4; 6-Mar-18; SR. VALVE,GATE,Size 8",ASME III, Div.1, Subsection ND, SC3,Seismic Cat. I,Valve ID: 8-G36,End preparation: BW,Body Material: CS,Design press: 230 psig,Design temp: 200 deg F,ASME pressure class: 600#,SP-G508A Sh.G36-4,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 11; 415757; EA; 1; 6-Mar-18; SR. VALVE,GATE,Size 8",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 8-GG76,End preparation: BW,Body Material: 316 SS,Design press: 600 psig,Design temp: 400 deg F,ASME pressure class: 600#,SP-G508A Sh.GG76-1,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 12; 415758; EA; 1; 6-Mar-18; SR. VALVE,BUTTERFLY,Size 8",ASME III, Div.1, Subsection NC, SC2,Seismic Cat. I,Valve ID: 8-X79BP,End preparation: BW,Body Material: 316 SS,Design pressure: 600 psig,Design temp: 400 deg F,ASME pressure class: 400#,SP-G508A Sh.X79BP-1,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf). 13; 415759; EA; 1; 6-Mar-18; SR. VALVE,RELIEF,SIZE: 3/4" / 1", ASME III, Div.1, Subsection ND, SC 3,Seismic Cat. I,Valve ID: RV36SWB,End preparation: FLANGED,Body Material: CS,Design press: 220psig,Design temp: 200 deg F,ASME pressure class: 600,SP-G508A Sh.RV36SWB-1,,SC:SR. Item shall be supplied in accordance with the following TQRs: A00, A02, A07, B00, B01, B04, B06, B07, B08, C00, C01, D00, F00, F05, F50, F51, F52, F521, F53, F55, F57, F59, F60, F90, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G22 - NONCONFORMANCE REPORTS G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) G34 - RADIOGRAPHIC (RT) TEST RECORD G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT G48 - HYDRO PRESSURE TEST RECORD G491 - SEAT LEAKAGE TEST G51 - PERFORMANCE/FUNCTIONAL TEST RECORD G54 - HEAT TREATMENT RECORDS OR REPORTS G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION G60 - ASME CODE MANUFACTURER'S DATA REPORT G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL) G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD G83 - DESIGN / STRESS REPORT G84 - SEISMIC ANALYSIS REPORT X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION X04 - ASME CODE REQUIREMENTS Attachments: SP-G508A r1_for mod. 1029-RH-L.pdf (SP-G508A r1_for mod. 1029-RH-L.pdf).

2. GENERAL NOTES N/A

3. TECHNICAL AND QUALITY REQUIREMENTS IMPORTANT: The quotation has to include Statement of Compliance with the following Technical and Quality Requirements. If any of these requirements could not be fulfilled, state the explanation

A00 - GENERAL REQUIREMENTS ALL REQUIRED DOCUMENTS SHALL BE SUPPLIED IN ENGLISH OR ENGLISH/ MULTILINGUAL. A02 - ORDER ENTRY REQUIREMENTS THE SALES OFFICE AND/OR IMPORTER RECEIVING THIS PURCHASE ORDER SHALL TRANSMIT ALL NEK PURCHASE ORDER REQUIREMENTS TO THE EQUIPMENT MANUFACTURER.. A07 - NEW ITEMS ALL ITEM(S) PROVIDED ON THIS PURCHASE ORDER SHALL BE SUPPLIED IN THE NEW CONDITION (NOT USED OR REFURBISHED IN ANY WAY)....

B00 - QUALITY ASSURANCE REQUIREMENTS B01 - QUALITY ASSURANCE PROGRAM ALL SERVICES AND ITEMS SHALL BE PERFORMED/PROVIDED IN ACCORDANCE WITH A QUALITY ASSURANCE PROGRAM THAT COMPLIES WITH ANSI N45.2-1972, 10CFR50 APPENDIX B OR EQUIVALENT AND HAS BEEN PREVIOUSLY EVALUATED AND ACCEPTED BY NEK. THIS INCLUDES ITEMS EXCLUDED FROM ASME CODE REQUIREMENTS.. B04 - SUPPLIER QA MANUAL CONTROLLED DISTRIBUTION IF A CONTROLLED COPY OF THE LATEST REVISION OF THE SUPPLIER'S QUALITY ASSURANCE PROGRAM MANUAL HAS NOT BEEN PREVIOUSLY SENT TO NEK, SUBMIT A COPY OF THIS DOCUMENT TO: DARKO KAVŠEK, QUALITY & NUCLEAR OVERSIGHT DIRECTOR tel.:++386 748 02 429 E-mail:darko.kavsek@nek.si IF A REVISION TO THE SUPPLIER'S QUALITY ASSURANCE MANUAL IS MADE DURING THE CONDUCT OF WORK ASSOCIATED WITH THIS ORDER, A COPY OF THE REVISION SHALL BE SUBMITTED TO THE ABOVE ADDRESS PRIOR TO IMPLEMENTATION OF THE CHANGE.. B06 - 10CFR21 REPORTING THIS ORDER IS SUBJECT THE PROVISIONS OF THE TITLE 10 OF THE U.S. CODE OF FEDERAL REGULATIONS PART 21, REPORTING OF DEFECTS AND NONCOMPLIANCE, REPORTING PURSUANT TO 10CFR21 SHALL BE MADE TO: ZVONIMIR ZEC , SUPERINTENDENT ENGINEERING SUPPORT tel:++386 748 02 262 E-MAIL: zvonimir.zec@nek.si. B07 - RIGHTS OF ACCESS NUCLEAR POWER PLANT KRSKO (NEK)) SHALL HAVE THE RIGHT OF ACCESS TO THE SUPPLIER'S AND ANY SUBTIER SUPPLIER'S FACILITIES AND RECORDS FOR INSPECTION OR AUDIT BY NEK , THEIR DESIGNATED REPRESENTATIVE AND/OR OTHER PARTIES AUTHORIZED BY NEK. THIS SHALL INCLUDE, BUT IS NOT LIMITED TO, THE RIGHT TO AUDIT MATERIAL, TEST, INSPECTION SERVICES AND QUALITY RECORDS; MAKE SURVEILLANCE VISITS DURING MANUFACTURING; AND WITNESS TESTS TO THE EXTENT NEK DEEMS NECESSARY TO ASSURE THAT WORK IS BEING PERFORMED IN ACCORDANCE WITH ALL PRODUCT DESIGN AND MANUFACTURING REQUIREMENTS. NEK WILL MAKE ARRANGMENT FOR ANY SUBTIER AUDIT OR INSPECTION THROUGH SUPPLIER. SUPPLIER WILL HAVE OPTION TO ACCOMPANY NEK DURING ANY SUB-SUPPLIER AUDITS OR INSPECTIONS. INSPECTIONS OR EXAMINATIONS PERFORMED BY NEK, OR ITS DESIGNATED REPRESENTATIVES, DO NOT RELIEVE THE SUPPLIER OF ITS RESPONSIBILITY TO MEET THE REQUIREMENTS OF THIS PURCHASE ORDER.. B08 - RETENTION OF RECORDS A RECORD SYSTEM (RETENTION TIME FOR LIFETIME AND NON-PERMANENT RECORDS) SHALL BE ESTABLISHED AND MAINTAINED BY THE SUPPLIER THAT PROVIDES DOCUMENTARY EVIDENCE OF THE QUALITY OF THE ITEMS AND ACTIVITIES AFFECTING QUALITY. THESE QA RECORDS SHALL INCLUDE RESULTS OF REVIEWS, INSPECTIONS,TESTS, AUDITS, MONITORING OF WORK PERFORMANCE AND MATERIAL ANALYSIS. RECORDS SHALL, AS A MINIMUM, IDENTIFY THE INSPECTOR OR DATA RECORDER, DATE INSPECTION WAS PERFORMED, TYPE OF OBSERVATION, PROCEDURES USED, RESULTS, ACCEPTABILITY, AND ACTION TAKEN FOR DEFICIENCIES NOTED. ADDITIONAL RECORDS OF SUPPORTING DATA SHALL ALSO BE MAINTAINED. ALL QUALITY VERIFICATION RECORDS, PROCEDURES AND QUALIFICATIONS SHALL BE IDENTIFIABLE TO THE ITEM OR ACTIVITY INVOLVED. THESE RECORDS SHALL BE RETRIVEABLE AND AVAILABLE FOR EXAMINATION. THE CLASSIFICATION AND RETENTION TIME OF LIFETIME AND NON-PERMANENT RECORDS SHALL BE IN COMPLIANCE WITH THE REQUIREMENTS OF USNRC REGULATORY GUIDE 1.28, REV. 3-1985. THE SUPPLIER MAY DISPOSE OF NON-PERMANENT RECORDS AFTER THE REQUIRED RETENTION PERIOD. IF THE SUPPLIER GOES OUT OF BUSINESS BEFORE THE END OF THE RETENTION PERIOD OF THESE RECORDS, THE SUPPLIER WILL SEND ONE COPY OF THESE RECORDS TO THE NEK IMPORT DEPARTMENT SUPERINTENDENT....

C00 - NONCONFORMANCE AND DEVIATION C01 - NO SUBSTITUTIONS THE SUPPLIER SHALL NOT SUBSTITUTE OTHER ITEMS FOR THE ITEMS REQUESTED WITHOUT SPECIFIC WRITTEN APPROVAL OF NEK PRIOR TO SHIPMENT. IF THE SUPPLIER IDENTIFIES CHANGES,NONCONFORMANCES, OR SEEKS WAIVERS FROM OTHER REQUIREMENTS OF THIS PURCHASE ORDER, THE SUPPLIER SHALL DESCRIBE SUCH CONDITIONS AND THIS INFORMATION SHALL BE TRANSMITTED, IN WRITING, TO NEK IMPORT DEPARTMENT SUPERINTENDENT. IF THE REQUESTED INFORMATION IS APPROVED BY NEK, THE SUPPLIER SHALL INCLUDE AN APPROVED COPY OF THE INFORMATION STATEMENT WITH THE ITEMS SHIPPED. THE SUPPLIER SHALL IDENTIFY ANY CHANGE MADE TO UPGRADE ANY ITEM ON THIS PURCHASE ORDER AS A RESULT OF USNRC CORRESPONDENCE (SUCH AS INFORMATION NOTICES, BULLETINS AND ADVISORY LETTERS). CHANGES AS PART OF THE SUPPLIER'S PRODUCT IMPROVEMENT PROGRAM SHALL ALSO BE IDENTIFIED AND TRANSMITTED IN WRITTING TO THE NEK IMPORT DEPARTMENT SUPERINTENDENT FOR APPROVAL....

D00 - ASME CODE ITEM.

F00 - SHIPPING, HANDLING, MARKING AND STORAGE INSTRUCTIONS F05 - HANDLING, PACKAGING AND SHIPPING SUPPLIER SHALL PROVIDE PACKAGING AND SHIPPING METHODS FOR PROTECTION FROM THE EFFECTS OF TEMPERATURE EXTREMES, HUMIDITY AND IN TRANSIT SHOCKS AND JARRING (CAPS AND PLUGS SHALL BE USED TO SEAL THE OPENINGS AND TO PROTECT THREADS AND WELD END PREPARATIONS).. F50 - ITEM MARKING REQUIREMENTS ALL ITEMS ARE TO BE PACKAGED INDIVIDUALLY AND IDENTIFIED WITH THE SPECIFIC PART NUMBER, OR ALL ITEMS OF A GIVEN PART NUMBER TO BE PACKAGED TOGETHER AND IDENTIFIED WITH THE SPECIFIC PART NUMBER AND THE FOLLOWING:. F51 - THE PURCHASE ORDER NUMBER. F52 - NEK ITEM NUMBER. F521 - PART/ CATALOG NO.. F53 - ASME CODE STAMP. F55 - HEAT NO./BATCH NO./LOT NO./CODE. F57 - MANUFACTURER'S NAME. F59 - ITEM SERIAL NUMBER. F60 - MATERIAL DESCRIPTION OR COMPOSITION. F90 - PACKING SLIPS ARE TO BE SHIPPED WITH ORDER...

G00 - SUPPLIER DOCUMENTATION REQUIREMENTS ALL REQUIRED DOCUMENTS SHALL BE SUPPLIED IN ORIGINAL HARD COPY OR ELECTRONIC FORM DEFINED BY SUPPLIER QA PROGRAM AND ACCEPTED BY NEK. G01 - DOCUMENT TRACEABILITY ALL REQUIRED DOCUMENTS SHALL BE TRACEABLE TO THE NEK PURCHASE ORDER. DOCUMENTS SUPPLIED WHICH ARE ASSOCIATED WITH SPECIFIC ITEMS ON THE ORDER, SUCH AS TEST REPORTS, SHALL BE IDENTIFIED IN ORDER TO ESTABLISH TRACEABILITY TO THAT ITEM OR GROUP OF ITEMS.. G02 - MATERIAL AND DOCUMENTATION SHIPPING MATERIAL AND ALL CERTIFICATIONS OR ACCOMPANYING DOCUMENTATION SUPPLIED UNDER THIS ORDER SHALL BE DIRECTLY SHIPPED FROM THE SUPPLIER/MANUFACTURER TO NEK. THE DISTRIBUTOR SHALL NOT TAKE POSSESSION OF MATERIAL OR DOCUMENTATION.. G03 - DOCUMENTATION DIRECTED ADDRESS DOCUMENTS SUBMITTED PRIOR TO SHIPMENT OR SUBSEQUENT TO SHIPMENT INCLUDING REVISIONS OF THESE DOCUMENTS, SHALL BE DIRECTED TO THE PURCHASING DEPARTMENT. EVERY PAGE OF EACH DOCUMENT SHALL BE TRACEABLE TO THE NEK'S PURCHASE ORDER AND APPLICABLE ITEM(S) NUMBER.. G22 - NONCONFORMANCE REPORTS THE SUPPLIER SHALL PROVIDE A COPY OF ALL NONCONFORMANCE REPORTS AS REPAIR OR USE-AS-IS DISPOSITION OF MATERIALS DURING THE MANUFACTURING PROCESS TO UTILITIES ONLY IN CASES THAT AFFECT ENVIRONMENTAL QUALIFICATION, INTERCHANGEABILITY, DESIGN OR OPERATING PARAMETERS GENERATED DURING MANUFACTURE OF PROCESSING OF THIS ORDER. THIS REPORT SHALL INCLUDE TECHNICAL JUSTIFICATION FOR NONCONFORMANCE DISPOSITIONS. ALL DISPOSITIONS WHICH DO NOT RETURN AN ITEM TO THE CONDITIONS STATED IN AN APPROVED DRAWING OR SPECIFICATION SHALL BE APPROVED BY NEK PRIOR TO THE SHIPMENT OF THE AFFECTED ITEM.. G30 - STATEMENT OF APPROVED CHANGE, NONCONFORMANCE OR WAIVER. G31 - STATEMENT OF CODE (ASME-II OR III) RECONCILIATION (IF REQUIRED) THE RECONCILIATION REPORT TO THE ORIGINAL CODE EDITION AND ADDENDA IS REQUESTED TO BE SUBMITTED FOR REVIEW PRIOR TO MACHINING OF PART. COMPONENT/ PART SHALL MEET THE LATEST ASME CODE EDITION AND ADDENDA APPROVED BY NRC NUCLEAR REGULATORY COMMISSION) AS STATED IN 10 CFR 50.55A.. G34 - RADIOGRAPHIC (RT) TEST RECORD. G36 - LIQUID PENETRANT (LP) TEST RECORD OR REPORT. G48 - HYDRO PRESSURE TEST RECORD. G491 - SEAT LEAKAGE TEST. G51 - PERFORMANCE/FUNCTIONAL TEST RECORD. G54 - HEAT TREATMENT RECORDS OR REPORTS. G55 - CERTIFIED MATERIAL TEST RECORDS-MECHANICAL PROPERTIES. G56 - CERTIFIED MATERIAL TEST RECORDS-CHEMICAL COMPOSITION. G60 - ASME CODE MANUFACTURER'S DATA REPORT. G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) THE DOCUMENT SHOULD INCLUDE THE FOLLOWING NPP KRŠKO SPECIFIC DATA: NPP KRŠKO PURCHASE ORDER NUMBER, NPP KRŠKO TECH.(EQUIPM.)SPEC. NUMBER, EQUIPMENT ID (TAG)NUMBER.. G69 - FINAL INSPECTION REPORT (VISUAL, DIMENSIONAL). G82 - MINIMUM WALL THICKNESS MEASUREMENT RECORD. G83 - DESIGN / STRESS REPORT THE DOCUMENT SHOULD INCLUDE THE FOLLOWING NEK SPECIFIC DATA: NPP KRŠKO PURCHASE ORDER NUMBER, NPP KRŠKO TECH.(EQUIPM.)SPEC. NUMBER, EQUIPMENT ID (TAG)NUMBER.. G84 - SEISMIC ANALYSIS REPORT THE DOCUMENT SHOULD INCLUDE THE FOLLOWING NEK SPECIFIC DATA: NPP KRŠKO PURCHASE ORDER NUMBER, NPP KRŠKO TECH.(EQUIPM.)SPEC. NUMBER, EQUIPMENT ID (TAG)NUMBER.. X002 - CERTIFICATE OF COMPLIANCE WITH TECHNICAL SPECIFICATION SUPPLIER SHALL CERTIFY THAT THE SUPPLIED VALVES ARE IN COMPLIANCE WITH TECHNICAL SPECIFICATION SP-G508A-536633-00026, REV.1.. For item: 415755 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 2 CODE EDITION: 1971 ADDENDA:WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415757 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 2 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415750,415756 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 3 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415753 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 2 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415752,415754 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 2 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415747 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 3 CODE EDITION:1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415759 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 3 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415748 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 3 CODE EDITION: 1971 ADDENDA: WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415758 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 2 CODE EDITION: 1971 ADDENDA:WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED.. For item: 415749,415751 X04 - ASME CODE REQUIREMENTS ASME SECTION III ITEMS SHALL MEET: CLASS 3 CODE EDITION: 1971 ADDENDA:WINTER 1972 MATERIAL FOR ITEMS GOVERNED BY THE RULES OF ASME SECTION III MAY BE SUPPLIED TO A LATER CODE EDITION OR ADDENDA PROVIDED THAT ALL THE REQUIREMENTS OF THE ORIGINAL CODE EDITION AND ADDENDA OF ASME SECTION II AND III AND NEK SPECIFIED CODE CASE(S) ARE MET AND THE MATERIAL IS CERTIFIED ACCORDINGLY. IN ADDITION, THE SUPPLIER SHALL PROVIDE NEK WITH A STATEMENT OF CODE RECONCILIATION, IF REQUIRED....