|  | REQUEST FOR QUOTATION NO: 59295 |  |
| --- | --- | --- |

| REQUISITION: 8182253 |  |  |
| --- | --- | --- |
|  |  | , |
|  | END USER | NPP Krško d.o.o. VRBINA 12 8270 KRŠKO SLOVENIA natasa.sagernik@nek.si Attn. of Nataša Sagernik |
|  | DATE: | 24.10.2018 |
|  | PAGES: | 1 |

1. SCOPE Line; NEK Item Number; Part Number; UOM; Quantity; Need by Date; Safety Classification. 1; 417337; EA; 12; 16-Dec-18; NSR. VALVE;GLOBE;1/2" NPT;Y-TYPE;CLASS 1500;316SS BODY;GRAFOIL PCKG;TYPE:1/2-T58-1;SP-G332 1/2-T58-1;,SC:NSR. Item shall be supplied in accordance with the following TQRs: A00, A07, F00, F05, F30, F31, F32, F33, F34, F40, F50, F51, F52, F56, F57, F61, G00, G01, G02, G03 The following documents and/or records shall be submitted with the above item: G04 - THE CERTIFICATE OF CONFORMANCE G48 - HYDRO PRESSURE TEST RECORD G49 - LEAKAGE TEST RECORD G62 - ASSEMBLY DRAWING (1 REPRODUCIBLE + 3 COPIES) G63 - BILL OF MATERIAL G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) G80 - MATERIAL CERTIFICATE (CHEMICAL COMPOSITION AND MECHANICAL PROPERTIES).

2. TECHNICAL AND QUALITY REQUIREMENTS IMPORTANT: The quotation has to include Statement of Compliance with the following Technical and Quality Requirements. If any of these requirements could not be fulfilled, state the explanation

A00 - GENERAL REQUIREMENTS ALL REQUIRED DOCUMENTS SHALL BE SUPPLIED IN ENGLISH OR ENGLISH/ MULTILINGUAL. A07 - NEW ITEMS ALL ITEM(S) PROVIDED ON THIS PURCHASE ORDER SHALL BE SUPPLIED IN THE NEW CONDITION (NOT USED OR REFURBISHED IN ANY WAY)....

F00 - SHIPPING, HANDLING, MARKING AND STORAGE INSTRUCTIONS F05 - HANDLING, PACKAGING AND SHIPPING SUPPLIER SHALL PROVIDE PACKAGING AND SHIPPING METHODS FOR PROTECTION FROM THE EFFECTS OF TEMPERATURE EXTREMES, HUMIDITY AND IN TRANSIT SHOCKS AND JARRING (CAPS AND PLUGS SHALL BE USED TO SEAL THE OPENINGS AND TO PROTECT THREADS AND WELD END PREPARATIONS).. F30 - CONTAINER MARKING REQUIREMENTS SHIPPING CONTAINERS OR CARTONS ARE TO BE CLEARLY MARKED OR TAGGED WITH:. F31 - THE PURCHASE ORDER NUMBER. F32 - SHORT DESCRIPTION. F33 - PART NUMBER. F34 - NEK ITEM NUMBER. F40 - SUPPLIER'S NAME. F50 - ITEM MARKING REQUIREMENTS ALL ITEMS ARE TO BE PACKAGED INDIVIDUALLY AND IDENTIFIED WITH THE SPECIFIC PART NUMBER, OR ALL ITEMS OF A GIVEN PART NUMBER TO BE PACKAGED TOGETHER AND IDENTIFIED WITH THE SPECIFIC PART NUMBER AND THE FOLLOWING:. F51 - THE PURCHASE ORDER NUMBER. F52 - NEK ITEM NUMBER. F56 - RATING. F57 - MANUFACTURER'S NAME. F61 - SIZE...

G00 - SUPPLIER DOCUMENTATION REQUIREMENTS ALL REQUIRED DOCUMENTS SHALL BE SUPPLIED IN ORIGINAL HARD COPY OR ELECTRONIC FORM DEFINED BY SUPPLIER QA PROGRAM AND ACCEPTED BY NEK. G01 - DOCUMENT TRACEABILITY ALL REQUIRED DOCUMENTS SHALL BE TRACEABLE TO THE NEK PURCHASE ORDER. DOCUMENTS SUPPLIED WHICH ARE ASSOCIATED WITH SPECIFIC ITEMS ON THE ORDER, SUCH AS TEST REPORTS, SHALL BE IDENTIFIED IN ORDER TO ESTABLISH TRACEABILITY TO THAT ITEM OR GROUP OF ITEMS.. G02 - MATERIAL AND DOCUMENTATION SHIPPING MATERIAL AND ALL CERTIFICATIONS OR ACCOMPANYING DOCUMENTATION SUPPLIED UNDER THIS ORDER SHALL BE DIRECTLY SHIPPED FROM THE SUPPLIER/MANUFACTURER TO NEK. THE DISTRIBUTOR SHALL NOT TAKE POSSESSION OF MATERIAL OR DOCUMENTATION.. G03 - DOCUMENTATION DIRECTED ADDRESS DOCUMENTS SUBMITTED PRIOR TO SHIPMENT OR SUBSEQUENT TO SHIPMENT INCLUDING REVISIONS OF THESE DOCUMENTS, SHALL BE DIRECTED TO THE PURCHASING DEPARTMENT. EVERY PAGE OF EACH DOCUMENT SHALL BE TRACEABLE TO THE NEK'S PURCHASE ORDER AND APPLICABLE ITEM(S) NUMBER.. G04 - THE CERTIFICATE OF CONFORMANCE THE CERTIFICATE OF CONFORMANCE SHALL SATISFY THE FOLLOWING CRITERIA: 1. IDENTIFY THE PURCHASE ORDER NUMBER, INCLUDING ITEM NUMBERS AND CHANGE ORDER NUMBER APPLICABLE TO THE ITEM BEING CERTIFIED. 2. A PERSON IDENTIFIED IN THE SUPPLIER'S QUALITY ASSURANCE PROGRAM DESCRIPTION AS RESPONSIBLE FOR CERTIFICATION SHALL ATTEST TO THE CERTIFICATE BY SIGNATURE, TITLE AND DATE OF SIGNING. 3. NEK RESERVES THE RIGHT TO DETERMINE THE VALIDITY OF THE CERTIFICATE OF CONFORMANCE BY AN AUDIT OF THE SUPPLIER OR BY AN INSPECTION OR TEST OF THE ITEM(S).. G48 - HYDRO PRESSURE TEST RECORD. G49 - LEAKAGE TEST RECORD. G62 - ASSEMBLY DRAWING (1 REPRODUCIBLE + 3 COPIES) THE DOCUMENT SHOULD INCLUDE THE FOLLOWING NEK SPECIFIC DATA: NPP KRŠKO PURCHASE ORDER NUMBER, NPP KRŠKO TECH.(EQUIPM.)SPEC. NUMBER, EQUIPMENT ID (TAG)NUMBER.. G63 - BILL OF MATERIAL BILL OF MATERIAL SHALL INCLUDE ITEM NUMBER, RECOMMENDED SPARE PART, ITEM DESCRIPTION, UANTITY, ITEM PART NUMBER, MATERIAL SPECIFICATION AND ITEMS DESIGNATION: ASME PRESSURE RETAINING PART / SAFETY-RELATED / NONSAFETY- RELATED. THE BILL OF MATERIAL SHOULD BE SENT AS A PART OF VENDOR DRAWING OR AS A SEPARATE DOCUMENT WITH THE APPLICABLE DOCUMENT NUMBER, SHEET NO. AND REVISION INCLUDING THE REFERENCE VENDOR DRAWING NUMBER/ SHEET/ REVISION AND NEK PO NUMBER.. G64 - OPERATION & MAINTENANCE MANUAL (2 COPIES) THE DOCUMENT SHOULD INCLUDE THE FOLLOWING NPP KRŠKO SPECIFIC DATA: NPP KRŠKO PURCHASE ORDER NUMBER, NPP KRŠKO TECH.(EQUIPM.)SPEC. NUMBER, EQUIPMENT ID (TAG)NUMBER.. G80 - MATERIAL CERTIFICATE (CHEMICAL COMPOSITION AND MECHANICAL PROPERTIES)...

3. ISO 14001 AND BS OHSAS 18001:2007 CERTIFICATIONS Please, put in your quotation corresponding statements related to your company ISO 14001 and BS OHSAS 18001:2007 certifications: - We have ISO 14001 qualification and apply it adequately - (please, enclose a copy of the ISO certificate). - We are in the initial process of implementing ISO 14001 - (please, quote expected date of your company ISO certification). - We do not have ISO 14001 qualification. and - We have BS OHSAS 18001:2007 and apply it adequately - (please, enclose a copy of the certificate). - We are in the initial process of implementing BS OHSAS 18001:2007 - (please, quote expected date of your company certification). - We do not have BS OHSAS 18001:2007.

4. COMMERCIAL TERMS AND CONDITIONS Please provide your best offer for delivery of items specified above, taking into account the above Technical and Quality Requirements and below listed Commercial Terms and Conditions:

4.1 PRICE Please quote unit and total prices.

4.2 PAYMENT TERMS 30 DAYS after receipt of goods in NPP Krško

4.3 DELIVERY TERMS DAP - NPP Krško