Project title: Upgrading National Research Infrastructures – RIUM

COMPLIANCE DECLARATION WITH MINIMUM TECHNICAL REQUIREMENTS OF THE OFFERED EQUIPMENT

Name of the tenderer:

Address of the tenderer:

Subject of the public procurement: Drive test bench system for hardware simulation of interaction between a drive/source and the power grid

Designation of the public procurement: 302/54 – RIUM25-FERI01/2020

INSTRUCTIONS:

- This Annex No. 2 must be completed and signed. The tenderer must indicate the actual technical specifications of the equipment offered in line with all requirements.

- The tenderer shall upload this Annex No. 2 in Word and PDF format (including technical documentation proving the fulfillment of all technical requirements to the e-JN information system in the section “Other Annexes”.

We declare that the offered equipment fully meets the minimum technical requirements of the client for the public procurement titled "Drive test bench system for hardware simulation of interaction between a drive/source and the power grid", and that we are ready to realize the order according to the following specification:

|  | EQUIPMENT |  |  |  |
| --- | --- | --- | --- | --- |
| No. | REQUIREMENTS | OFFERED |  | DATASHEET |
| 1.1 | Three-phase induction machine: |  |  |  |
|  | - It has a wound three-phase rotor and allows double fed operation of the IM | - YES | - NO |  |
|  | - Nominal power: from ≥ 30 kW to ≤ 35 kW (S1 – continuous operation); the nominal power must be equal to the nominal power of the three-phase synchronous machine under item 1.2 | Nominal power: ________ kW (S1 – continuous operation); the nominal power is equal to the nominal power of the three-phase synchronous machine under item 1.2: |  |  |
|  |  | - YES | - NO |  |
|  | - Nominal stator voltage: 400 V | - YES | - NO |  |
|  | - Number of poles: 4 | - YES | - NO |  |
|  | - Nominal frequency: 50 Hz | - YES | - NO |  |
|  | - Designed for operation under inverter supply | - YES | - NO |  |
|  | - Force-ventilated | - YES | - NO |  |
|  | - Built-in temperature monitoring element | Element type: ________ |  |  |
|  | - Maximum machine speed: ≥ 3000 rpm | Maximum machine speed: ________ rpm |  |  |
|  | - Foot-mounted design (B3) | - YES | - NO |  |
|  | - Equipped with an incremental encoder – speed/position sensor with ≥ 5000 pulses per revolution. | Number of pulses per revolution: ________ |  |  |
|  |  | Equipped to the shaft of the machine: |  |  |
|  |  | - YES | - NO |  |
| 1.2 | Three-phase synchronous machine: |  |  |  |
|  | - It has an excitation winding on the rotor | - YES | - NO |  |
|  | - Nominal power: from ≥ 30 kW to ≤ 35 kW (S1 – continuous operation); the nominal power must be equal to the nominal power of the three-phase induction machine under item 1.1. | Nominal power: ________ kW (S1 – continuous operation); the nominal power is equal to the nominal power of the three-phase induction machine under item 1.1: |  |  |
|  |  | - YES | - NO |  |
|  | - Nominal stator voltage: 400 V | - YES | - NO |  |
|  | - Number of poles: 4 | - YES | - NO |  |
|  | - Nominal frequency: 50 Hz | - YES | - NO |  |
|  | - Power factor range: from 0.1 to 1 (inductive/capacitive in both motoring and generating operation). | - YES | - NO |  |
|  | - Designed for operation under inverter supply | - YES | - NO |  |
|  | - Force-ventilated | - YES | - NO |  |
|  | - Built-in temperature monitoring element | Elementa type: ________ |  |  |
|  | - Maximum machine speed: ≥ 3000 rpm | Maximum machine speed: ________ rpm |  |  |
|  | - Foot-mounted design (B3) | - YES | - NO |  |
|  | - Equipped with an incremental encoder – speed/position sensor with ≥ 5000 pulses per revolution | Number of pulses per revolution: ________ |  |  |
|  |  | Equipped to the shaft of the machine: |  |  |
|  |  | - YES | - NO |  |
| 1.3 | Controllable excitation voltage source: |  |  |  |
|  | - Nominal power: it must cover the maximum power required to excite the synchronous machine under item 1.2. | Nominal power: ________W |  |  |
|  |  | Covers the maximum power required to excite the synchronous machine under item 1.2: |  |  |
|  |  | - YES | - NO |  |
|  | - Voltage range: must cover the voltage range required to excite the synchronous machine under item 1.2. | Voltage range: from ________ to ________ V |  |  |
|  |  | Covers the voltage range required to excite the synchronous machine under item 1.2: |  |  |
|  |  | - YES | - NO |  |
|  | - Current range: must cover the current range allowed to excite the synchronous machine under item 1.2. | Current range: from ________ to ________ A |  |  |
|  |  | Covers the current range allowed to excite the synchronous machine under item 1.2: |  |  |
|  |  | - YES | - NO |  |
|  | - The time profile of the excitation voltage can be changed using the software interface on the central computer of the test bench system under item 1.13 or by means of a microcontroller in the power converter of the synch. machine under item 1.9. | - YES | - NO |  |
|  | - Enables operation of synchronous machine in over- and under-excited state within permissible values of stator and rotor currents | - YES | - NO |  |
| 1.4 | Torque sensor: |  |  |  |
|  | - Number of measurement ranges: ≥ 2 | Number of measurement ranges: ________ |  |  |
|  | - Nominal torque of the first meas. range: from ≥ 300 Nm to ≤ 500 Nm | Nominal torque of the first meas. range: ________ Nm |  |  |
|  | - Nominal torque of the second meas. range: from ≥50 Nm to ≤ 100 Nm | Nominal torque of the second meas. range: ________ Nm |  |  |
|  | - Maximum rotation speed: ≥ 5.000 rpm | Maximum rotation speed: ________ rpm |  |  |
|  | - Accuracy class: ≤ 0,05 % full scale (first meas. range) and ≤ 0,1 % full scale (second meas. range) | Accuracy class first meas. range: ________ % full scale Accuracy class second meas. range: ________ % full scale |  |  |
|  | - Cut-off frequency: ≥ 10 kHz. | Cut-off frequency: ________ kHz |  |  |
|  | - Resolution: ≤ 0,25 Nm (first meas. range) and ≤ 0,1 Nm (second meas. range) | Resolution (first meas. range): ________ Nm Resolution (second meas. range): ________ Nm |  |  |
| 1.5 | Interface for adjusting torque/speed sensor measurement signals: |  |  |  |
|  | - Analogue outputs: from -10 V to +10 V | Analogue outputs: from _________ to _________ V |  |  |
|  | - Analogue low-pass filter that enables for a constant torque control of the machines | - YES | - NO |  |
| 1.6 | Mechanical couplings: |  |  |  |
|  | - Elastomer coupling with split clamping, which allows easy disassembly without moving of the machines or the torque sensor | - YES | - NO |  |
|  | - Allows for adjusting the damping/flexibility of the couplings by replacing the elastomer elements | - YES | - NO |  |
| 1.7 | Shaft and clutch protection elements: |  |  |  |
|  | - Foldable, with the possibility of easy opening to access the clutch/couplings | - YES | - NO |  |
|  | - Built-in safety sensors that are connected to the central computer under item 1.11, whereas in the open/unprotected state of the shaft the operation of the test bench system is disabled | - YES | - NO |  |
| 1.8 | Machine table for the test bench: |  |  |  |
|  | - Welded steel base frame with leveling feet to which a carrier clamping plate is attached, which enables perfect coaxial installation of the synchronous machine under item 1.2, the torque sensor under item 1.3 and the induction machine under item 1.1. | - YES | - NO |  |
|  | - Carrier clamping plate (machine table) length: ≤ 2500 mm | Carrier clamping plate (machine table) length: ________ mm |  |  |
|  | - Carrier clamping plate (machine table) width: ≤ 900 mm | Carrier clamping plate (machine table) width: ________ mm |  |  |
|  | - Carrier clamping plate thickness: ≥ 70 mm | Carrier clamping plate thickness: ________ mm |  |  |
|  | - The carrier clamping plate has adequate T-slots in the longitudinal direction, which serve to align both machines, the torque sensor and the shaft protection elements and enables an adequate coaxial mechanical coupling of the shafts of both machines via a torque sensor. | - YES | - NO |  |
|  | - Support elements that enable the height-alignment of shafts of the synchronous machine, the torque sensor and the induction machine and the installation of shaft guards. | - YES | - NO |  |
|  | - It has a built-in cabinet that contains power supplies for sensors mounted on both machines and the machine table and interface for adjustment of measured signals under item 1.5. | - YES | - NO |  |
| 1.9 | Modular system of power electronics converters for powering three-phase AC machines: |  |  |  |
|  | - It is based on back-to-back topology of two three-phase bridge converters with an DC-link energy storage | - YES | - NO |  |
|  | - Enables 4-quadrant operation | - YES | - NO |  |
|  | - Maximum DC bus voltage: ≥ 850 V | Maximum DC bus voltage: ________ V |  |  |
|  | - Nominal power of the system: ≥ 100 kW (continuous operation at a switching frequency of 5 kHz and a maximum DC bus voltage) | Nominal power of the system (continuous operation at a switching frequency of 5 kHz and a maximum DC bus voltage): ________ kW |  |  |
|  | - Nominal current: ≥ 120 Arms (continuous operation at a switching frequency of 5 kHz) and ≥ 70 Arms (continuous operation at a switching frequency of 10 kHz) | Nominal current (continuous operation at a switching frequency of 5 kHz): ________ Arms Nominal current (continuous operation at a switching frequency of 10 kHz): ________ Arms |  |  |
|  | - Switching frequencies range of the inverter on the drive side: from ≤ 5 kHz to ≥ 10 kHz | Switching frequencies range of the inverter on the drive side: from ________ kHz to ________ kHz |  |  |
|  | - Maximum grid voltage: ≥ 480 Vrms | Maximum grid voltage: ________ Vrms |  |  |
|  | - Nominal line (grid) frequency: od ≤50 Hz od ≥60 Hz | Nominal line (grid) frequency: from _________ Hz to ________ Hz |  |  |
|  | - Contains an Active Front End (AFE) LCL filter on the grid side that is tuned to the switching frequency of the inverter on the grid side. | - YES | - NO |  |
|  | - Grid inverter switching frequency: between ≥5 kHz and ≤10 kHz and tuned to the LCL filter | Grid inverter switching frequency: _________ kHz |  |  |
|  |  | Grid inverter switching frequency is tuned to the LCL filter: |  |  |
|  |  | - YES | - NO |  |
|  | - Enables operation with power factor: from -1 to 1 | Enables operation with power factor: from ________ to ________ |  |  |
|  | - It has forced air cooling of the three-phase converter modules | - YES | - NO |  |
|  | - Measurements of all line currents on both the grid side converter and drive side converter (6x in total), as shown in Figure 1. | Measurements of all line currents on the grid side converter (3x): |  |  |
|  |  | - YES | - NO |  |
|  |  | Measurements of all line currents on the drive side converter (3x): |  |  |
|  |  | - YES | - NO |  |
|  | - Measurement of the instantaneous values of the DC-link voltage | - YES | - NO |  |
|  | - Measurement of the instantaneous values of line currents (3x) and of phase voltages (3x) on the grid side of the LCL filter, which enables power factor control on the grid side of the LCL filter | Measurement of the instantaneous values of line currents (3x) on the grid side of the LCL filter: |  |  |
|  |  | - YES | - NO |  |
|  |  | Measurement of the instantaneous values of phase voltages (3x) on the grid side of the LCL filter: |  |  |
|  |  | - YES | - NO |  |
|  |  | Enables power factor control on the grid side of the LCL filter: |  |  |
|  |  | - YES | - NO |  |
|  | - It contains a modular control unit for acquisition, processing and generation of signals based on an open-programmable processor (e.g., adequate microprocessor, digital signal processor, FPGA unit, etc.) and corresponding peripheral units (i.e., AD and DA converters), which enable closed-loop real-time control of the entire system based on measurements. The control unit is connected to the central computer under item 1.11 and enables the implementation of custom control algorithms (i.e., rapid prototyping - adequate control algorithms can be programmed by the user using block programming and loaded on the programmable processor, where the algorithms are executed in real time). | Contains a modular control unit for acquisition, processing and generation of signals based on an open-programmable processor and corresponding peripheral units (i.e., AD and DA converters), which enable closed-loop real-time control of the entire system |  |  |
|  |  | - YES | - NO |  |
|  |  | Type of the open-programmable processor (e.g., adequate microprocessor, digital signal processor, FPGA unit, etc.): _____________________________________ |  |  |
|  |  | The control unit is connected to the central computer under item 1.11: |  |  |
|  |  | - YES | - NO |  |
|  |  | The control unit enables the implementation of custom control algorithms (i.e., rapid prototyping): |  |  |
|  |  | - YES | - NO |  |
|  | - The control unit enables block programming of control algorithms (for example in the software environment Matlab Simulink, NI LabView or equivalent) | The control unit enables block programming of control algorithms in software environment: _________________________________ |  |  |
|  | - The control unit is acquiring in real time (via digital or analog inputs) at least the following signals: - the pulses from the incremental encoder of the corresponding machine under item 1.1 or 1.2, - torque from the torque sensor under item 1.4, - all 3 line currents of the bridge converter on the drive side, - all 3 line currents of the bridge converter on the grid side, - DC-link voltage, - all 3 line currents on the grid side of the LCL filter, - all 3 phase voltages of the grid, - temperature of both bridge converter modules, - PLC interface reference values under item 1.11, - state of the emergency stop button, - status of the main contactor for connecting the system to the grid, - states of safety functions (errors) of both converter modules. | The control unit is acquiring in real time (via digital or analog inputs) the following signals: - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - Additionally: ______________________________________ _________________________________________________ _________________________________________________ _________________________________________________ |  |  |
|  | - The control unit is generating (via digital or analog outputs) at least the following output signals: - PWM signals for switching of all transistors in both converter modules, - switching on/off the main contactor for connecting the converter system to the grid, - excitation voltage reference value for the power supply under item 1.3. | The control unit is generating (via digital or analog outputs) the following output signals: - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - Additionally: ______________________________________ _________________________________________________ _________________________________________________ |  |  |
|  | - It has the function of "soft" synchronization to the grid (pre-charge circuit) | - YES | - NO |  |
|  | - It has at least the following basic hardware or software self-protection functions: - adequate fuses in the circuits, - hardware implementation of dead time in transistor driver circuits, which prevents the possibility of a short-circuit of the DC-link in the three-phase converter modules, - over-current protection of three-phase converter modules, - over-temperature protection of three-phase converter modules. | It has the following hardware or software self-protection functions: - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - Additionally:________________________________________________________________________________________ _________________________________________________ _________________________________________________ |  |  |
|  | - Has implemented basic control algorithms (i.e., 4 quadrant drive operation) containing at least: - control of DC-link voltage by means of the grid converter. - speed control of the machine under item 1.1 or 1.2 by means of the drive converter | Has implemented the following basic control algorithms (i.e., 4 quadrant drive operation): - _________________________________________________ _________________________________________________ - _________________________________________________ _________________________________________________ - Additionally:________________________________________________________________________________________ |  |  |
|  | - All components are installed in a protective cabinet that has an emergency shutdown button | - YES | - NO |  |
| 1.10 | Switching, measurement, and safety system: |  |  |  |
|  | - Contains adequate switches, which, by means of switching manipulations, enable at least the following power supply combinations of the individual windings of the induction and synchronous machine under items 1.1 and 1.2: - The stator winding of the synchronous machine is powered by the inverter system under item 1.9 or directly from the grid. - The rotor winding of the synchronous machine is powered by the power supply under item 1.3 or is not powered. - The rotor winding of the induction machine is powered by the corresponding inverter system under item 1.9 and the stator winding of the asynchronous machine is connected directly to the grid. - The stator winding of the induction machine is powered by the corresponding inverter system under item 1.9 and the rotor winding of the induction machine is short-circuited. - The stator winding of the induction machine is powered directly from the grid and the rotor winding of the induction machine is short-circuited. | Contains adequate switches, which, by means of switching manipulations, enable the following power supply combinations of the individual windings of the induction and synchronous machine under items 1.1 and 1.2: |  |  |
|  |  | 1) The stator winding of the synchronous machine is powered by the inverter system under item 1.9 or directly from the grid: |  |  |
|  |  | - YES | - NO |  |
|  |  | 2) The rotor winding of the synchronous machine is powered by the power supply under item 1.3 or is not powered: |  |  |
|  |  | - YES | - NO |  |
|  |  | 3) The rotor winding of the induction machine is powered by the corresponding inverter system under item 1.9 and the stator winding of the asynchronous machine is connected directly to the grid: |  |  |
|  |  | - YES | - NO |  |
|  |  | 4) The stator winding of the induction machine is powered by the corresponding inverter system under item 1.9 and the rotor winding of the induction machine is short-circuited: |  |  |
|  |  | - YES | - NO |  |
|  |  | 5) The stator winding of the induction machine is powered directly from the grid and the rotor winding of the induction machine is short-circuited: |  |  |
|  |  | - YES | - NO |  |
|  | - Includes adequate fuses/safety devices in all circuits of both machines. | - YES | - NO |  |
|  | - Contains adequate wired connections for direct measuring of the voltages at all winding terminals of both machines, as shown in Figure 1. | - YES | - NO |  |
|  | - Contains at least 7 current sensors that enable simultaneous measurement of instantaneous values of all line currents of the test specimen (i.e., induction or synchronous machine): - 1 sensor for measuring the excitation current of the synchronous machine (the nominal value must include the maximum value of the current allowed to excite the synchronous machine under item 1.2), - at least 6 current sensors that enable simultaneous measurement of stator and rotor currents of the induction machine, where the stator current sensors are adjustable/movable to allow measurement of stator currents of the synchronous machine or 9 corresponding current sensors that enable simultaneous measurement of stator and rotor currents of both machines under item 1.1 and 1.2, - sensors must be able to measure instantaneous values of AC and DC currents, - frequency range: 0 to >= 500 kHz at +-3dB attenuation, - accuracy <= 0.05 % - galvanic isolation of the output signal, - non-battery power supply, - measuring ranges of the sensors must cover corresponding maximum current values of both machines under items 1.1 and 1.2. | Total number of current sensors: ________ |  |  |
|  |  | Number of current sensors for measuring rotor currents of both the synchronous and induction machines: ________ |  |  |
|  |  | Meas. range of the sensor for measuring of the excitation current of the synchronous machine: from _______ to _______ A |  |  |
|  |  | Maximum permissible excitation (rotor) current of the synchronous machine: _______ A |  |  |
|  |  | Meas. range of the sensors for measuring of the rotor currents of the induction machine: from _______ to _______ A |  |  |
|  |  | Maximum permissible rotor current of the induction machine: _______ A |  |  |
|  |  | Number of current sensors for measuring stator currents of both the synchronous and induction machines: ________ |  |  |
|  |  | Meas. range of the sensors for measuring of the stator currents of the synchronous and induction machines: from _______ to _______ A |  |  |
|  |  | Maximum permissible stator current of: - the induction machine: _______ A - the synchronous machine: _______ A |  |  |
|  |  | Stator current sensors can be easily switched/moved between induction and synchronous machines (only valid if only three stator current sensors are used): |  |  |
|  |  | - YES | - NO |  |
|  |  | Sensors can measure instantaneous values of AC and DC currents: |  |  |
|  |  | - YES | - NO |  |
|  |  | Sensor frequency range (at +-3dB attenuation): from ________ to ________ kHz |  |  |
|  |  | Sensor accuracy: ________ % |  |  |
|  |  | Sensors galvanic isolation of the output signal: |  |  |
|  |  | - YES | - NO |  |
|  |  | Sensors non-battery power supply: |  |  |
|  |  | - YES | - NO |  |
|  |  | the measuring ranges of the sensors cover corresponding maximum current values of both machines under items 1.1 and 1.2: |  |  |
|  |  | - YES | - NO |  |
|  | - Installed in a protective cabinet | - YES | - NO |  |
| 1.11 | Control and data logging /measurement system: |  |  |  |
|  | - Built-in industrial computer with at least the following specifications: - processor Intel Core i7 2.5 GHz or equivalent or better, - ≥ 16 GB DDR3 RAM, - hard drive SSD ≥ 500 GB, - graphic card, - ≥ 2x network interface 1000/100 MBit/s (Ethernet), - ≥ 3x USB, - ≥ 1x serial port. | Built-in industrial computer with the following specifications: - Processor: _________________________________________________ - RAM: _________________________________________________ - Hard drive: _________________________________________________ - Graphic card: _________________________________________________ - Network interface: _________________________________________________ Number of network cards: _________ - Type and number of USB ports: _________________________________________________ - Number of serial ports: _________________________________________________ - Other specifications: __________________________________________________________________________________________________ __________________________________________________________________________________________________ __________________________________________________________________________________________________ |  |  |
|  | - Central industrial computer, based on dedicated software, enables: - centralized control/automation of the entire test bench system at the system level, - programming of processor units in both modular converter systems under item 1.9 (i.e., rapid prototyping of control algorithms). | Central industrial computer, based on dedicated software, enables centralized control/automation of the entire test bench system at the system level: |  |  |
|  |  | - YES | - NO |  |
|  |  | Central industrial computer, based on dedicated software, enables programming of processor units in both modular converter systems under item 1.9 (i.e., rapid prototyping of control algorithms): |  |  |
|  |  | - YES | - NO |  |
|  | - Contains an adequate PLC interface for the automation of the entire test bench system. | Manufacturer and series of the implemented PLC: ____________________________________ |  |  |
|  | - Based on the PLC interface, it enables acquisition of basic measuring signals in real time, which contain at least: - torque measurement based on the torque sensor under item 1.4, - speed measurement based on incremental encoders mounted on machines under items 1.1 and 1.2, - ≥ 4 channel temperature measurement of machines under items 1.1 and 1.2. | Based on the PLC interface, it enables acquisition of following basic measuring signals in real time: |  |  |
|  |  | - Torque measurement based on the torque sensor under item 1.4: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Speed measurement based on incremental encoders mounted on machines under items 1.1 and 1.2: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Number of channels for temperature monitoring: ________ |  |  |
|  |  | - Additional measurements: ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |
|  | - Based on the PLC interface, it enables generation of basic control signals in real time, which contain at least: - reference values for the DC-link voltage control of both converter modules on the grid side under item 1.9, - reference values for speed, current or torque control of both converter modules on the drive side under item 1.9, - reference value of the excitation voltage for the power supply under item 1.3, - signals for activation of modular converters under item 1.9. | Based on the PLC interface, it enables generation of following basic control signals in real time: |  |  |
|  |  | - Reference values for the DC-link voltage control of both converter modules on the grid side under item 1.9: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Reference values for speed, current or torque control of both converter modules on the drive side under item 1.9: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Reference value of the excitation voltage for the power supply under item 1.3: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Signals for activation of modular converters under item 1.9: |  |  |
|  |  | - YES | - NO |  |
|  |  | - Additional signals: _______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |
|  | - It has implemented, wired, and marked connections for parallel measurement of at least the following measuring quantities (low voltage signals from corresponding sensors): - 3x phase voltage on the grid side of the LCL filter of the modular converter supplying the synchronous machine, - 3x line current on the grid side of the LCL filter of the modular converter that supplies the synchronous machine, - 3x line current on the grid side of the modular converter that supplies the synchronous machine, - 1x DC-link voltage of the modular converter that supplies the synchronous machine, - 3x line current on the drive side of the modular converter that supplies the synchronous machine, - 3x line current on the stator side of the synchronous machine (connected to the corresponding current sensors under item 1.10), - 1x line current on the rotor side of the synchronous machine (connected to the corresponding current sensor under item 1.10), - 3x line current on the stator side of the induction machine (connected to the corresponding current sensors under item 1.10), - 3x line current on the rotor side of the induction machine (connected to the corresponding current sensors under item 1.10), - speed measurement based on the incremental encoder mounted on the synchronous machine under item 1.2, - speed measurement based on the incremental encoder mounted on the induction machine under item 1.1, - torque measurement based on the sensor under item 1.4, - 3x phase voltage on the grid side of the LCL filter of the modular converter supplying the induction machine, - 3x line current on the grid side of the LCL filter of the modular converter that supplies the induction machine, - 3x line current on the grid side of the modular converter that supplies the induction machine, - 1x DC-link voltage of the modular converter that supplies the induction machine, - 3x line current on the drive side of the modular converter that supplies the induction machine. | It has implemented, wired, and marked connections for parallel measurement of the following measuring quantities (low voltage signals from corresponding sensors): |  |  |
|  |  | - 3x phase voltage on the grid side of the LCL filter of the modular converter supplying the synchronous machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the grid side of the LCL filter of the modular converter that supplies the synchronous machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the grid side of the modular converter that supplies the synchronous machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 1x DC-link voltage of the modular converter that supplies the synchronous machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the drive side of the modular converter that supplies the synchronous machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the stator side of the synchronous machine (connected to the corresponding current sensors under item 1.10): |  |  |
|  |  | - YES | - NO |  |
|  |  | - 1x line current on the rotor side of the synchronous machine (connected to the corresponding current sensor under item 1.10): |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the stator side of the induction machine (connected to the corresponding current sensors under item 1.10): |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the rotor side of the induction machine (connected to the corresponding current sensors under item 1.10): |  |  |
|  |  | - YES | - NO |  |
|  |  | - speed measurement based on the incremental encoder mounted on the synchronous machine under item 1.2: |  |  |
|  |  | - YES | - NO |  |
|  |  | - speed measurement based on the incremental encoder mounted on the induction machine under item 1.1: |  |  |
|  |  | - YES | - NO |  |
|  |  | - torque measurement based on the sensor under item 1.4: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x phase voltage on the grid side of the LCL filter of the modular converter supplying the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the grid side of the LCL filter of the modular converter that supplies the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the grid side of the modular converter that supplies the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 1x DC-link voltage of the modular converter that supplies the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - 3x line current on the drive side of the modular converter that supplies the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  | - It has implemented, wired, and marked connections for direct parallel measurement of at least the following voltages (high voltage signals from corresponding machine winding terminals): - on the terminals of the stator windings of the synchronous machine, - on the terminals of the rotor winding of the synchronous machine, - on the terminals of the stator windings of the induction machine, - on the terminals of the rotor windings of the induction machine. | It has implemented, wired, and marked connections for direct parallel measurement of the following voltages (high voltage signals from corresponding machine winding terminals: |  |  |
|  |  | - on the terminals of the stator windings of the synch. machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - on the terminals of the rotor winding of the synch. machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - on the terminals of the stator windings of the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  |  | - on the terminals of the rotor windings of the induction machine: |  |  |
|  |  | - YES | - NO |  |
|  | - It has an Ethernet communication connection between the main computer and the control units in both modular converters, which enables adequate programming of the control units (i.e., rapid prototyping) | - YES | - NO |  |
|  | - It is installed in a protective cabinet | - YES | - NO |  |
|  | - The cabinet has a dedicated shelf on which it is possible to place an independent measuring interface for parallel acquisition of measuring signals whereas the interface can be easily connected to the marked connections. | - YES | - NO |  |
| 1.12 | Power, signal, and communication connection cables: |  |  |  |
|  | - Supply and installation of all necessary power, signal and communication cables within the test bench system | - YES | - NO |  |
|  | - The tenderer must provide adequate functional connections for the full operation of the entire test bench system in accordance with the schematic in Figure 1 and the technical specifications. | - YES | - NO |  |
|  | - Signal and communication cables must be installed separately from power cables in such a way that the influence of EM interference on the measuring signals is adequately reduced. | - YES | - NO |  |
| 1.13 | System-level automation/control software: |  |  |  |
|  | - It is installed on a central industrial computer under item 1.11 | - YES | - NO |  |
|  | - Operating system: the latest version of MS Windows | Operating system: ________________ |  |  |
|  | - Software interface for controlling of the entire test bench at the system level and for capturing of measurement signals, which enables at least: - Choice of different control modes (for example, speed or torque control of an individual machine). - Entering of reference values for speed, torque, DC-link voltage control. - Data logging of basic measured signals given under requirement 1.11b). - Selection of various measuring modules (at least continuous operation S1, short-term operation S2, intermittent operation S3, adjustable load cycles and torque characteristics measurements). | Software interface for controlling of the entire test bench at the system level and for capturing of measurement signals enables following features: |  |  |
|  |  | - Choice of following control modes: _____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |
|  |  | - Entering of following reference values: ____________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |
|  |  | - Data logging of basic measured signals given under requirement 1.11b): |  |  |
|  |  | - YES | - NO |  |
|  |  | - Selection of following measuring modules: ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ |  |  |

| Special requirements for the Built-in industrial computer to the fulfilment of the objectives in accordance with the Decree on green public procurement (Official Journal of the Republic of Slovenia RS, Nos. 51/17 and 64/19): (built in computer under 1.11a) |  |  |  |
| --- | --- | --- | --- |
| 1. Computer complies with the latest Energy Star energy efficiency standards in force on the date of publication of the public tender’s notice or on the date of the invitation for submitting the tender, or equivalent standards. | - YES | - NO |  |
| 2. Computer is designed in such a way that the hard disk and memory are easily accessible using universally available tools (i.e. with a screwdriver, spatula, pliers, or tweezers) and are replaceable. | - YES | - NO |  |
| 3. The volume of the computer is measured in accordance with the SIST EN ISO 7779 standard and does not exceed 26 dB at the time of writing to the hard disk. The data must be labelled with the designation LpAm in dB. | - YES | - NO |  |

| Place and date: | Seal: | Authorized signature: |
| --- | --- | --- |

The operation is co-financed by the Republic of Slovenia, Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is carried out under priority axis 1: ''International competitiveness of research, innovation and technological development, in line with smart specialization for increased competitiveness and greening of the economy'', priority investment 1.1 ''Strengthening R&D infrastructure and capacity to develop excellence in this and the promotion of competent centers, especially those of European importance'', specific objective 1.1.1 ''Efficient use of research infrastructure and development of knowledge / competences for better national and international cooperation in the knowledge triangle'' under the Operational Program for implementation of the European cohesion policy 2014-2020.

The tenderer must indicate the actual technical specifications of the equipment offered in accordance with all requirements.

The tenderer must give the name of the attached catalogue or brochure of the equipment manufacturer and the page number(s), together with the relevant technical specification. In exceptional cases, the tenderer may also provide a link to the equipment manufacturer's website with the relevant technical specification. Where a link to the online document is provided, the tenderer shall indicate a page number, indicating the relevant technical specification. The tenderer shall attach to the tender the proof of fulfilling all technical requirements and in case the tenderer provides a link to the manufacturer's website, he must ensure their accuracy and functionality (active connection) in the Word format.

For the item Built-in industrial computer (personal computer) the tenderer must provide proof which shows fulfilment of the objectives in accordance with the Decree on green public procurement (Official Journal of the Republic of Slovenia RS, Nos. 51/17 and 64/19), as written in Technical specification, which are a part of the documentation of this public tender.