Ref. No.: 285-9/4-2021

Date: 15. 4. 2021

TECHNICAL SPECIFICATION AND REQUIREMENTS FOR THE SUPPLY OF VHF RADIO SYSTEMS AT AIRPORTS PORTOROŽ AND MARIBOR AND RADIO SITE JANČE

Identification: Version: 0.1

THIS DOCUMENT CONTAINS PROPRIETARY INFORMATION, AND EXCEPT WITH WRITTEN PERMISSION OF SLOVENIA CONTROL Ltd. SUCH INFORMATION SHALL NOT BE PUBLISHED OR DISCLOSED TO OTHERS, OR USED FOR ANY OTHER PURPOSE AND THE DOCUMENT SHALL NOT BE DUPLICATED IN WHOLE OR IN PART.

Intentionally Left Blank

Table of Contents

1	Introduction	6

1.1	Purpose	6

1.2	Scope	6

1.3	General	6

1.4	Reference Documents	7

1.4.1	Standards	7

1.4.2	Regulatory Documents	7

1.5	Definitions	8

1.6	Abbreviations	9

2	Functional and Technical Requirements	10

2.1	General	10

2.2	Functional	11

2.2.1	Position and Replacement of Equipment	11

2.2.1.1	Positioning of Equipment	11

2.2.2	Quantity of Radio Equipment	11

2.2.3	Quantity of 19'' Cabinet Racks	13

2.2.4	Quantity of Multicouplers	13

2.2.5	Quantity of Ethernet Switches	13

2.2.6	Antenna System Quantity	14

2.2.7	Functionality for Portorož, Maribor and Janče	15

2.3	System	15

2.3.1	Antenna Specification	15

2.3.1.1	Antenna type 1	16

2.3.1.2	Antenna type 2	17

2.3.2	Multicoupler specification	18

2.3.2.1	VHF Multicoupler type 1	18

2.3.3	Automatic filter specification	18

2.3.4	Lightning Protector Specification	19

2.3.5	VSWR monitor specification	19

2.3.6	Ethernet switch specification	20

2.3.7	Telecommunication Cabinet Specification	21

2.3.8	RF Cable Specification	22

2.3.9	Remote Control and Monitoring System (RCMS)	23

2.3.9.1	General requirements	23

2.3.9.2	RCMS connections	24

2.3.9.3	Requirements for the new RCMS (Possibility 2)	25

2.3.9.4	Requrements for upgraded RCMS (Possibility 1)	25

2.3.9.4.1	IO Extender	26

2.3.9.4.2	RCMS data presentation	26

2.3.9.4.3	Presentation window for radios series 4200	28

2.3.9.4.4	Presentation window for existing radios series 200	31

2.3.9.4.5	Presentation window for IO Extender	32

2.3.9.4.6	Presentation window for VSWR Monitor	32

2.3.10	Radio Equipment Interface Requirements	33

2.3.10.1	Analogue interfaces	33

2.3.10.2	Digital Interfaces	35

2.3.10.3	Parallel operation	36

2.3.10.4	RF Interfaces	36

2.3.11	VHF Transceiver Requirements	36

2.3.12	VHF Transmitter Requirements	37

2.3.12.1	Environmental Conditions	37

2.3.12.2	Construction and Design	37

2.3.12.3	Electrical Characteristics	38

2.3.12.4	Radiation Characteristics	40

2.3.12.5	Tuning	40

2.3.12.6	Power Supply	40

2.3.12.7	Control and Monitoring	41

2.3.12.8	Remote Control	41

2.3.12.9	Safety Precautions	42

2.3.13	VHF Receiver Requirements	42

2.3.13.1	Environmental Conditions	42

2.3.13.2	Construction and Design	42

2.3.13.3	Electrical Characteristics	43

2.3.13.4	Overall Selectivity	44

2.3.13.5	Audio Output	45

2.3.13.6	Radiation Characteristics	46

2.3.13.7	Power Supply	46

2.3.13.8	Control and Monitoring	47

2.3.13.9	Remote Control	47

2.3.13.10	Safety Precautions	48

2.3.14	VHF Multicoupler Requirements	48

2.3.14.1	General	48

2.3.14.2	Environmental Conditions	48

2.3.14.3	Construction and Design	49

2.3.14.4	Power Supply	50

2.3.14.5	Safety Precautions	50

2.4	Software	51

2.5	Accessories and instrumentation	51

2.6	Interoperability	51

2.7	RAM (Reliability, Availability, Maintainability)	52

2.7.1	General	52

2.7.2	Reliability	53

2.7.3	Maintainability	53

2.7.4	Availability	53

2.8	Redundancy	54

3	Logistic Support	54

3.1	Maintenance Concept	54

3.2	Spares and Support	55

3.2.1	General	55

3.2.2	Mandatory Spare Parts List	57

3.2.3	Recommended spare parts	57

3.3	Warranty	57

3.3.1	General	57

3.3.2	Warranty Period	58

3.3.3	Obligations of the Contractor	58

4	Technical System Documentation	60

5	Training	62

6	Safety and Quality Assurance	66

6.1	Safety	66

6.1.1	General	66

6.1.2	Safety Audit	67

6.1.3	Safety Plan	67

6.1.4	Safety Reports	68

6.1.5	Software Safety Requirements	69

6.2	Quality	72

6.2.1	General	72

6.2.2	Quality Assurance Plan	72

6.2.3	Quality Audit	73

7	Project Management	73

7.1	General	73

7.2	Project Management Plan	75

7.3	Documentation Plan	76

7.4	Logistic Support Plan	76

7.5	Installation and Commissioning Plan	77

7.6	Acceptance Test Plans and Procedures	78

8	Installation and Commissioning/Transition	79

8.1	Installation	79

8.2	Pre-Commissioning and Completion	80

8.3	Commissioning and Transition	80

9	Testing and Acceptance	81

9.1	Factory Acceptance	81

9.2	Site Acceptance	82

10	Drawings and pictures	84

10.1	Block schemes	84

10.1.1	Block scheme, site Maribor and Portorož	84

10.1.2	Block scheme, site Janče	85

10.2	Principal positioning	86

10.2.1	Principal positioning of antennas, site Maribor	86

10.2.2	Principal positioning of antennas, site Portorož	87

10.2.3	Principal positioning of antennas, site Janče	88

# Introduction

## Purpose

Slovenia Control Ltd. (referred also as: the Contracting authority) plans to replace the existing VHF radio equipment at the sites: Airports Portorož and Maribor and radio site Janče.

The replace of existing VHF equipment is intended for:

- Better reliability – the existing equipment is at the end of the life cycle;

- Necessity for 8.33 kHz channel separation;

- To increase capacity (number of frequencies) at the location Janče;

- Future smooth and easy migration to VoIP technology.

## Scope

A replacing process shall be divided in two phases.

The first phase shall include the following:

- Replacement of the existing radio equipment at the airports Portorož and Maribor. The number of frequencies to be installed at this two locations is defined in the chapter Quantity of Radio Equipment;

- Upgrade of RCMS at ATCC and integration of RCMS equipment for Portorož and Maribor;

- Installation of antenna systems;

- System and equipment rating training, training of ATSEP.

The second phase shall include the following:

- Replacement of the existing radio equipment at location Janče. The number of frequencies to be installed at this location is defined in the chapter Quantity of Radio Equipment;

- Integration of RCMS equipment for Janče;

- Installation of antenna system;

- System and equipment rating training, training of ATSEP.

## General

GERQ0010	The submitted tender documentation together with submitted technical documentation shall be neatly sorted in adequate sections and bound.

GERQ0020	The response to the specification is required to be comprehensive with a completed Compliance table as set out below.

GERQ0030	Tenderers are encouraged to offer the existing baseline products that are compliant with or equivalent to all mandatory requirements.

GERQ0040	The offered system shall have at least the same or better technical characteristics as requested in this tender.

GERQ0050	The Compliance table included in each chapter of this document provides an entry for each requirement.

The Tenderer shall also provide additional remarks if they are considered helpful for assessing the response (column Remarks in the Compliance table). Each remark shall be uniquely referred to corresponding document (Chapter, Paragraph).

GERQ0060	The Tenderer compliance status shall be indicated against each paragraph of this specification in the ’Compliance’ column with a Yes for Compliance or a No for Non-Compliance. No other response shall be recognised during the evaluation and absence of Yes or No shall be counted as Not Compliant.

GERQ0070	The Tenderer shall provide the proof/explanation of every indicated Yes - Compliance in the ’Compliance’ column. Additionally, where the answer can be found in the submitted documentation, the Tenderer shall provide the reference and point to the specific line/paragraph/chapter/document where the compliance status can be verified.

## Reference Documents

Wherever reference is made in this technical specification to specific regulations, standards and codes, the provisions of the latest current edition or revision of the relevant regulations, standards or codes in effect shall apply unless otherwise expressly stated in the technical specifications. Where such standards and codes are national or related to a particular country or region, other authoritative standards that ensure substantial equivalence to the standards and codes specified will be acceptable.

### Standards

RDST0010	The whole supplied equipment shall work completely in Compliance with the following documents:

ICAO Annex 10, Vol. II

ICAO Annex 10, Vol. III

ICAO Annex 10, Vol. V

ICAO EUR Doc 011, EUR Frequency Management Manual

ICAO Doc 9718, Handbook on Radio Frequency Spectrum Requirements for Civil Aviation

ETSI EN 300 676 (latest official edition) Ground-based VHF hand-held, mobile and fixed radio transmitters, receivers and transceivers for the VHF aeronautical mobile service using amplitude modulation; Technical characteristics and methods of measurement.

ETSI EN 301 489-22 (latest official edition) - EMC standard for radio equipment and services; Specific conditions for ground-based VHF aeronautical mobile and fixed radio equipment.

RDST0020	The equipment shall also meet the following standards for electrical safety:

IEC 60950 - International Safety Standard for Information Technology Equipment EN 60215 - Safety of Radio Frequency Transmitters.

RDST0030	The radio equipment shall be newly-produced and shall meet the requirements for ground based aeronautical stations laid down in standard documents listed in RDST0010.

### Regulatory Documents

RDRD0010	Commission Implementing regulation (EU) No 1079/2012 of 16 November 2012 laying down requirements for voice channels spacing for the single European sky (OJ L 320, 17. 11. 2012, p. 14), as last amended by the  Commission Implementing Regulation (EU) 2017/2160 of 20 November 2017 amending Implementing Regulation (EU) No 1079/2012 as regards certain references to ICAO provisions (OJ L 304, 21. 11. 2017, p. 47).

RDRD0020	The offered concept, design and equipment shall be fully compliant with Directive 2014/30/EU of the European Parliament and of the Council of 26 February 2014 on the harmonisation of the laws of the Member States relating to electromagnetic compatibility (recast) (OJ L OJ L 96, 29. 3. 2014, p. 79), as last amended by the Regulation (EU) 2018/1139 of the European Parliament and of the Council of 4 July 2018 on common rules in the field of civil aviation and establishing a European Union Aviation Safety Agency, and amending Regulations (EC) No 2111/2005, (EC) No 1008/2008, (EU) No 996/2010, (EU) No 376/2014 and Directives 2014/30/EU and 2014/53/EU of the European Parliament and of the Council, and repealing Regulations (EC) No 552/2004 and (EC) No 216/2008 of the European Parliament and of the Council and Council Regulation (EEC) No 3922/91(OJ L 212, 22. 8. 2018, p. 1).

RDRD0030	The offered concept, design and equipment shall be fully compliant with the latest ED-137 Interopability Standards for VoIP ATM Components Part 1.

RDRD0040	The offered training for the equipment shall be fully compliant with the ATSEP.OR.215 System and equipment rating training requirement of the Commission implementing regulation (EU) 2017/373 of 1 March 2017 laying down common requirements for providers of air traffic management/air navigation services and other air traffic management network functions and their oversight, repealing Regulation (EC) No 482/2008, Implementing Regulations (EU) No 1034/2011, (EU) No 1035/2011 and (EU) 2016/1377 and amending Regulation (EU) No 677/2011 (OJ L 62, 8. 3. 2017, p. 1), as last amended by the Commission Implementing Regulation (EU) 2020/469 of 14 February 2020 amending Regulation (EU) No 923/2012, Regulation (EU) No 139/2014 and Regulation (EU) 2017/373 as regards requirements for air traffic management/air navigation services, design of airspace structures and data quality, runway safety and repealing Regulation (EC) No 73/2010 (OJ L 104, 3. 4. 2020, p. 1).

## Definitions

| Term/Phrase | Definition |
| --- | --- |
| Availability | A measure of the degree to which an item is in an operable state at any time. |
| Reliability | The probability that an item will perform its intended function for a specified interval under stated conditions. |
| Maintainability | A measure of the ability of an item to be retained in, or restored to, a specified condition when maintenance is performed using prescribed procedures and technician skill levels. |
| Consumable spare | Expendable item, such as fuses, lamps, air filters, etc., that can be easily replaced by use of standard tools and procedures. |
| Civil Aviation Authority | Republic of Slovenia, Civil Aviation Agency |

Note: Terms “Tenderer”, “Contractor” and “Supplier” are synonims and shall mean entity, acting as bidder for public tender / entity, which implements public procurement.

## Abbreviations

| Abbreviation | Meaning, description |
| --- | --- |
| ACC | Area Control Centre |
| AC | Alternating Current |
| ATCC | Air Traffic Control Centre |
| BITE | Built-in Test Equipment |
| COTS | Commercial-Of-The-Shelf |
| DT | Delivery Time. Defined as the time elapsed between the date of order of a part by the Contracting authority (in case of need of the order of additional spare parts) and the date of shipment of ordered part from the Contractor to the Contracting authority. |
| DC | Direct Current |
| EMP | Electromagnetic pulse |
| FAT | Factory Acceptance Test |
| LAN | Local Area Network |
| LRU | Line Replaceable Unit that is repairable and that shall: - in case of failure be identified by the Built-In-Test-Equipment (BITE) monitoring system, with the help of Technical Manual(s) and use of standard tool; - be easily accessible and replaceable (e.g. plug-in unit, screwed terminals or connectors); - have minimal adjustment requirements (e.g. voltage level setting, etc.) - be designed in such a way to ensure that the system returns to its full operational status within a meantime of 60 minutes (MTTR < 1 hour) when only one LRU has failed. |
| MDF | Main Distribution Frame |
| MRT | Mean Response Time in hours (i.e. the average time from notification of failure for a technician to be ready to commence repair action). |
| MTBF | Mean Time Between Failures. A basic measure of reliability for repairable items. The average time during which all parts of the item perform within their specified limits, during a particular measurement period under stated conditions. |
| MTTR | Mean Time To Repair. A basic measure of maintainability. The sum of corrective maintenance times divided by the total number of failures within an item. The average time it takes to fully repair a failed system. Typically includes fault isolation, remove and replacement of failed item(s) and checkout. It excludes logistics downtime needed for spare part transport from stock to the site. |
| PC | Personal Computer |
| RF | Radio Frequency |
| RCMS | Remote Control and Monitoring System |
| Rx | Receiver |
| SAT | Site Acceptance Test |
| SIP | Session Initiation Protocol |
| SRU | Shop Replaceable Unit, defined as a unit which is replaced within a LRU and is repairable. |

| TAT | Turn-Around-Time. The time elapsed between the date of arrival of the faulty item at repair workshop and the date of shipment of the repaired (or replaced) item from the repair workshop to the Contracting authority. |
| --- | --- |
| TMA | Terminal Area |
| TRx | Transceiver |
| TWR | Air traffic control tower |
| Tx | Transmitter |
| UHF | Ultra High Frequency |
| VoIP | Voice over IP |
| VSWR | Voltage standing wave ratio |

# Functional and Technical Requirements

## General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTGE0010 | The Contractor is solely responsible for the System Design. If any changes to the design are necessary to meet the Specifications, the Contractor shall carry out such changes at no cost to the Contracting authority. |  |
| FTGE0020 | The Contractor shall be responsible for the system engineering efforts associated with the design, production, installation, testing and audit of the systems and equipment being provided. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTGE0030 | The Tenderer shall deliver a detailed rack layout and equipment placement scheme for every location, from which the placement of all equipment (transmitters, multicouplers, filters, switches remote control system…) shall be clearly visible inside the object and each telecommunication rack. |  |
| FTGE0040 | The Tenderer shall design a radio system according to the specified system described in this document and in its attachment diagrams. |  |
| FTGE0050 | The Tenderer shall deliver detailed schematic diagrams for RCMS layout interconnection from which a general and detailed design of the RCMS system shall be clearly visible. |  |
| FTGE0060 | The Tenderer shall deliver detailed schematic diagrams for radio and antenna system interconnection from which a general and detailed design of the radio system shall be clearly visible for every radio site independently. |  |

## Functional

### Position and Replacement of Equipment

#### Positioning of Equipment

Portorož:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| POSE0010 | At Portorož, the equipment (transceivers) shall be installed in the same technical room. |  |

Maribor:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| POSE0020 | At Maribor, the equipment (transceivers) shall be installed in the same technical room. |  |

Janče:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| POSE0030 | At Janče, the equipment (transceivers) shall be installed in the same technical room. |  |

### Quantity of Radio Equipment

The Tenderer shall offer appropriate equipment and ensure its full functionality for the following sites/frequencies:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| QTYE0010 | Portorož: Portorož VHF site shall consist of 3 VHF frequencies designed in the Main/Stand By mode. A block scheme of Portorož site is shown in paragraph 10.1.1. Block scheme, Maribor and Portorož site. The quantity of radio equipment and required frequencies installed at Portorož site are shown in table 1. |  |

| Frequency | Transceivers – TRx |
| --- | --- |
| VHF1: 121.500 MHz | Main/STBY |
| VHF2: 124.875 MHz | Main/STBY |
| VHF3: 129.800 MHz | Main/STBY |

Table 1: Quantity and required frequencies for Portorož site

| QTYE0020 | Maribor: Maribor VHF site shall consist of 3 VHF frequencies designed in the Main/Stand By mode. A block scheme of Maribor site is shown in paragraph 10.1.1. Block scheme, Maribor and Portorož site. The quantity of radio equipment and required frequencies installed at Maribor site are shown in table 2. |  |
| --- | --- | --- |

| Frequency | Transceivers – TRx |
| --- | --- |
| VHF1: 119.200 MHz | Main/STBY |
| VHF2: 121.500 MHz | Main/STBY |
| VHF3: 134.300 MHz | Main/STBY |

Table 2: Quantity and required frequencies for Maribor site

| QTYE0030 | Janče: Janče VHF site shall consist of 3 fixed VHF frequencies designed in the Main/Stand By mode, and one multichanell system. A block scheme of Janče site is shown in paragraph 10.1.2. Block scheme, Janče site. The quantity of radio equipment and required frequencies installed at Janče site are shown in table 3. |  |
| --- | --- | --- |

| Frequency name | Frequency [MHz] | Transceivers - TRx |
| --- | --- | --- |
| VHF1: 119.885 | VHF1: 119.8833 | Main/STBY |
| VHF2: 121.330 | VHF2: 121.325 | Main/STBY |
| VHF3: 132.475 | VHF3: 132.475 | Main/STBY |
| VHF4: Multichannel | VHF4: Multichannel | Main only |

Table 3: Quantity and required frequencies for Janče site

### Quantity of 19'' Cabinet Racks

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CRMR0010 | Portorož: The Tenderer shall offer one (1) TRx cabinet rack cabled for placing 3 pairs of VHF transceivers (main+standby), Tx and Rx filters. The Tenderer shall offer a rack blower unit, if necessary. |  |
| CRMR0020 | Maribor: The Tenderer shall offer one (1) TRx cabinet rack cabled for placing 3 pairs of VHF transceivers (main+standby) Tx and Rx filters. The Tenderer shall offer a rack blower unit, if necessary. |  |
| CRMR0030 | Janče: The Tenderer shall offer one (1) TRx cabinet rack cabled for placing 3 pairs of VHF transceivers (main+standby) Tx and Rx filters and one VHF transceiver with automatic filter. The Tenderer shall offer a rack blower unit, if necessary. |  |

### Quantity of Multicouplers

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CRMC0010 | Portorož: The Tenderer shall offer two (2) VHF Tx/Rx multicoupler such as defined in chapter 2.3.2.1 Multicoupler type 1 of this document for the TRx site. |  |
| CRMC0020 | Maribor: The Tenderer shall offer two (2) VHF Tx/Rx multicoupler such as defined in chapter 2.3.2.1 Multicoupler type 1 of this document for the TRx site. |  |
| CRMC0030 | Janče: The Tenderer shall offer two (2) VHF Rx multicoupler such as defined in chapter 2.3.2.1 Multicoupler type 1 of this document for the TRx site. |  |
| CRMC0060 | Janče: The Tenderer shall offer one (1) VHF automatic filter such as defined in chapter 2.3.3 Automatic filter of this document for the TRx site. |  |

### Quantity of Ethernet Switches

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CRES0010 | Portorož: The Tenderer shall offer two (2) Ethernet Switches such as defined in chapter 2.3.6 Ethernet switch of this document for the TRx site. |  |
| CRES0020 | Maribor: The Tenderer shall offer two (2) Ethernet Switches such as defined in chapter 2.3.6 Ethernet switch of this document for the TRx site. |  |
| CRES0030 | Janče: The Tenderer shall offer two (2) Ethernet Switches such as defined in chapter 2.3.6 Ethernet switch of this document for the TRx site. |  |

### Antenna System Quantity

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CRAN0010 | Portorož: The principal positioning of antenna system at Portorož site is shown in paragraph 10.2.2. Principal positioning of antennas, Portorož site. TRx antennas shall be mounted on a communication mast on Portorož Tower. Portorož antenna system shall consist of: - Two (2) TRx antenna with similar or better characteristics than described in chapter 2.3.1.1 Antenna type 1 of this document shall be used. - Low loss cables for connecting TRx antenna shall have similar or better characteristics than described in 2.3.8. RF Cable shall be used. - Two (2) EMP protector devices with similar or better characteristics than described in chapter 2.3.4 Lightning protector specification shall be used. - At least two (2) grounding kits per RF cable for grounding an outer connector of all RF cables shall be used. |  |
| CRAN0020 | Maribor: The principal positioning of antenna system at Maribor site is shown in paragraph 10.2.1. Principal positioning of antennas, Maribor site. Antennas shall be mounted on a communication mast on Maribor Tower. The Maribor antenna system shall consist of: - Two (2) VHF TRx antennas with similar or better characteristics than described in chapter 2.3.1.1 Antenna type 1 of this document shall be used. - Four (2) EMP protector devices with similar or better characteristics than described in chapter 2.3.4 Lightning protector specification shall be used. - Low loss cables for connecting TRx antenna shall have similar or better characteristics than described in 2.3.8. RF Cable shall be used. - At least four (2) grounding kits per RF cable for grounding an outer connector of all RF cables shall be used. |  |
| CRAN0030 | Janče: The principal positioning of antenna system at Janče site is shown in paragraph 10.2.3. Principal positioning of antennas, Janče site. Tx/Rx antenna shall be mounted on a communication mast on Janče Tower. The Janče antenna system shall consist of: - One (1) VHF TRx antenna with similar or better characteristics than described in chapter 2.3.1.2 Antenna type 2 of this document shall be used. - Two (2) EMP protector devices with similar or better characteristics than described in chapter 2.3.4 Lightning protector specification shall be used. - Low loss cables for connecting TRx antenna shall have similar or better characteristics than described in 2.3.8. RF Cable shall be used. - At least two (2) grounding kits per RF cable for grounding an outer connector of all RF cables shall be used. |  |

### Functionality for Portorož, Maribor and Janče

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FPMJ0010 | The tenderer shall offer best solution for the blank boxes (e.g. relays, splitters/couplers) for the concept shown on the block scheme of the site Portorož (paragraph 10.1.1. Block scheme, site Portorož), taking into account possible interferences. |  |
| FPMJ0020 | The tenderer shall offer best solution for the blank boxes (e.g. relays, splitters/couplers) for the concept shown on the block scheme of the site Maribor (paragraph 10.1.1. Block scheme, site Maribor), taking into account possible interferences. |  |
| FPMJ0030 | The tenderer shall offer best solution for the blank boxes (e.g. relays, splitters/couplers) for the concept shown on the block scheme of the site Janče (paragraph 10.1.2. Block scheme, site Janče), taking into account possible interferences. |  |

## System

### Antenna Specification

In designing the antenna systems the following types of antennas shall be used and offered:

#### Antenna type 1

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| ANTE0010 | The antennas shall consist of two vertically stacked and independently fed dipoles (stacked antenna). |  |
| ANTE0020 | Antennas shall be delivered together with mounting brackets necessary for fixing antennas to masts |  |
| ANTE0030 | Antenna systems shall be designed to withstand at least 15 years constant outdoor environmental conditions typical for high-altitude mountain conditions. At least the following antenna handling requirements shall be met: - Air temperature: -40˚C to 70˚C; - Air humidity: up to 99%; - Exposure to direct sunlight; - Exposure to ice; - Exposure to severe wind gusts at speed of 200 km/h; - Exposure to severe lightning strikes; Exposure to combination of above mentioned conditions i.e. extreme wind and ice. |  |
| ANTE0040 | Lightning protection solutions based on high speed gas-tube protectors shall be provided for each delivered antenna. |  |
| ANTE0050 | Elements necessary for connecting antennas to the radios shall be determined, specified and delivered by the Contractor. |  |
| ANTE0060 | All antenna systems shall be fully installed at all locations (including lightning protections, grounding kits, all necessary connectors, all necessary mounting brackets for mast fixation, cable hangers, etc.). |  |
| ANTE0070 | Mounting flanges shall be made from anticorrosive material or shall be hot dip galvanized steel. All screws and nuts shall be made from stainless steel. |  |
| ANTE0080 | Antennas shall be vertically erected omni-directional dipoles, stacked or individually mounted, depending on the local situation. |  |
| ANTE0090 | Standard broadband VHF dipole antennas shall be required with an omnidirectional radiation diagram. Horizontal plane omni shall be within ±1 dB. |  |
| ANTE0100 | Transmit and receive antennas shall be of such a type with power handling capacity to sufficiently afford the total power emission needs. |  |
| ANTE0110 | The frequency range shall be 118-137 MHz for VHF equipment with SWR better than 1:2. |  |
| ANTE0120 | The antenna polarization shall be vertical. |  |
| ANTE0130 | The antenna frequency range shall be 118-137 MHz (VHF Air band). The bandwidth shall not exceed 50 MHz. |  |
| ANTE0140 | The antenna height shall be less than 4300 mm. |  |
| ANTE0150 | An RF connector shall be N female. |  |
| ANTE0160 | The antenna impedance shall be 50Ω. |  |
| ANTE0170 | The antenna weight shall not exceed 15 kg. |  |
| ANTE0180 | The maximal wind velocity of the antenna shall be at least 200 km/h. |  |
| ANTE0190 | The antenna gain shall be at least 2 dBi. |  |
| ANTE0200 | The antenna VSWR shall be less than 2.0. |  |

#### Antenna type 2

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| ANTE0210 | The antenna shall consist at least of two vertically stacked and independently fed dipoles (stacked antenna). |  |
| ANTE0220 | The antenna polarization shall be vertical. |  |
| ANTE0230 | The antenna frequency range shall be 118-137 MHz (VHF Air band). The bandwidth shall not exceed 50 MHz. |  |
| ANTE0240 | Antenna shall be of such a type with power handling capacity to sufficiently afford the total power emission needs. |  |
| ANTE0250 | The antenna height shall be less than 6500 mm. |  |
| ANTE0260 | RF connectors shall be N female. |  |
| ANTE0270 | The antenna impedance shall be 50Ω. |  |
| ANTE0280 | Antenna system shall be designed to withstand at least 15 years constant outdoor environmental conditions typical for high-altitude mountain conditions. At least the following antenna handling requirements shall be met: - Air temperature: -40˚C to 70˚C; - Air humidity: up to 99%; - Exposure to direct sunlight; - Exposure to ice; - Exposure to severe wind gusts at speed of 200 km/h; - Exposure to severe lightning strikes; Exposure to combination of above mentioned conditions i.e. extreme wind and ice. |  |
| ANTE0290 | The antenna weight shall not exceed 50 kg. |  |
| ANTE0300 | The maximal wind velocity of antenna shall be at least 200 km/h. |  |
| ANTE0310 | The antenna gain shall be at least 2 dBi. |  |
| ANTE0320 | The antenna SWR shall be less than 1.8. |  |
| ANTE0330 | The antenna shall be DC grounded by an appropriate cross section to sustain severe lightning strikes. |  |
| ANTE0340 | The antenna shall be constructed in a way to be mounted on the top of the mast. |  |

### Multicoupler specification

In designing the antenna systems the following types of multicouplers shall be used:

#### VHF Multicoupler type 1

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| COUP0010 | The number of cavities per channel shall be 1. |  |
| COUP0020 | The number of channels shall be 3. |  |
| COUP0030 | The cavity tuning range shall be within 118-137MHz. |  |
| COUP0040 | The cavity isolation channel to channel shall be not less than 15 dB at 0.5 MHz. |  |
| COUP0050 | The maximum power handling per channel shall be at least 50W. |  |
| COUP0060 | The maximum channel insertion loss shall be less than 2.5 dB. |  |
| COUP0070 | The minimum channel separation in the multicoupler shall be at least 200 kHz. |  |

### Automatic filter specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FILT0100 | The number of cavities shall be 2. |  |
| FILT0110 | Tuning method shall be fully automatic |  |
| FILT0120 | Tuning range shall cover Air band118 - 137MHz. |  |
| FILT0130 | The overall tuning range shall not exceed 50 MHz. |  |
| FILT0140 | The filter insertion loss shall not exceed 2.5 dB within the frequency range 118 – 137 MHz. |  |
| FILT0150 | The frequency raster shall be configurable to 8.33 kHz or 25 kHz spacing. |  |
| FILT0160 | The selectivity shall be at least 36dB in the frequency range ±0,5% and at least 48dB in the frequency range ±1% from the tuned frequency. |  |
| FILT0170 | The tuning accuracy shall not exceed 8 kHz from the desired frequency through the whole band. |  |
| FILT0180 | The maximum power handling shall be at least 200W CW. |  |
| FILT0190 | Due to the lack of space, the maximum filter dimension shall not be more than 700mm - depth. |  |
| FILT0200 | VHF automatic filter shall be controlled directly from a connected TX/TRX using a dedicated interface. |  |
| FILT0210 | The automatic filter shall have an automatic emergency power switchover from AC to DC power supply in order to provide continuous uninterrupted operation. |  |

### Lightning Protector Specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LIGT0010 | The lightning protector shall be a Gas Tube type. |  |
| LIGT0020 | The return loss shall be less than 26 dB (1.1:1). |  |
| LIGT0030 | The insertion loss shall be less than 0.15 dB. |  |
| LIGT0040 | All connectors shall be an N female type. |  |
| LIGT0050 | The frequency range shall be at least DC – 1500 MHz. |  |
| LIGT0060 | The surge voltage and current shall be adequate for the system. |  |

### VSWR monitor specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SWRM0010 | The Antenna VSWR Monitor shall have at least 250W power ratings. |  |
| SWRM0020 | Impedance of the Antenna VSWR Monitor shall be 50Ω. |  |
| SWRM0030 | The main line return loss of the Antenna VSWR Monitor shall be greater than 30dB. |  |
| SWRM0040 | The frequency range of the Antenna VSWR Monitor shall be at least from 100-200 MHz. |  |
| SWRM0050 | The insertion loss of the Antenna VSWR Monitor shall be less than 0.1dB. |  |
| SWRM0060 | The Antenna VSWR Monitor shall have an N type connector. |  |
| SWRM0070 | The VSWR Monitor shall be equipped by LAN interface and shall be integrated into the RCMS system. |  |
| SWRM0080 | The Antenna&Cable Monitor shall provide measuring of RF parameters such as forward and reflected power, VSWR and loss between antennae and filters. |  |
| SWRM0090 | The VSWR monitor shall be able to communicate with the RCMS system in the way to activate an alarm in case when VSWR exceeds configurable value. |  |
| SWRM0100 | The Antenna&Cable Monitor shall be powered by a battery DC power supply of 28V DC. |  |

### Ethernet switch specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| ETHS0010 | The ethernet switch shall have at least 24 ports. |  |
| ETHS0020 | The ethernet switch shall meet requirements according to: - IEEE802.3 for 10BASE-T, - IEEE802.3u for 100BASE-TX - IEEE802.3ab for 1000BASE-T - IEEE802.3x for Flow Control - IEEE802.1p Class of Service |  |
| ETHS0030 | The ethernet switch shall have redundand power inputs. |  |
| ETHS0040 | The operating temperature of ethernet switch shall be at least -0 to +60˚C. |  |
| ETHS0050 | The ethernet switch shall meet environmental requirements according to following standards: - IEC60068-2-27 (Shock) - IEC60068-2-6 (Vibration) |  |

### Telecommunication Cabinet Specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TRCA0010 | All telecommunication cabinets shall be designed for housing the 19” equipment. |  |
| TRCA0020 | All telecommunication cabinets shall be prepared for connecting all equipment to the AC dual line (A&B) supply – wiring line A for main radios, line B for STBY radios. |  |
| TRCA0030 | All telecommunication cabinets shall have an AC (A&B) power distribution panel with individual fuses dedicated for every device. |  |
| TRCA0040 | All telecommunication cabinets shall have all necessary cabling for the connection of all radio equipment; audio cables, data cables, RF cables, grounding cables. |  |
| TRCA0050 | All telecommunication cabinets shall have wiring blocks necessary for connecting analog audio and signaling data to the MDF. |  |
| TRCA0060 | All telecommunication cabinets shall be equipped with all necessary additional RF equipment: Multicouplers, Filters, antenna relays, etc. |  |
| TRCA0070 | Due to the technical room dimensions, all 19’’ telecommunication cabinets prepared for housing Tx and Rx radio equipment shall be maximum 43U high. |  |
| TRCA0080 | The cabinet shall have a lockable rear door that restricts access to the rear AC distribution. |  |
| TRCA0090 | In case that the telecommunication cabinet has front doors it shall be transparent so that the radio equipment is visible. |  |
| TRCA0100 | The equipment cabinets shall be professionally made. |  |
| TRCA0110 | The cabinet front unused spaces shall be fitted with blank 19’’ panels. |  |

### RF Cable Specification

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RFCA0010 | The cable type shall be LMR-400 or similar. |  |
| RFCA0020 | The cable diameter size shall be 12 mm or less. |  |
| RFCA0030 | The cable shall meet flame and fire retardant requirements according to IEC 60332-1 and IEC 603323-3.C |  |
| RFCA0040 | The cable shall meet requirements according to IEC 61034 (up to 2-1/4) for a smoke emission test. |  |
| RFCA0050 | The cable shall meet requirements according to UL 1581-1991 Sect. 1080 VW 1 Flame test, for a flame test on a single cable and UL 1581-1991 Sect. 1160 Vertical Tray Flame test, for a fire test on a bunched cable. |  |
| RFCA0060 | The cable shall meet requirements according to IEC 60754-1 and IEC 60754-2 for gasses emitted during the combustion of material form cables. |  |
| RFCA0070 | The cable weight shall not exceed 0.15 kg/m. |  |
| RFCA0080 | The cable Minimum Bending Radius shall be equal or less than 120 mm for repeated bends |  |
| RFCA0090 | The cable impedance shall be 50Ω. |  |
| RFCA0100 | The declared cable Attenuation (dB/100m) shall be 5 dB/100m or less, measured on a 150 MHz frequency (Standard conditions; VSWR 1.0, cable temperature 20˚C). |  |
| RFCA0110 | The cable operating temperature shall be at least in a range -40˚C to 80˚C. |  |

### Remote Control and Monitoring System (RCMS)

The contractor has two (2) possibilities to provide an RCMS for the systems provided:

- To perform an upgrade of the existing RCMS system

- To deliver completely new RCMS

Picture 1:RCMS overview

#### General requirements

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0010 | - The RCMS system shall be able to monitor and control new radio equipment to be installed on site Portorož, Maribor and Janče. |  |
| RCMS0020 | - A general approach to a solution of the RCMS according to the RCMS principal diagram shall be based on the following requirements: - Automatic switchover of radio equipment in the case of failure based on an alarm signal derived from the BIT information, - Manual switchover of radio equipment in the case of failure or maintenance/reconfiguration purposes available locally or from the supervision room, - Various remote control and monitoring interfaces embedded in the system. |  |
| RCMS0030 | The RCMS shall be designed to reach high availability and reliability. Malfunction of any part of RCMS system shall not have impact on the Radio system functionality. |  |
| RCMS0040 | The RCMS shall allow connecting a laptop with local RCMS in order to monitor and control the operation of radio equipment installed at site during on-site maintenance. |  |
| RCMS0050 | The RCMS shall monitor and control all receivers, transmitters and transceivers configured in the system and indicate their real statutes and failures. |  |
| RCMS0060 | The RCMS shall provide manual and automatic user-initiated switching between main and standby radios at any predefined time or time periods (e.g. specific time, specific date, periodically…). |  |
| RCMS0070 | The RCMS shall provide an interruptive BIT test to be performed on any selected individual radio in the system. |  |
| RCMS0080 | The RCMS shall be extendable for at least 200 radio equipment and 12 radio sites. |  |
| RCMS0090 | The RCMS shall monitor temperature and AC/DC power supply at all sites. |  |
| RCMS0100 | The RCMS shall provide the user-defined multiple I/O ports for control and monitoring of additional equipment (such as alarms and auxiliary switching). |  |
| RCMS0110 | The RCMS system shall communicate by TCPIP connection. |  |
| RCMS0120 | The RCMS shall monitor all data links (end to end) in the system and indicate failures. |  |
| RCMS0130 | The Contractor shall deliver and install all necessary hardware components that includes also the new RCMS server and a laptop for on site maintenance purpose. |  |
| RCMS0140 | The Contractor shall deliver and install all necessary software supported with at least two (2) licences. |  |
| RCMS0150 | - The complete software shall be available on USB key. |  |

#### RCMS connections

In the current situation for the communication between ATCC Ljubljana and radio sites mainly TCPIP connection type is in use. Only one leased line is in use supporting serial RS232 connection for site Krim (R&S 200), other sites with R&S 200 are connected via serial/TCP IP interface, that means that serial virtual com ports are in use on the RCMS side.

Picture 2: RCMS principal diagram

Note:	Laptop with RCMS application shall be connected on radio site during preventive or corrective on-site maintenance.

#### Requirements for the new RCMS (Possibility 2)

In addition to the general requirements contractor is obligated to provide the new RCMS system with as much as posibble or even more functionalities than existing RCMS (functionalities are described in 2.3.9.4.2).

#### Requrements for upgraded RCMS (Possibility 1)

The contractor shall upgrade the existing RCMS system in the way to control and monitor the new systems installed on sites. Contractor may use all the parts of the existing RCMS system except RCMS server shall be replaced.

##### IO Extender

- The IO Extender will provide outputs and inputs from/to Central RCMS in order to control user-defined multiple I/O ports for control and monitoring of additional equipment such as alarms, air-condition and auxiliary switching, at least 4 input and 4 output functions for each radio site.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0160 | The IO Extender shall have at least 4 relay outputs. Each output shall have a pair of independent contacts and shall be able to drive 230VAC/2A. |  |
| RCMS0170 | The IO Extender shall have at least 4 digital inputs. |  |
| RCMS0180 | The IO Extender shall provide monitoring of the 230AC and DC supply at radio site. DC voltage can be in range 0-48VDC. |  |
| RCMS0190 | The IO Extender shall be equipped by configurable LAN interface for connection to Central RCMS Server with minimum requirements: - configurable connection type – client or server, - configurable local connection port, - configurable local connection mask and - configurable local connection gateway |  |
| RCMS0200 | The IO Extender shall be powered by 230VAC single phase. The frequency 50Hz ±2% shall be met. AC Supply Voltage Variation tolerance -10/+15 % shall be met. |  |

##### RCMS data presentation

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0210 | The RCMS shall be based on a commercial PC platform with the operating system Microsoft Windows 7/64bit (with implemented latest available Service Pack). |  |
| RCMS0220 | Critical operational alarms shall be presented on an RCMS alarm message window. The alarm data shall contain all alarm details, e.g. date, time, alarm description, equipment location, etc. |  |
| RCMS0230 | At the supervision room, the control and monitoring of remote operating equipment shall be indicated (visually and by audible alarms in critical cases). |  |
| RCMS0240 | - Combined colour alarm signals from radios and sites shall be used to present the current operational status of system elements. For example, the following colour palette is desirable: - No fault - Green colour, - Fault - Red colour, - Warning - Yellow colour, - Unknown state - Grey colour. |  |
| RCMS0250 | The RCMS application shall facilitate the generation of various reports according to the user selection (visible on the screen and/or printed on paper). |  |
| RCMS0260 | - Reports shall cover the current and past statuses in several combined or divided forms: - Radio equipment status and alarms; - Transmission and ancillary equipment status and alarms; - Radio equipment configuration (operational parameters and settings) for each unit and/or system. |  |
| RCMS0270 | - All reports and configurations shall be made possible to archive and recall in the RCMS application with a version date/number. |  |
| RCMS0280 | A full range of features shall be provided within the database application to perform a statistical analysis (for longer time periods at least 1 year). |  |
| RCMS0290 | - All collected events and data shall be logged for a period of minimum 1 year. |  |
| RCMS0300 | It shall be possible to create a report of listed logs between two specified dates/times. |  |
| RCMS0310 | - It shall be possible for the administrator to configure the access levels for each user separately for each radio site. Security/access levels: - standard access – except control functions, - control access – all control functions, - administrator access – system configuration possibilities. |  |
| RCMS0320 | The Main screen shall be organized for at least 8 radio sites in one window with displaying all radios connected into RCMS. The radio shall be represented by object filled by colour specified by RCMS0420. The objects represented the radios shall be organized in line according to their affiliation to the radio site. The radio screen or radio window shall be displayed by double click on radio displayed in main window. The Tenderer can offer his own solution for presentation all required parameters stated in this specification. |  |
| RCMS0330 | The user shall be allowed to change and organize the names of radio sites. |  |
| RCMS0340 | - It shall be possible to configure a password for each individual user. |  |
| RCMS0350 | It shall be possible to save (back-up) the RCMS configuration parameters of radio and ancillary equipment on HDD and external media and restore the previous (saved) configuration at any time, if needed. |  |
| RCMS0360 | Temperature on site shall be presented on main window together with site name. The alarm shall be activated, if measured temperature on site exceeds a range of predefined value. Range of values shall be configured by the administrator user. |  |
| RCMS0370 | The RCMS software application shall be configurable for Site-RCMS or Central-RCMS part. |  |
| RCMS0380 | The RCMS software application installed on the Central RCMS Server shall be able to monitor and/or control equipment at all radio sites. Equipment means a Receiver, Transmitter, Transceiver, Voltage Sensor, Temperature Sensor, Logic status sensors and Antenna&Cable Monitor. |  |
| RCMS0390 | The RCMS software application installed on a Site RCMS Computer and on the Laptop shall be able to monitor and/or control only equipment at the relevant radio site. Access to equipment installed at other radio sites shall be forbidden. |  |
| RCMS0400 | Alarms and warnings shall be triggered based on the radio status, connection status, high temperature in a cabinet, low power supply AC and DC and high WSVR on the Antenna&Cable Monitor. |  |
| RCMS0410 | Alarms and warnings based on temperature, power supply AC and DC and high WSVR shall be configurable by the administrator who will set minimum and maximum values for each sensor. If a parameter is beyond the defined interval, the Central RCMS system and the Local RCMS system shall automatically trigger alarm or warning. |  |
| RCMS0420 | The administrator shall be able to create and modify a frequency object at a radio site by grouping radio equipment installed at the particular radio site based on their IP addresses. |  |

##### Presentation window for radios series 4200

Due to large number of parameters and functions to be displayed for radio equipment, the presentation window shall divide parameters into separate logical groups - (tabs or sections or tickets).

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0430 | - Presentation window for radios shall include tabs or sections or tickets: - Status, - Measurement, - Setting (of operational parameters), - Preset (setting of preset parameters) and - Events (reading alarm, warning and info events) |  |
| RCMS0440 | - The Status tab or section or ticket shall display at least next statuses/parameters: - Global states: - GO/NOGO status, - OK/NO_OK (Provides all currently recognized CBIT events) and - ACTIVE/INACTIVE status - Error states: - Voltage (Measure DC Voltage of internal supply), - VSWR LED (Indicates mishmashes antenna (VSWR above 2), - Battery (Measure DC Voltage of battery connector of secondary supply) and - Temperature. - Access to the radio: - Remote/Local - For Transmitter part: - PTT status, - Trigger Test Tone, - Carrier status (Indicates Carrier value (1 = power > 500mW), - Frequency, - Channel, - Channel Spacing (Raster), - TX Carrier of 5 Offsets (Provides information about actual configuration of 5 carrier offsets possibility), - TX power set, - Main/Stby status, - Measure current Tx Power, - Measure modulation depth, - Measure current VSWR level (Measures current analogue value of VSWR during active transmission). - For Receiver part: - Squelch status, - Current reception (Rx Indicator indicates that received signal is above S/N and/or RSSI squelch threshold; the logic to combine both thresholds can be configured via USB), - Frequency, - Channel, - Channel Spacing (Raster), - Squelch Threshold RSSI (analogue value for Rx Signal Strength), - Squelch Threshold (signal + noise), - Main/Stby status, - Measure current RSSI level, - Measure audio level (Measures current audio level on Rx audio output). |  |
| RCMS0450 | - The Status tab or section or ticket shall allow to control at least next parameters of the radio: - For Transmitter part: - PTT button for activate/deactivate PTT, - Tone button for activate/deactivate PTT with tone (Transmitter shall be keyed with 1 kHz test modulation; transmission can be stopped using this command and/or it is stopped automatically after defined PTT timeout - configurable via USB), - For Receiver part: - SQ button for activate/deactivate squelch. |  |
| RCMS0460 | - The Measurements tab or section or ticket shall display at least next measured parameters: - For Transmitter part: - DC battery voltage level, - DC internal voltage level, - Temperature of the device, - Tx Power, - Modulation depth, - VSWR level, - Audio level input. - For Receiver part: - DC battery voltage level, - DC internal voltage level, - Temperature of the device, - RSSI level, - Audio output level (Measures current audio level on Tx audio input). |  |
| RCMS0470 | - The Setting tab or section or ticket shall allow to set and monitor at least next parameters: - For Transmitter part: - Frequency in range of VHF or UHF band, - Channel Spacing - Raster (8.33 kHz or 25 kHz), - Carrier Offset, - Singlechannel (Enable/Disable single channel) provides information about the state of single channel flag. If single channel is enabled, only one frequency value is allowed (that means frequency cannot be changed), - Tx Power Level (e.g. low/normal/high) - each level has a defined transmission power value, - Modulation mode, - PTT Timeout, - Inhibit Tx (Suppressing/permitting transmission), - Main/Stby mode A/B, - Inactivate Radio (switch the radio to AUTO and/or OFF state which can cause transitions between ACTIVE and INACTIVE state), - Restart Radio, - Restart Connection TCPIP. - For Receiver part: - Frequency in range of VHF or UHF band, - Channel Spacing - Raster (8.33 kHz or 25 kHz), - Squelch Threshold RSSI (in uV), - Squelch Threshold SN (in dB), - Singlechannel (Enable/Disable), - Demodulation mode, - Main/Stby mode (A/B), - Inactivate Radio (switch the radio to AUTO and/or OFF state which can cause transitions between ACTIVE and INACTIVE state), - Restart Radio, - Restart Connection TCPIP. |  |
| RCMS0480 | - The Preset tab or section or ticket shall include next buttons in TX and RX part: - SAVE for saving the preset table to HDD, - LOAD for loading the preset table from HDD, - GET for getting channel preset from radio, - SET for setting the preset table to the radio. |  |
| RCMS0490 | - The Events tab or section or ticket shall include next buttons in TX and RX part: - Read Events List for receiving all events from radio, - Clear Events List for clearing all events into radio. |  |
| RCMS0500 | - The Events tab or section or ticket shall allow reading all events receiving from the radio. |  |

##### Presentation window for existing radios series 200

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0510 | - The RCMS application shall be able to display also existing statuses/parameters of radios series 200: - For Transmitter part: - SUPPLY – power supply, - ON – test indication, - LOC – local control ON, - LOW – low voltage, - USER – user defined action, - GO – device is ON, - VSWR – high RF signal reflection, - SYNT – synthesizer error, - AMPL – amplifier error, - TEMP – high temperature, - CARR – carrier signal generated, - PTT – PTT status, - POWER – indication of the transmission power. - For Receiver part: - SUPPLY – power supply, - GO – device or RX part is ON, - LOC – local control ON, - AGC – indication of a received signal in 4 levels, - USER – user defined action, - SQL – SQ status, - IBIT – control element for TEST generator ON. |  |

##### Presentation window for IO Extender

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0520 | Extender shall indicates at “Sensor and IO” tab statuses of: - AC Main power supply (alarm if AC supply for Extender is switched off) - DC Rectifier (alarm come from DC Rectifier connected to Extender) - Battery Low (alarm come from DC Rectifier in case of low battery) - Temperature in cabinet (alarm by threshold settings) - statuses of 4 input signals - statuses of 4 output with possibility to control those statuses |  |
| RCMS0530 | Extender shall indicates statuses of 4 relays. Rellays outputs shall be controlled by clicking on predefined value. |  |
| RCMS0540 | Extender shall allows to set alarm threshold levels for: - AC Power Supply, - DC Power Supply - Cabinet temperature measuring. |  |

##### Presentation window for VSWR Monitor

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMS0550 | VSWR Monitor shall indicates measuring values of RF parameters: - forward power - reflected power - VSWR and - loss between antenna and filters |  |

### Radio Equipment Interface Requirements

#### Analogue interfaces

Audio interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| AUDI0010 | The radios shall have the following audio interfaces: - Transmitter audio input, - Receiver audio output. |  |
| AUDI0020 | Audio interfaces shall be wired to the Main Distribution Frame (MDF). Slovenia Control mostly uses the KRONE type LSA Plus standard on sites and locations. |  |
| AUDI0030 | The line input on transmitters (AF input) shall be designed for a direct connection to communication lines – 600Ω balanced at a nominal level between -30dBm and +10dBm) adjustable by 1dBm steps. |  |
| AUDI0040 | The line output on receivers (AF output) shall be designed for a direct connection to communication lines – 600Ω balanced at a nominal level between -30dBm and +10dBm) adjustable by 1dBm steps. |  |
| AUDI0050 | For the purpose of legal recording each transmitter and receiver shall have a dedicated 600Ω recording output. |  |

Contact signalling interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CONI0010 | Radios shall be able to operate using contact signalling for PTT activation and Squelch indication. |  |
| CONI0020 | Dedicated connections shall be used for contact signaling for PTT and Squelch activation operation. |  |
| CONI0030 | Contact signaling interfaces shall be wired to the MDF. |  |
| CONI0040 | The contact interfaces shall be over-voltage and over-current protected. |  |

Push-to-talk (PTT) activation interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PTTI0010 | Transmitters shall be capable of both local and remote keying. In order to enable remote keying, a PTT activation contact interface shall be available. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PTTI0020 | The PTT activation signal shall be sent via: - Signaling lead of the E&M 4 wire analogue audio interface of the PCM multiplex equipment. - Configurable tone keying form 1800 to 3000 Hz, an adjustable referent line level from -5 to -25 dBm. |  |
| PTTI0030 | A relay contact outside the transmitter closing a circuit to earth shall key the transmitter. |  |
| PTTI0040 | The time delay between the activation of the relay and full power shall be less than 20 ms. |  |

Squelch indication interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SQLI0010 | Each receiver shall have a squelch indication interface, which shall enable systems connected to this interface to track when a squelch circuit on the receiver is closed. |  |
| SQLI0020 | The squelch indication signal shall be available for the control of the frequency and external systems (multiplex equipment, VCS). Therefore, when the squelch opens, an insulated relay contact shall close a circuit to earth within 20 ms. |  |
| SQLI0030 | The squelch indication signal shall be sent to VCS systems via: - Signalling lead of the E&M 4 wire analogue audio interface of the PCM multiplex equipment. - Configurable squelch tone signalling form 1800 to 3000 Hz, an adjustable referent line level from -5 to -25 dBm |  |

#### Digital Interfaces

Serial Interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| DATI0010 | In order to enable local configuration of the radio equipment, each radio shall have a dedicated RS232 or USB interface. |  |
| DATI0020 | It shall be possible to perform radio diagnostics and change radio equipment settings with the local PC/notebook maintenance software connected to the radio external connector(s). |  |

Ethernet Interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| DATI0060 | In order to enable connection to the VCS, each radio shall have a dedicated integral Ethernet interface for VoIP according to latest EUROCAE ED 137. |  |
| DATI0070 | In order to enable RCMS connection between radio devices and the TMCS system, each radio shall have a dedicated integral Ethernet interface with the SNMP protocol according to EUROCAE ED 136, ED 137C volume 5, physical separated from VoIP interfaces. |  |
| DATI0077 | The VCS and RCMS/TCMS interfaces shall be managed independently in the radio, in order that a malfunction on a network does not impair the data traffic on the remaining interfaces or the the functioning of the radio itself |  |
| DATI0080 | It shall be possible to configure the radio to support simultaneous audio (SIP/RTP) and remote control (RCMS) via IP. |  |
| DATI0090 | The Contractor shall support The Contracting authority to establish appropriate connection between the radio equipment and VCS. |  |

External Interfaces:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| DATI0100 | In order to enable monitoring of the external devices (e.g. Power supply, SWR indicator, …), each radio shall have a dedicated optoisolated interface or TTL interface. |  |
| DATI0110 | In order to enable the Main/Standby antenna relay changeover, each radio shall have a dedicated relay interface. |  |
| DATI0120 | In order to control automatic filter each radio shall have a dedicated interface. |  |

#### Parallel operation

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PARO0010 | For migration from analog to VoIP the radios shall be capable to operate in analog mode and VoIP mode in parallel. |  |
| PARO0020 | Each radio shall be possible to handle/operate at least 10 SIP sessions simuntaneusly without any interruptions and influencies on radio remote control. |  |

#### RF Interfaces

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RFIN0010 | RF interfaces shall be used for connecting antennas to radios. |  |
| RFIN0020 | The impedance of the RF interfaces shall be 50, unbalanced. |  |
| RFIN0030 | RF input and output connectors shall be a coaxial type N female for transceiver and transmitters. BNC female or SMA is allowed for RX input. |  |
| RFIN0040 | Transmitters shall not be damaged if the RF interface is open (Z=) or short circuited (Z=0). |  |
| RFIN0050 | Protection against mismatch shall be provided and damage due to any type of mismatch shall be prevented. |  |

### VHF Transceiver Requirements

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TRAR0010 | Unless otherwise stated, VHF transceivers shall satisfy all requirements specified for VHF transmitters and receivers (VHF transceivers shall comply both - with requirements for VHF transmitters and with requirements for VHF receivers in meaning as a standalone units or as a modules/parts). |  |
| TRAR0020 | The transceiver shall have a multichannel feature allowing up to 20 frequency channels to be programed, sorted and recalled by channel number. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TRAR0030 | VHF transceivers shall have dedicated recording output (600Ω impedance). When connected to this output Digital Voice Recording and Playback system shall record both directions of communication (transmitter input and receiver output) simultaneously. |  |

### VHF Transmitter Requirements

#### Environmental Conditions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| ENVT0010 | The Ambient Air Temperature tolerance shall be at least from -20 oC to +55 oC. |  |
| ENVT0020 | The Transmitter shall operate according to it’s specification at relative humidity in range at least between 10% and 90%. |  |
| ENVT0030 | The transmitter shall be capable of 100% duty cycle at full power and modulation across the full temperature range at least from -20 to +40 oC, without any degradation of RF parameters (e.g. for ATIS and VOLMET usage). |  |

#### Construction and Design

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CONT0010 | The design and construction of the equipment shall be modern, based on the up-to-date technical conceptions related with such equipment. It shall be fully transistorized or/and fitted with integrated circuits (ICs). |  |
| CONT0040 | The equipment assembly shall be fitted with necessary circuits, which will permit the operation on any desirable frequency within the frequency range mentioned in the relevant paragraph. |  |
| CONT0050 | The equipment shall be of compact size, to the maximum possible extent. |  |
| CONT0060 | Any special tools and special test equipment required shall be provided with the equipment. |  |
| CONT0070 | All radio equipment shall be housed in individual 19” rack enclosures. |  |
| CONT0080 | The reliability shall be the highest possible, warranting at least a 15.000 hours mean time between failures (MTBF) with normal periodic maintenance and a mean time to repair (MTTR) of at least 30 min. |  |
| CONT0090 | Remote site maintenance requirements shall in general be as minimal as possible. This particularly refers to the following: - preventive maintenance inspections on site shall be required not more often than once per month; - corrective maintenance on site shall be easy module replacement, without requiring complex skills or tools; - repair of modules shall be possible by the personnel of the concerned ANSP or of a company different from the Contractor. |  |
| CONT0100 | Each Transmitter shall be accompanied by a set of technical manuals. The technical instructions manual provided shall be clear, concise and fully illustrated including circuit schematic diagrams. Any special technique shall be fully described. The aforesaid technical manuals shall include complete installation, operation and maintenance instructions. |  |
| CONT0110 | It shall be possible to replace any fan with minimal effort. |  |

#### Electrical Characteristics

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| ELCT0010 | The equipment shall be tunable over the range of 118 to 137 MHz for VHF without any change of elements. The equipment shall comply with all aspects of this specification within the specified frequency ranges. |  |
| ELCT0020 | The transmitter shall have a multichannel feature allowing up to 10 frequency channels to be programmed, sorted and recalled by channel number. |  |
| ELCT0030 | The frequency error shall be within ± 1 x 10-6 over the whole VHF range and under the above mentioned environmental conditions. |  |
| ELCT0040 | The type of emission shall be Double Side Band Amplitude Modulation, A3E for speech. |  |
| ELCT0050 | The channel spacing shall be both 25 and 8.33 kHz for VHF for all radio equipment. |  |
| ELCT0060 | The carrier power delivered into a 50 Ohms load with a VSWR up to 1:2 shall be 50 W for VHF TWR A/G Communications (Portorož, Maribor), 50 W for VHF ACC and APP A/G communications (Janče). |  |
| ELCT0070 | VHF Transmitters shall support at least 4 carrier frequency offset for 25 kHz spacing and at least 2 carrier frequency offset for 8.33 kHz spacing. |  |
| ELCT0080 | There shall be a possibility of continuous adjustment of output power from 5 W to upper limit in steps. This operation shall be easily performed by the Contracting authority (from the device front panel or through RCMS software application, or by portable PC through an external dedicated port) at any time. |  |
| ELCT0090 | Protection against mismatch shall be provided and any mismatch shall not cause damage. |  |
| ELCT0100 | The input impedance shall be 600 Ohms, balanced. |  |
| ELCT0110 | Special input accepting a suitable microphone, capable to modulate the transmitter up to 90% shall be available. |  |
| ELCT0120 | The 3 dB AF bandwidth shall be 300 - 3400 Hz for 25 kHz channel spacing, as appropriate for 8.33 kHz, and as per ETSI EN 300676 Paragraph 7.4.4.3. |  |
| ELCT0130 | The response shall be attenuated by at least 25 dB at 5 kHz, as per ETSI EN 300676 paragraph 7.4.4.4. |  |
| ELCT0140 | The modulation depth shall be adjustable at least up to 90 %. |  |
| ELCT0150 | The Total Harmonic Distortion (THD) shall be less than 10 % to be measured with a test tone 1000 Hz at m = 90 % and below. |  |
| ELCT0160 | Local operation from the front-panel shall be provided by means of a Microphone modulation input, which is insulated from the telephone circuit input. |  |
| ELCT0170 | A limiter shall limit the modulation depth to 95 %. |  |
| ELCT0180 | The adjacent channel power shall not exceed the following values below the carrier power of the transmitter: 50dB (for 8.33 kHz channel spacing); and 60dB (for 25 kHz channel spacing). |  |
| ELCT0190 | It shall be possible to disconnect the Automatic Gain Control. |  |
| ELCT0200 | For the purpose of recording the transmission, an output with demodulated signal shall be available. |  |
| ELCT0210 | For the maintenance it shall be possible to download a radio configuration from a radio to a PC. |  |
| ELCT0220 | For the maintenance it shall be possible to upload a radio configuration from a PC to a radio. |  |

#### Radiation Characteristics

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RADT0010 | Harmonics of the carrier frequency shall not exceed 5 x 10-7 watts. |  |
| RADT0020 | Any other radiation shall not exceed 5 x 10-7 watts. This radiation shall be measured at a 50-ohm load directly at the transmitter output up to 1.200 MHz. |  |
| RADT0030 | The Cabinet Radiation shall be in accordance with common ATC standards EN 300 676-1 / EN 300 113-1 (refer to chapter „cabinet radiation). |  |
| RADT0040 | Shielding shall be in accordance with common ATC standards EN 301 489-1 and -22 and EN55022. |  |
| RADT0050 | Intermodulation Products Emission shall be in accordance with common ATC standards EN 300 676-1 (chapter 7.8 „Intermodulation attenuation“). |  |
| RADT0060 | Out Of-Band Noise shall be at least 155 dB/Hz below the carrier for frequencies separated more than 1 % from the carrier frequency. |  |

#### Tuning

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TUNT0010 | The transmitter tuning shall be an easy and quick operation carried out by means of built-in monitoring devices. |  |
| TUNT0020 | The application of a tuning procedure shall avoid the generation of any other frequency than the one determined by the crystal or synthesizer. |  |
| TUNT0030 | The whole tuning procedure shall be briefly described and easily accessible when working on the transmitter. |  |
| TUNT0040 | The spurious radiation requirements shall be met when the transmitter has been tuned in accordance with the procedure given in the manual. |  |

#### Power Supply

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PSPT0010 | For AC supply the equipment shall be designed for a 230 V AC single phase. |  |
| PSPT0020 | The frequency 50 Hz ± 2 % shall be met. |  |
| PSPT0030 | The AC Supply Voltage Variation tolerance +10/-15 % shall be met. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PSPT0050 | The DC Supply Voltage Variation tolerance from - 30% to +10% shall be met. |  |
| PSPT0060 | The transmitter performance as specified above shall be met for unlimited periods, with the supply voltage variation as specified. |  |

#### Control and Monitoring

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| CMMT0010 | The front panel lamps shall indicate the following functions: a) Power on b) Equipment transmitting c) Alarm Indication |  |
| CMMT0020 | The front panel controls shall include: a) Power on/off indication b) Local/Remote control switching capability |  |
| CMMT0030 | One or several measuring instruments on the front panel shall provide the following measurements: - All AC and DC voltages necessary for fault location - All RF levels necessary for tuning and fault location c) Output power d) Modulation depth e) VSWR monitor |  |

#### Remote Control

Overall remote control requirements are stated in chapter 2.3.9 Remote Control and Monitoring System (RCMS). On top of that all transmitters shall satisfy the following requirements:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| REMT0010 | Suitable Interfaces shall be available for remote operation and signaling via LAN. The following functions shall at least be performed: a) Push-to-talk b) General alarm signaling |  |
| REMT0020 | Built-in-test facilities enabling immediate fault location in case of failure shall be available. |  |

#### Safety Precautions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SAFT0010 | Measures shall be taken to protect the equipment against the effects of over-current or over-voltage. |  |
| SAFT0020 | The design and construction of the equipment shall not permit any significant change to its normal operation resulting from usual dust accumulation on its composing parts. |  |
| SAFT0030 | In order to ensure normal operation, special precautions against dust shall generally not be required at the equipment installation site. |  |
| SAFT0040 | Special safety devices, such as protective covers, shall be provided. |  |

### VHF Receiver Requirements

#### Environmental Conditions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RENR0010 | The Ambient Air Temperature tolerance shall be from -20 ˚C to +55 ˚C. |  |
| RENR0020 | The Reciever shall operate according to it’s specification at relative humidity in range at least between 10% and 90%. |  |

#### Construction and Design

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCOR0010 | The design and construction of the equipment shall be modern, based on the up-to-date technical conceptions related with such equipment. It shall be fully transistorised or/and fitted with integrated circuits (ICs). |  |
| RCOR0040 | The equipment assembly shall be fitted with necessary circuits, which will permit the operation on any desirable frequency within the frequency range mentioned in the corresponding paragraph. |  |
| RCOR0050 | The equipment shall be of compact size, to the maximum possible extent. |  |
| RCOR0060 | Any special tools and special test equipment required shall be provided with the equipment. |  |
| RCOR0070 | The reliability shall be the highest possible, warranting at least a 15.000 hours mean time between failures (MTBF) with normal periodic maintenance and mean time to repair (MTTR) of at least 30 min. |  |
| RCOR0080 | Remote site maintenance requirements shall in general be as minimal as possible. This particularly refers to the following: - preventive maintenance inspections on site shall be required not more often than once per month, - corrective maintenance on site shall be easy module replacement, without requiring complex skills or tools, - repair of modules shall be possible by the personnel of the concerned ANSP or of a company different from the Contractor. |  |
| RCOR0090 | Each Receiver shall be accompanied by a set of technical manuals. The technical instructions manual provided shall be clear, concise and fully illustrated including circuit schematic diagrams. Any special technique shall be fully described. The aforesaid technical manuals shall include complete installation, operation and maintenance instructions. |  |

#### Electrical Characteristics

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RELR0010 | The equipment shall be tunable over the range of 118 to 137 MHz for VHF equipment without any change of elements. The equipment shall comply with all aspects of this specification within the specified frequency ranges. |  |
| RELR0020 | The receiver shall have a multichannel feature allowing up to 10 frequency channels to be programmed, sorted and recalled by channel number. |  |
| RELR0030 | The type of reception shall be Double Side Band Amplitude Modulation, A3E for speech. |  |
| RELR0040 | The channel spacing shall be 25 and 8.33 kHz for VHF. |  |
| RELR0050 | The tuning of the receiver shall be an easy and quick operation. No measuring devices except a DC multimeter shall be required to tune the receiver with sufficient accuracy to the frequency determined by the inserted crystal. |  |
| RELR0060 | The spurious radiation requirements described below shall be met when applying the tuning procedure. |  |
| RELR0070 | The receiver shall withstand continuous 24 hour uninterrupted reception (100% duty cycle). |  |
| RELR0080 | The frequency error shall be within ± 1x10-6 over the whole range and under the above-mentioned environmental conditions. |  |
| RELR0090 | The input impedance shall be 50 Ohms and the VSWR shall be better than1:2 at the tuned frequency. |  |
| RELR0100 | The sensitivity shall be1.5 μV (or 3 μV EMF) or less for a signal-to-noise ratio (S+N/N) at the receiver output 10 dB at least for an input signal modulated 30% at 1 kHz. |  |
| RELR0110 | The audio output level variation shall not exceed 3 dB for an input voltage between 1.5 μV and 200 mV. |  |
| RELR0130 | The receiver shall incorporate a means to suppress the noise at the 600 Ohms output by at least 70 dB in the absence of a wanted signal. |  |
| RELR0140 | When the input signal exceeds the threshold by 3 dB, the squelch shall open at all RF inputs above 1 μV, and in less than 20 ms. |  |
| RELR0150 | A pre-set squelch control which meets the requirements of ETSI EN 300676 Paragraph 8.11.3 shall be provided. |  |
| RELR0160 | The squelch shall be independent of the audio output level control. |  |
| RELR0170 | A failure of squelch circuit itself shall not inhibit transmission of the audio signal to line. |  |
| RELR0180 | The squelch characteristics shall be such that the receiver is effectively silenced after cessation of a signal without any noticeable rise of the noise level at the receiver output. |  |
| RELR0190 | The squelch operation shall be accompanied by the illumination of a Front panel lamp or any other suitable indicator. |  |
| RELR0240 | The temperature stability of the squelch threshold shall be such that the variation of the threshold shall not exceed 3 dB for the temperature range defined above. |  |
| RELR0250 | For the maintenance it shall be possible to download a radio configuration from a radio to a PC. |  |
| RELR0260 | For the maintenance it shall be possible to upload a radio configuration from a PC to a radio. |  |

#### Overall Selectivity

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RSLR0010 | Spurious Response Rejection The spurious response rejection shall be in accordance with common ATC standard test procedure (EN 300 676). |  |
| RSLR0020 | Image Response Rejection The Image Response Rejection is measured by the two signal generators method. Two input signals are applied to the receiver input as follows: - Signal A (on channel), modulated 30 % at 1 % kHz to produce an S/N ratio of 10 dB. - Signal B (on image frequency), modulated by 60 % at 400 Hz. Then signal B shall be at least 80 dB above signal A to reduce the S+N/N by 3 dB. |  |
| RSLR0030 | IF Response Rejection The IF Response Rejection is measured by the two signal generators method. Two input signals are applied to the receiver input as follows: - Signal A (on channel), modulated 30 % at 1 % kHz to produce an S/N ratio of 10dB. - Signal B (on IF frequency), modulated by 60 % at 400 Hz. Then signal B shall be at least 80 dB above signal A to reduce the S+N/N by 3 dB. |  |
| RSLR0040 | Adjacent Channel Selectivity The Adjacent Channel Selectivity shall be in accordance with common ATC standard test procedure (EN 300 676). |  |
| RSLR0050 | Cross-modulation The cross-modulation shall be in accordance with common ATC standard test procedure (EN 300 676). |  |
| RSLR0060 | Blocking (Desensitization) The Blocking (Desensitization shall be in accordance with common ATC standard test procedure (EN 300 676). |  |
| RSLR0070 | Intermodulation The Intermodulation shall be in accordance with common ATC standard test procedure (EN 300 676). |  |

#### Audio Output

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RAOR0010 | The overall gain shall be sufficient to produce an audio output level of at least 1 mW into 600 ohms for a RF input of 1μV modulated to a depth of 60 % at 1 kHz. |  |
| RAOR0020 | The audio output shall be provided by means of a balanced 600-ohm transformer suitable for direct connection to Telekom Lines. |  |
| RAOR0030 | With a centre RF input frequency of 1 mV modulated 30 % and in the frequency band from 300 to 3400 Hz, the audio output level shall not vary by ± 3 dB with reference to the level at 1 kHz. |  |
| RAOR0040 | The response shall be attenuated by at least 30 dB at 5 kHz. |  |
| RAOR0050 | With the audio gain control set to a power output of 1 mW at the 600 ohm terminals for a RF input of 3 μV (EMF) modulated 60 % at 1000 Hz, any RF input between 1mV (EMF) and 100 mV (EMF) modulated 60 % at 1000 Hz shall not produce in excess of 5 % T.H.D. respectively at the 600 ohm output. |  |
| RAOR0060 | With set to a power output of 1 mW at the 600 ohm terminals for an RF input of 3 μV (EMF) modulated 60 % at 1000 Hz, any RF input between 1 mV (EMF) and 100 mV (EMF) modulated 90% at 1000 Hz the ratio (S/S+N) shall be better (higher) than 50 dB. |  |
| RAOR0070 | If the receiver includes a noise limiter, it shall be efficient with 100% modulation. The operation of the noise limiting circuits shall not involve a noticeable depression of the wanted audio output. The distortion of audio signals whose peaks modulate up to 100 % shall be a minimum. |  |

#### Radiation Characteristics

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RRDR0010 | The receiver shall be adequately screened to permit operation of any number of receivers at one location and co-location with transmitters. |  |
| RRDR0020 | The oscillator radiation shall not exceed 2 nW (10 μV/50 ohms) within a bandwidth of 1 kHz when measured at the 50 ohm antenna input over the frequency range from 9 kHz to 1 GHz. |  |
| RRDR0030 | The receiver shall meet the relevant international requirements for screening and direct radiation. |  |

#### Power Supply

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RPSR0010 | For AC supply the equipment shall be designed for a 230 V AC single phase. |  |
| RPSR0020 | The frequency 50 Hz ± 2 % shall be met. |  |
| RPSR0030 | The AC Supply Voltage Variation tolerance +10/-15 % shall be met. |  |
| RPSR0040 | The 28 V DC operation shall be available for back-up purposes. |  |
| RPSR0050 | The DC Supply Voltage Variation tolerance shall be from -30 % to +10 %. |  |
| RPSR0060 | The receiver performance as specified above shall be met for unlimited periods with the supply voltage variation as specified. |  |

#### Control and Monitoring

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMR0010 | The front panel lamps have to indicate the following functions: a) Power on b) Equipment receiving c) Squelch indication d) Alarm indication |  |
| RCMR0020 | The front panel controls shall include: a) Power on/off indication b) Local / Remote control switching capability c) A jack for headset connection |  |
| RCMR0030 | One or several measuring instruments shall provide the following measurements: - All AC and DC voltages necessary for fault location b) Received signal level |  |

#### Remote Control

Overall remote control requirements are stated in chapter 2.3.9 Remote Control and Monitoring System (RCMS). On top of that all, receivers shall satisfy the following requirements:

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RCMR0040 | Suitable Interfaces shall be available for remote operation and signalling via LAN or analogue or digital telephone lines in order to monitor and control the radio by RCMS. The following functions shall at least be performed remotely from the RCMS: a) Rx-ready indication b) Squelch c) Power on/off switching d) Power on/off signalling e) General alarm signalling |  |
| RCMR0050 | Built-in-test facilities enabling immediate fault location in case of failure shall be available. |  |

#### Safety Precautions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| RSFR0010 | Measures shall be taken to protect the equipment against the effects of over-current or over-voltage. |  |
| RSFR0020 | The permissible input voltage without causing damage shall be 5V at least independently of the signal frequency. On the system level the receiver station shall be protected against at least 15V. |  |
| RSFR0030 | The design and construction of the equipment shall not permit any significant change to its normal operation resulting from usual dust accumulationon its composing parts. |  |
| RSFR0040 | In order to protect normal operation, special precautions against dust shall generally not be required at the equipment installation site. |  |
| RSFR0050 | Special safety devices, such as protective covers, shall be provided. |  |

### VHF Multicoupler Requirements

#### General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| MCGR0010 | The Antenna Multicoupler shall be installed in the same cabinet with the transceivers if possible. |  |

#### Environmental Conditions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| MCER0010 | The Ambient Air Temperature tolerance shall be from -10 ˚C to +50 ˚C. |  |
| MCER0020 | The Radio equipment shall operate according to it’s specification at relative humidity in range at least between 10% and 90%. |  |
| MCER0030 | The Installation Altitude tolerance shall be from 0 to 1600 meters. |  |

#### Construction and Design

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| MCCR0010 | The design and construction of the equipment shall be modern, based on the up-to-date technical conceptions related with such equipment. |  |
| MCCR0020 | Modular construction shall be used to the maximum extent. |  |
| MCCR0030 | All components shall be readily accessible to the maintenance personnel, easily disassembled and replaced. |  |
| MCCR0040 | The equipment assembly shall be equipped with the necessary circuits, which will permit the operation on any desirable frequency within the frequency range mentioned in the relevant paragraph. |  |
| MCCR0050 | The equipment shall be of compact size, to the maximum possible extent. |  |
| MCCR0060 | Any special tools and special test equipment required shall be provided with the equipment. |  |
| MCCR0070 | Each multicoupler shall be accompanied by a set of technical manuals. The technical instructions manual provided shall be clear, concise and fully illustrated including circuit schematic diagrams. Any special technique shall be fully described. The aforesaid technical manuals shall include complete installation, operation and maintenance instructions. |  |
| MCCR0080 | The reliability shall be the highest possible, warranting at least a 15.000 hours mean time between failures (MTBF) with normal periodic maintenance and a Mean Time To Repair (MTTR) of at least 30 min. |  |
| MCCR0090 | Remote site maintenance requirements shall in general be as minimal as possible. This particularly refers to the following: - preventive maintenance inspections on site shall be required not more often than once per month - corrective maintenance on site shall be easy module replacement, without requiring complex skills or tools - repair of modules shall be possible by the personnel of the concerned ANSP or of a company different from the Contractor. |  |
| MCCR0100 | The multicoupler shall operate over the band 118-137 MHz for VHF Multicouplers without resorting to any serious component changes. The multicoupler shall be broadband, in case of active types, or field tuneable, in case of selective filter type MC. |  |
| MCCR0110 | The multicoupler shall be fully field-tuneable. |  |
| MCCR0120 | The input and output impedance shall be 50 ohms. |  |
| MCCR0130 | The input and output VSWR shall be better than 1:1.5 for VHF Multicouplers. |  |
| MCCR0140 | In case of a passive multicoupler, the insertion loss on the tune frequency between an input port and any output to a receiver port shall be less than 2dB. |  |

#### Power Supply

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| MCPR0010 | For AC supply the equipment shall be designed for a 230 V AC single phase. |  |
| MCPR0020 | The frequency 50 Hz ± 2 % shall be met. |  |
| MCPR0030 | The AC Supply Voltage Variation tolerance +10/-15 % shall be met. |  |

#### Safety Precautions

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| MCSR0010 | Measures shall be taken to protect the equipment against the effects of over-current or over-voltage. |  |
| MCSR0020 | The permissible input voltage without causing damage shall be 5V at least independently of the signal frequency. On the system level the receiver station shall be protected against at least 15V. |  |
| MCSR0030 | The design and construction of the equipment shall not permit any significant change to its normal operation resulting from usual dust accumulationon its composing parts. |  |
| MCSR0040 | Special safety devices, such as protective covers, shall be provided. |  |

## Software

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTSW0010 | The tenderer shall deliver all the necessary software available for control, monitor and maintain the radios and other equipment such as automatic filters, I/O extenders, SWR Monitors, etc. |  |
| FTSW0020 | The complete software shall be available on installation CDs or USB stick. All the software delivered (maintance software, RCMS software, shall be installed on the maintanance laptop. |  |
| FTSW0030 | The software, i.e. a software installation media, shall enable the installation on a standard server with certain minimum requirements, regardless of the processor or memory type, hard disk size or the type of graphics adapter. |  |
| FTSW0040 | A Specific Document (e.g. a version description document or equivalent) shall be issued for each software release or upgrade. |  |
| FTSW0050 | A software upgrade shall be made through an interface on the equipment. The upgrade shall be possible without any change or additional hardware. |  |

## Accessories and instrumentation

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTAI0010 | The Tenderer shall offer at least two sets of tools and service kits (e.g. cable connector adapters, etc.) for efficient maintenance, control/ monitoring and repair of delivered equipment. |  |
| FTAI0020 | The tenderer shall deliver a laptop (notebook) for maintenance support – so called maintanance laptop. Laptop should be capable to easily run all the software required. |  |

## Interoperability

The role of the Tenderer, is to ensure and declare compliance of its EATMN constituent with the essential requirements, specific requirements contained within the relevant implementing rules for interoperability and other relevant technical specifications (e.g. Community specifications, standards).

The purpose of the following requirements is to ensure that the procured and implemented system is compliant by all means with the requirements laid down in The Interoperability Regulation: REGULATION (EC) No 552/2004 on the interoperability of the European Air Traffic Management network.

Instruction to tenderers:

Tenderer shall assure compliance with the regulatory requirements, directives and applicable standards.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTIO0010 | The Tenderer shall attach with the tender documentation the EC Declaration of Compliance for all parts of the equipment/system (the Constituents) that is part of the offer and future delivery. |  |
| FTIO0020 | The Tenderer shall prepare and attach with the system documentation the EC Declaration of Suitability for use for the delivered radio systems. |  |
| FTIO0030 | The Tenderer shall offer radio equipment and systems which comply with Commission Implementing regulation (EU) No 1079/2012 of 16 November 2012 laying down requirements for voice channels spacing for the single European sky (OJ L 320, 17. 11. 2012, p. 14), as last amended by the Commission Implementing Regulation (EU) 2017/2160 of 20 November 2017 amending Implementing Regulation (EU) No 1079/2012 as regards certain references to ICAO provisions (OJ L 304, 21. 11. 2017, p. 47). |  |

## RAM (Reliability, Availability, Maintainability)

### General

Reliability, Maintainability and Availability are characteristics of the overall system which shall be specified, designed, implemented, tested, validated and documented. The quality of equipment can be considered as its ability to satisfy the user needs for the specified period of time and can be expressed with its operational availability. Two major contributors to the quality are: - Reliability, and - Maintainability.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTRG0010 | The methodology, techniques, processes and tools the Tenderer intends to use to achieve the specified RAM objectives shall be described or referenced in specific plans addressing architecture, hardware and software aspects. |  |

### Reliability

Reliability is defined as a probability that equipment will perform its intended function without error, under stated conditions, for a specified period of time.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTRR0010 | The Tenderer shall provide in the tender documentation a reliability model consisting of reliability block diagrams covering all functions of the system. |  |
| FTRR0020 | The MTBF and MTTR in hours and the availability shall be clearly shown in either a block diagram or a list showing the equipment breakdown to the functional unit level, with the identification of a specific common failure mode (e.g. switch over equipment). |  |
| FTRR0030 | The Tenderer shall provide in the tender documentation reliability predictions and analysis as per standard MIL-HDBK-217 or other commonly used and accepted method which shall be clearly stated. |  |

### Maintainability

Maintainability is the measure of the ability of an item to be retained in or restored to a specified condition when maintenance is performed by personnel having specified skill levels, using prescribed procedures and resources, at each prescribed level of maintenance and repair. MTTR is the sum of corrective maintenance times at any specified level of repair, divided by the total number of failures within an item repaired at that level, during a particular interval under stated conditions.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTRM0010 | The Tenderer shall provide in its response the MTTR estimates and prediction for each of the following: - Line Replaceable Unit (LRU); - Each major equipment group; - Each single channel of the system. |  |
| FTRM0020 | The Tenderer shall state in its response the average turn-around time (TAT) for a Line Replaceable Unit (LRU). |  |

### Availability

For the purpose of this specification, Availability is defined as a ratio of the total time the system is capable of performing its mission, against the time for which it is required to perform that mission, expressed as a percentage. The availability calculation excludes all planned downtimes.

The figures for Availability quoted in this Specification are for Operational Availability (Ao) and shall be calculated using the following equation:

Ao =

MTBF = Mean Time Between Failures in hours.

MTTR = Mean Time To Repair in hours.

MRT = Mean Response Time in hours (i.e. the average time from a notification of failure for a technician to be ready to commence a repair action).

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTRA0010 | The offered Radio System shall be considered as failed or unavailable when one or more radio frequencies are not available per radio site. |  |
| FTRA0020 | The lifetime of the Radio System shall be not less than 15 years or 130.000 hours operation. |  |
| FTRA0030 | The estimated value of Operational Availability of the Radio System shall be not less than 99.998%. |  |
| FTRA0040 | All calculation shall be provided to indicate the system ability to meet the requirements with full explanation of all assumptions. |  |

## Redundancy

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| FTRE0010 | To ensure a high level of availability, all radios shall operate with redundancy. |  |
| FTRE0020 | The redundancy system shall provide at least the following functions: in analog operation, the switch between the main and the stand by radios shall be possible without requiring any equipment external to the radio cabinet. |  |
| FTRE0030 | The Tenderer may propose one or more solutions that meet the functional requirements above. |  |

# Logistic Support

## Maintenance Concept

Maintenance of this system and associated environmental features will be efficient and responsive to operational needs and the requirements of aviation safety.

There are two levels of maintenance:

- "Level one": on-site, where the staff will isolate and replace faulty LRUs and SRUs, and

- "Level two": performed either in the Repair Workshop (at the Contractor's premises) or on-site (technical staff of Slovenia Control), or exceptionally, at any other site agreed between the Contractor and the Contracting authority. Faulty LRUs and SRUs will be repaired down to the component level, to a maximum possible extent.

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSMC0010 | The Tenderer shall propose his own vision and concept of the most appropriate and the most efficient maintenance approach (especially corrective maintenance). |  |
| LSMC0020 | The Contractor shall guarantee the shortest possible MTTR (Mean Time to Repair), which shall be less than 2 hours. |  |
| LSMC0030 | The Contractor shall support the establishment of a support and, if necessary, service agreement with local companies covering the repair/replacement of COTS items by the original suppliers. |  |
| LSMC0040 | The Contractor shall guarantee support and repair/replacement of all hardware, software and documentation (including COTS products as well), including the supply of spare parts for a period corresponding to the expected life of the unit, but not less than 10 years. In case the Contractor cancels the hardware service agreement, the Supplier shall be obliged to continue the support by his own means. |  |

## Spares and Support

### General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSSU0010 | The offered initial set of spare parts shall be specified in a tender documentation, in a form of the Mandatory Spare Parts List, which shall contain all spare parts (LRUs, SRUs, consumables), with the price for each individual item. |  |
| LSSU0020 | A Spare Parts List shall, for each spare part, include: - Contractor/Manufacturer Name, - Part Number (P/N), - Spare Part Description, - Recommended Quantity of Spare Parts to be delivered, - Quantity of parts to be installed in the system, - MTBF, MTTR, TAT, DT of each part, - Category: repairable (repair/replace on site, repair in repair workshop) or non-repairable (discard on failure), - Warranty period (only in case of specifically defined warranty period for certain parts). |  |
| LSSU0030 | If at the end of the warranty period it is proven that the measured spares consumption and/or MTBF is not within the limits as defined in the Contract, the Contractor shall review its spares calculations, as well as upgrade and supply the spares package at its own cost. |  |
| LSSU0040 | The Contractor shall guarantee the shortest possible TAT (turn-around time) for parts sent to the Repair Workshop for repair or exchange, which shall be less than 30 calendar days. |  |
| LSSU0050 | The Contractor shall guarantee the shortest possible DT (delivery time) for parts that will be additionally ordered by the Contracting authority, which shall be less than 60 calendar days. |  |
| LSSU0060 | The Contractor shall deliver all spare parts to the Contracting authority, in the quantity and quality which is in accordance with the Contract requirements, at the latest 7 calendar days before the start of commissioning. These spare parts shall be inspected and tested (except consumables) during the Site Acceptance Test (Guarantee Test). |  |
| LSSU0070 | The Contractor shall notify the Contracting authority if the delivery of a particular type of spares is becoming difficult or if the manufacturing of that part has stopped. Such notification accompanied by a spare parts replacement proposal shall be given at least 6 months in advance. |  |
| LSSU0080 | The above stated requirement (LSSU 0070) shall be valid for parts procured from the Contractor or any of the suppliers involved in the procurement of spare parts for this system. |  |
| LSSU0090 | The Contractor shall include technical customer support maintenance by a professional/specialist ATC radio engineer, based in the Contractor’s premises. Upon contract signature, the Contractor shall provide the contact details (Name/Surname, Title, Position/Department, Telephone, Fax, E-mail address, Office Working time) for the entitled professional mentioned above. |  |

### Mandatory Spare Parts List

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSSL0010 | The Tenderer shall include in its offer a mandatory set of spare parts to enable the efficient seven (7) years maintenance of all parts of the system, and for each site separately. The Contracting authority reserves the right to procure an additional set of spares. |  |
| LSSL0020 | The offered initial set of mandatory spare parts shall be based on the RAM calculation, but not less than: - Three (3) transceivers - Two (2) VHF antenna type 1 - One (1) VHF antenna type 2 - Five (5) Lightning protectors - One (1) SWR monitor |  |
| LSSL0030 | The Tenderer shall include in its offer also a mandatory set of spare parts for the RCMS: - One (1) LAN switch |  |
| LSSL0040 | The offered mandatory set of spares shall be of the same type/model/version as the equipment installed. |  |

### Recommended spare parts

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSSL0050 | The Tenderer is encouraged to propose an additional set of spare parts according to its own experience and maintenance concept, which shall be useful. This list shall be clearly divided from a Mandatory Spares Part list from 3.2.2. |  |

## Warranty

### General

Warranty is the firm and written obligation of the Contractor to fix, at its costs and over an agreed period, the defects and deficiencies occurring on the accepted deliverables (hardware, software, documentation, etc.).

This obligation does not apply when the Contractor can prove that such defect or deficiencies fall outside the warranty coverage.

### Warranty Period

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSWP0010 | The warranty period for all HW and SW mentioned above shall be at least 48 months from the date the Site Acceptance is approved - signing the Site Acceptance Certificate by Contracting Authority and Contractor submits to the Contracting Authority Bank Guarantee for elimination of defaults within warranty period. |  |
| LSWP0020 | The Contractor’s obligation to correct defects and deficiencies shall apply until all the reported and pending defects and deficiencies from the Site Acceptance, as well as from the warranty reporting period, have been corrected in a satisfactory way. |  |
| LSWP0030 | The warranty reporting period for the HW/SW items and documentation shall automatically be extended by the period during which the Radio System (per site) is not operationally available (DOWN TIME). |  |
| LSWP0040 | After the warranty reporting period, any newly detected deficiencies shall not fall under the warranty any longer. |  |

### Obligations of the Contractor

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSWO0010 | If defects and deficiencies affecting the contractually agreed supplies and services arise during the appropriate warranty period, the Contractor shall begin to rectify such defects in compliance with the provisions listed hereinafter. |  |
| LSWO0020 | Where the warranty period is concerned, it is generally agreed that, in case of the Contractor receiving a notification in the time from Monday to Friday, 8:00 to 16:00 hours, rectification of the defects shall begin within 5 working days after the receipt of the Contractor’s notification. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSWO0030 | In this respect the Contractor shall guarantee a support service for the above mentioned periods. Based on a detailed description of the fault, an appropriate system specialist shall, without delay, contact by telephone the Slovenia Control’s personnel who have given the notification of the fault. |  |
| LSWO0040 | Where complex system faults are identified, the project managers shall reach an agreement regarding the suitable location for troubleshooting and regarding deadlines. |  |
| LSWO0050 | Upon expiry of the above deadlines Slovenia Control may rectify the fault by itself, or have it rectified at the Contractor’s expense. The Contractor’s warranty obligation shall thereby remain unaffected if the work has been performed properly and Slovenia Control has informed the Contractor thereof without delay. |  |
| LSWO0060 | The rectification of faults and any other work performed on accepted or operational systems shall be carried out in compliance with the Slovenia Control’s internal work instructions; where applicable, special arrangements will be made in individual cases. |  |
| LSWO0070 | After completion of the repair or modification the Contractor shall demonstrate the compliance with the specification of the HW/SW and Documentation. |  |
| LSWO0080 | A report (including a Version Description Document for SW items) describing the nature of the deficiency/defect, the cause and the corrective measures taken, shall be prepared by the Contractor and submitted to Slovenia Control with the correction. |  |
| LSWO0090 | Replacement parts already delivered to Slovenia Control may be used by the Contractor during the warranty period. |  |
| LSWO0100 | The Contractor shall provide replacements, at no expense to Slovenia Control, for defective parts during the warranty period. |  |
| LSWO0110 | The Contractor shall correct, at no expense to Slovenia Control, all defects in the design of hardware or software during the warranty period, where these defects cause the system not to meet specified performance requirements. |  |
| LSWO0120 | Fixes to the software shall undergo regression testing. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| LSWO0130 | The documentation shall be updated as required to reflect the HW/SW deficiencies corrected under the warranty. |  |
| LSWO0140 | During the warranty period, the Contractor shall provide support to Slovenia Control in solving any problem arising from the license arrangements that the Contractor has negotiated with the vendors in place and name of Slovenia Control. |  |

# Technical System Documentation

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TSDO0010 | The documentation shall be compliant by all means with the requirements laid down in The Interoperability Regulation (EC) No 552/2004. |  |
| TSDO0020 | The documentation shall be in English. |  |
| TSDO0030 | The system documentation for each site shall consist of: - Hardware documentation, - Software documentation, - Operational user manual, - Training documentation - Site installation documentation |  |
| TSDO0040 | The final versions of the system documentation shall consist of 3 hard copies per site and 1 copy on CD or USB stick. |  |
| TSDO0050 | The system documentation shall be written and printed in accordance with either ISO A4 or ISO A3 format standards. All drawings shall be documented in a commonly available CAD utility. |  |
| TSDO0060 | The Tenderer shall specify a list of all system documentation specifying reference numbers (if applicable at the time of tender procurement), description of documentation, delivery milestones in relation to project milestones (Contract Signature, FAT, SAT, etc.) |  |
| TSDO0070 | The Tenderer shall also supply for each site the COTS documentation i.e. a set of technical documents of the system hardware and software units, provided by the manufacturer of COTS products (such as PCs, workstations, modems, network equipment). These documents shall be delivered in the same quantity as the system documentation. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TSDO0080 | Defects, misunderstandings, inadequate or incomplete descriptions and other findings in the documentation, which compromise the quality of the documentation and/or influence the usability of the documentation for the planned purposes, shall be corrected without any extra costs to the Contracting authority. |  |
| TSDO0090 | All documentation to be delivered shall be issued in a final version and provided to the Contracting authority, prior to the Site Acceptance. |  |
| TSDO0100 | The documentation shall include the following: - Equipment description; - Equipment block diagram; - Unit layout in cabinets and cabinet wiring; - Detailed description and purpose of each unit; - Interconnections within units; - Wiring and assembly diagrams down to the LRU/SRU level; - Fault finding diagrams; - Setting-up and operating procedures; - Preventive maintenance and its procedures; - Corrective maintenance and its procedures; - Specification of all equipment units with part number, serial number and manufacturer; - Other information necessary to perform regular and preventive maintenance. |  |
| TSDO0110 | The software documentation (e.g. the Software User Manual or equivalent and the Version Description Document or equivalent) shall contain a description of the system software and shall comprise sufficient details to permit full understanding and maintenance of the system. |  |
| TSDO0120 | The documentation shall provide guidelines for the technical staff to: - Understand the equipment function in detail; - Perform preventive and corrective maintenance of the equipment; - Troubleshoot the system down to the LRU/SRU level; - Install and configure any LRU/SRU in case of complete equipment failure or malfunction of that LRU/SRU; - Perform all necessary measurements and adjustments. |  |

# Training

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TRNG0010 | The training for the equipment shall be fully compliant with the ATSEP.OR.215 System and equipment rating training requirement of Commission Implementing Regulation (EU) 2017/373 of 1 March 2017 laying down common requirements for providers of air traffic management/air navigation services and other air traffic management network functions and their oversight, repealing Regulation (EC) No 482/2008, Implementing Regulations (EU) No 1034/2011, (EU) No 1035/2011 and (EU) 2016/1377 and amending Regulation (EU) No 677/2011 (OJ L 62, 8.3.2017, p. 1–126). |  |
| TRNG0020 | The following training of personnel is required: - System and equipment rating training of existing COM qualified ATSEP. |  |
| TRNG0025 | The Tenderer shall define the course manager and compliance monitioring manager for the TRNG0020 training and their competencies after contract signature. |  |
| TRNG0030 | The Tenderer shall submit in its offer an initial version of the Training Plan. The Training Plan shall be discussed in detail with the Contractor and approved by the Contracting authority in accordance with the agreed schedule. |  |
| TRNG0040 | The Tenderer shall suggest a maximum number of trainees. The number of trainees shall not be less than three (3). |  |
| TRNG0050 | In compliance to the ATSEP.OR.215 as per TRNG0010, the technical staff shall be trained by theoretical and practical on-the-job training as well as to: - Understand equipment architecture and configuration, - Understand operational functions and features of all equipment units, - Understand and use system software application(s) which are part of delivered equipment, - Perform preventive maintenance of the equipment, - Perform corrective maintenance, i.e. troubleshoot the system down to the LRU and SRU or component (where applicable) levels, - Perform system test and alignment procedures for LRU/SRUs, - Perform required measurements and adjustments and to be able to use all necessary tools and instruments, - Understand the diagnostic and test utilities and procedures. |  |
| TRNG0060 | The Training shall address all of the hardware and software delivered in the scope of the Contract, which means that the training for installation, maintenance and operation of COTS products shall be included in the Tender Documentation and in the Training Plan, as well. |  |
| TRNG0070 | The training language shall be English. The Contractor shall provide necessary training on all products delivered within the project. |  |
| TRNG0080 | Contractor shall ensure that personnel engaged in the theoretical and practical on-the-job training of Contracting authority ATSEP shall be fluent in English or Slovenian, and are suitably qualified to perform task of training instructor as well as competence assessor and experienced in the system domains ad per TRNG0050, additionaly, personnel engaged in the practical course shall have multicoupler-filter systems competences as well. |  |
| TRNG0090 | The content of the theoretical (theoretical training syllabus and theoretical training objectives) and practical (practical training syllabus, including practical exercises and the tools and instruments used for the practical hands-on training and pactical training objectives) training courses, and practical on-the-job training during installation (on-the-job training objectives), shall be included in the Training Plan and shall be approved by the Contracting authority. |  |
| TRNG0100 | The training material in electronic form (on CD or USB stick) shall be made available for the trainees at least 1 week before the course begins. Paper copies of the training material shall be available for the trainees at the time the course begins. |  |
| TRNG0110 | The Contracting authority shall have the right to use such material for further courses within its own organis ation. |  |
| TRNG0120 | The system and equipment rating training of ATSEPs shall be applicable to the duties to be performed and include the following: (1) theoretical courses; (2) practical courses; (3) on-the-job training during system installation. |  |
| TRNG0130 | A detailed ATSEP training schedule for the training elments of TRNG0120 shall be defined after contract signature and approved by the Contracting authoirity. |  |
| TRNG0140 | The Tenderer shall conduct a final written examination and practical assessment for each trainee (student) at the end of the training. A Trainee shall be considered as competent ("A trainee successfully completed the course.") for system maintenance and operation if he passes a practical assessment with a score of at least 75% and a theoretical (written) examination of at least 75%. |  |
| TRNG0145 | Contracor shall define the procedure in the event that an individual candidate fails to achieve the training objectives or fails to complete the training within the stipulated time of the training. |  |
| TRNG0150 | After the completion of each course, the following reports shall be provided to the Contracting authority: - final training log book for all training elements of TRNG0120 signed by the Contractors head of training; - Each student's performance during each training element of TRNG0120; - A summary report for practical assessment of each student; - a copy of theoretical examination for each student; - An attendance report; - Certificate of competence for each student who successfully completed a course with a score of at least 75% for each exam. |  |
| TRNG0160 | The technical staff shall be trained in the first phase (see chapter 1.2 “Scope”) several parts: - Prior to FAT (Factory Acceptance Test) to be able to take part in Factory Acceptance Tests, held in the Contractor’s training facilities (Factory Training); - During the system installation, which will be considered as an on-the-job-training, as it is intended that trainees participate in installation activities as much as possible, while the Contractor shall make the supervision and have the responsibility of correct system installation. The Contractor is required to perform at least 12 hours of pre-installation training as part of theoretical training in order that the Contracting authority’s technical staff will be able to actively participate in the system installation. |  |
| TRNG0170 | The duration of the required ATSEP theoretical training course and practical training course shall last 10 working days with 75 training hours at least, including pre-instalation part of theoretical training as per TRNG0160. |  |
| TRNG0180 | The theoretical training course programme shall at least consist of: - Radio device training (Tx, Rx, TRx): theory of operation, block-schemes, front and back panels, functions, menus, adjustments, connections/integration/installation, practical tips etc. Practical “hands-on” training is required. - Dividers, multicouplers, filters, VSWR meters training: theory of operation, block-schemes, functions, adjustments, connections/integration/installation, practical tips, etc. Practical “hands-on training” is required. - RCMS sub-system training: installation (HW & SW), troubleshooting, functions, menus, applications, site/equipment management, user/password management etc. - 19’’ equipment cabinets: cabling installation, wiring and cabling, equipment fitting and connecting, AC/DC power distribution and cabling, grounding and fusing, EMP protection, etc. |  |
| TRNG0190 | The practical training course focoused to the multicoupler-filter system tuning/cable phase matching training shall address at least: - Design of combiner/multicoupler-cavity filter systems; - Tx/Tx and Tx/Rx isolation techniques using filters and isolators; fighting intermodulation’s; - Impact of Tx/Rx frequency change (re-tuning) on filters, couplers and critical length cables in various examples; - Cable phase matching theory and methodology (calculations and measurements); - Cable phase matching techniques, usage of tools and instruments. |  |
| TRNG0200 | Contracting authority covers travel and accommodation costs, incurred with attendance of Contracting authority’s technical staff to training. All other costs, related to training, are included in contractual price. Contracting authority unilaterally designates trainees. |  |
| TRNG0210 | The Contractor shall issue Certificates for the ATSEP personnel who successfully completed the System and Equipment Rating Training in accordance with the ATSEP.OR.215. |  |
| TRNG0215 | The Contractor shall monitor compliance of training to the requirements from TRNG0010 to TRNG0210. |  |
| TRNG0220 | The Contractor shall grant the Contracting authority access to the premises and applicable documentation for the purposes of TRNG requirememts compliance monitoring by the Contracting authority, including access to the documentation of TRNG0215 compliance monitoring condiucetd by the Contractor itself. |  |

# Safety and Quality Assurance

## Safety

### General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SARQ0010 | When creating the safety documentation, including a safety plan and safety reports, the Contractor shall take into account these ATM and programme related applicable standards/regulations/directives (and any amendments of documents released before the establishment of the contractual relationship in which this document is referenced): - Regulation (EC) No 552/2004 of the European Parliament and of the Council of 10 March 2004 on the interoperability of the European Air Traffic Management network (the interoperability Regulation) (OJ L 96, 31.3.2004, p. 26–42); Latest consolidated version: 11/09/2018; - Regulation (EC) No 1070/2009 of the European Parliament and of the Council of 21 October 2009 amending Regulations (EC) No 549/2004, (EC) No 550/2004, (EC) No 551/2004 and (EC) No 552/2004 in order to improve the performance and sustainability of the European aviation system (OJ L 300, 14.11.2009, p. 34–50); - All Directives, Community Specifications and Implementing Rules related to 3. and 4.; - EUROCONTROL Guidelines on conformity assessment for the Interoperability Regulation of the Single European Sky (IOP, EUROCONTROL-GUID-137), 2012-02-20, Edition 3.0; additionally, the Contractor shall provide the verification specifications and the verification records upon request; - IEC 61078 (RMA) Analysis techniques for dependability - Reliability block diagram and Boolean methods; - AIR NAVIGATION SYSTEM SAFETY ASSESSMENT METHODOLOGY (SAM) of EUROCONTROL, 2.2 - Regulation (EU) No 376/2014 of the European Parliament and of the Council of 3 April 2014 on the reporting, analysis and follow-up of occurrences in civil aviation, amending Regulation (EU) No 996/2010 of the European Parliament and of the Council and repealing Directive 2003/42/EC of the European Parliament and of the Council and Commission Regulations (EC) No 1321/2007 and (EC) No 1330/2007 (OJ L 122, 24.4.2014, p. 18–43); Consolidated version: 11/09/2018; - Commission Implementing Regulation (EU) 2017/373 of 1 March 2017 laying down common requirements for providers of air traffic management/air navigation services and other air traffic management network functions and their oversight, repealing Regulation (EC) No 482/2008, Implementing Regulations (EU) No 1034/2011, (EU) No 1035/2011 and (EU) 2016/1377 and amending Regulation (EU) No 677/2011 (OJ L 62, 8.3.2017, p. 1–126); Consolidated version: 05/11/2020. |  |
| SARQ0020 | Safety reports shall include the safety recommendations to Slovenia Control, as a possible means for risk mitigation. This shall include procedural and human mitigations, e.g. training, shift management, maintenance procedures, operational procedures etc. |  |
| SARQ0030 | The safety documentation shall be provided to the Contracting authority for review and approval in accordance with the milestones specified in the Documentation Plan. |  |

### Safety Audit

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SASA0010 | The safety audit shall be conducted by the Contracting authority in duration of approximately one working day according to the Contracting authority’s safety audit plan previously delivered, subject to mutual agreement. |  |
| SASA0020 | The safety assurance process shall be audited and the Contractor shall provide full support to the Contracting authority’s safety audit team including access to evidence and arguments. |  |

### Safety Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SASP0010 | The Contractor shall provide a Safety Plan which shall include all activities to be performed by the Contractor to meet the requirements related to safety, in such a way that the delivered technical system is safe for operation and minimises the risks which may contribute to aircraft accidents as far as reasonably practicable. |  |
| SASP0020 | A preliminary version of the Safety Plan shall be delivered at the latest 30 days after contract signature, and shall be reviewed and approved by the Contracting authority. |  |
| SASP0030 | Safety activities to be specified in the Safety Plan shall be carried out to cover: - All hardware and software to be delivered (components to be developed, procured, modified, or re-used), up to external interfaces of the system, - Safety related procedures and training of the Slovenia Control’s staff, - The whole time span of the Project and all activities with safety significance, i.e. the system specification, design, development, integration, installation, acceptance, commissioning, transition to operation and maintenance of the system. |  |
| SASP0040 | The Contractor shall produce updates of the Safety Plan, if necessary. |  |

### Safety Reports

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SASR0010 | Safety reports shall cover the Safety Assessment process, containing as a minimum, the following: - Hazards identification and analysis for each system function, including the determination of hazard likelihood and severity, and possible effect on operation, - Identification of possible effect on operation for each hazard, - Identification of risk mitigation measures for each hazard. |  |
| SASR0020 | Safety Reports shall be produced in accordance with the Safety Plan. |  |
| SASR0030 | Safety Reports shall include, as a minimum, the following set of documents and due dates: - Functional Hazard Assessment Report (FHAR), due date: 30 days after contract signature; - Preliminary System Safety Assessment Report (PSSAR), due date: 2 calendar weeks before the FAT starting date; - System Safety Assessment Report (SSAR), due date: 1 calendar week before installation of the target system; - Safety Issue Log (SIL). The document shall contain all safety critical issues which shall be mitigated in a procedural or human related matter. Due date: Start of the equipment installation; - Software Safety Folder. The Contractor shall use the Guidelines for ANS Software Safety Assurance, EUROCAE document ED-153. |  |

### Software Safety Requirements

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| SASS0010 | These ATM and programme related applicable standards/regulations/directives (and any amendments of documents released before the establishment of the contractual relationship in which this document is referenced) are applicable for software safety assurance: - Annex IV to ED Decision 201/022/R of 30 Oct 2019 establishing a software safety assurance system to be implemented by air navigation service providers; - As a means of compliance with Annex IV to ED Decision 201/022/R ED-153/Eurocae GUIDELINES FOR ANS SOFTWARE SAFETY ASSURANCE. The Contractor shall produce evidence and arguments demonstrating that: - the software safety requirements correctly state what is required by the software, in order to meet safety objectives and requirements, as identified by the risk assessment and mitigation process; - traceability is addressed in respect of all software safety requirements; - the software implementation contains no functions which adversely affect safety, particularly there must not be the CSCI whose single failure would induce the effect with severity class 1 as per ESARR 4; - the software satisfies its requirements with a level of confidence which is consistent with the software criticality; - assurances are provided confirming that the general safety requirements set out in the previous points are satisfied, and the arguments that demonstrate the required assurances are at all times derived from: - a known executable version of the software; - a known range of configuration data; - a known set of software products and descriptions, including specifications that have been used in the production of that version. | h |
| SASS0020 | The Contractor shall allocate software assurance levels (SWAL) to all operational software, in compliance with the following: - The software assurance level shall relate the rigour of the software assurances to the software criticality by using the severity classification scheme of Contracting authority combined with the likelihood of occurrence of a certain adverse effect. A minimum of four software assurance levels shall be identified, with software assurance level 1 indicating the most critical level (if such software exists); - An allocated software assurance level shall be commensurate with the most severe effect that software malfunctions or failures may cause. This shall, in particular, take into account the risks associated with software malfunctions or failures and the architectural and/or procedural defences identified. - Software components that cannot be shown to be independent of one another shall be allocated the software assurance level of the most critical of the dependent components. |  |
| SASS0030 | To assure software safety requirements validity, the Contractor shall describe the functional behaviour of software in nominal and downgraded modes, timing performances, capacity, accuracy, software resource usage on the target hardware, robustness to abnormal operating conditions and overload tolerance, as appropriate. Software safety requirements shall be complete and correct, and compliant with the system safety requirements. |  |
| SASS0040 | To assure the software safety requirements verification, the Contractor shall ensure that: - The software functional behaviour, timing performances, capacity, accuracy, software resource usage on the target hardware, robustness to abnormal operating conditions and overload tolerance, shall comply with the software requirements. - The software shall be adequately verified by analysis and/or testing and/or equivalent means. - The software verification shall be correct, complete and documented. |  |
| SASS0050 | To assure the software configuration management, the Contractor shall ensure that: - Configuration identification, traceability and status accounting facilitate that the software life cycle data is shown to be under configuration control throughout the software life cycle. - Problem reporting, tracking and corrective actions facilitate that safety related problems associated with the software are shown to have been mitigated. - Retrieval and release procedures facilitate that the software life cycle data is regenerated and delivered throughout the software life cycle. |  |
| SASS0060 | To assure the software safety requirements traceability, the Contractor shall ensure that: - Each software safety requirement is traced to the same level of design at which its satisfaction is demonstrated. - Each software safety requirement, at each level in the design at which its satisfaction is demonstrated, is traced to a system safety requirement. |  |
| SASS0070 | The assurances from SASS0030 to SASS0060 shall include the rigour for each software assurance level which shall increase as the software increases in criticality. For that purpose: - the variation in rigour of the assurances per software assurance level must include the following criteria: - required to be achieved with independence; - required to be achieved; - not required; - the assurances corresponding to each software assurance level must give sufficient confidence that the software can be operated tolerably safely. |  |
| SASS0080 | For any software (such as COTS, non-developmental software or previously used software, etc.), for which some of the requirements cannot be applied, the Contractor shall provide, through other means, the same level of confidence as the relevant software assurance level whenever defined. Those means must give sufficient confidence that the software meets the safety objectives and requirements, as identified by the safety risk assessment and mitigation process. |  |

## Quality

### General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| QGEN0010 | The Tenderer shall be certificated, holding a valid ISO 9001:2015 or equivalent Quality Management System Certificate. |  |
| QGEN0020 | The Tenderer shall submit documentary evidence in the Tender establishing to the Contracting authority satisfaction that the Tenderer has a valid Quality Management System certificate. |  |

### Quality Assurance Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| QAAP0010 | The Contractor shall develop a Quality Assurance Plan, subject to the approval by the Contracting authority. |  |
| QAAP0020 | The Quality Assurance Plan shall describe the organisation, processes, tasks and responsibilities with respect to quality assurance. |  |
| QAAP0030 | The Quality Assurance Plan shall identify the documents to be produced in appropriate phases of the lifecycle (see also the Project Management Plan), and shall state how these documents are checked for adequacy. |  |
| QAAP0040 | The Quality Assurance Plan shall identify the standards, practices, conventions and metrics to be applied and shall state how compliance with these items is to be monitored and assured. |  |
| QAAP0050 | The project applicable and referenced documents and standards shall be listed in the Quality Assurance Plan, with their title and version number. |  |
| QAAP0060 | The Preliminary Quality Assurance Plan shall be provided by the Contractor to the Contracting Authority within 10 working days from the day of the signature of the contract on. |  |
| QAAP0070 | The final version of the Quality Assurance Plan shall be available within 60 days from contract signature. |  |

### Quality Audit

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| QAQA0010 | The Quality Assurance processes, technical documentation and products produced shall be subject to quality audit. The date, schedule and scope shall be mutually agreed. |  |
| QAQA0020 | The duration of quality audit shall be approximately one working day. |  |
| ID | Requirements | Compliance YES/NO – references to evidence |
| QAQA0030 | The Contractor shall provide full support to the Contracting authority’s quality audit team including access to evidence and arguments. |  |

# Project Management

## General

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMGE0010 | The Contractor shall be responsible for the management, performance, monitoring and coordination of the whole project from the project kick-off until the completion of the contract. |  |
| PMGE0020 | The Contractor shall establish a project organization in accordance with the requirements included herein, having necessary resources, qualification and experience to fulfill all of its obligations. |  |
| PMGE0030 | The Tenderer shall define and describe the part of its organization which will manage or be involved in the project. |  |
| PMGE0040 | The Contractor shall appoint the Contractor’s Representative, who will be an interface to the Contracting authority's Project Manager, and be at the Contracting authority’s disposal for all matters relating to contract execution. |  |
| PMGE0050 | In case another person will act as his substitute, the Contracting authority shall be notified at least 2 weeks in advance of the other person proposed to substitute him. |  |
| PMGE0060 | The Contractor's Representative (or his substitute) shall be present at all meetings during the contract execution. |  |
| PMGE0070 | The Contractor's Representative shall be responsible for project co-ordination and will take all necessary actions to ensure the project progress according to the agreed schedule. |  |
| PMGE0080 | Communication between the Contractor and the Contracting authority shall be in accordance with the Contracting authority's practice. Details of a Communication Procedure between the Contractor and the Contracting authority shall be defined by the Contracting authority. Concluded contract defines contact persons. |  |
| PMGE0090 | If deemed necessary during contract execution, the Contracting authority or the Contractor can propose progress meetings. |  |
| PMGE0100 | The agenda for this meeting(s) shall be mutually agreed and prepared at least 3 working days prior the date of a meeting. |  |
| PMGE0110 | The following persons shall be present at progress meetings: - The Contractor's Representative, - The Contracting authority's Project Manager, - Any other persons who the above representatives consider important to be present. |  |
| PMGE0120 | The venue of meetings shall be premises of Contracting authority or location (Portorož, Maribor, Janče). Contractor covers all costs, related to attendance to the meeting (travel costs etc.). Meeting can be held on alternative location, if so agreed in writing by Contracting authority. |  |
| PMGE0130 | The Contractor shall prepare minutes of the meetings and submit them for approval to the Contracting authority not later than 3 working days after the meeting finishes. |  |
| PMGE0140 | The Contractor shall issue and manage a Progress Chart (Master Time Schedule). The contract shall be executed in accordance with the progress chart. This chart shall be set out in the form of a linear timetable (preferably using the MS Project). The starting date is to be the date of entering the contract into force. |  |
| PMGE0150 | The Progress Chart shall specify dates of all major actions and decisions to be taken by both the Contracting authority and the Contractor. Any alterations to this chart shall be examined and mutually agreed. |  |
| PMGE0160 | The initial progress chart shall be included in the tender documentation. |  |
| PMGE0170 | The progress chart shall be agreed between the Contractor and the Contracting authority and shall be kept updated during ontract execution. The version control of an electronic file of the Progress Chart shall be established. |  |
| PMGE0180 | The Contractor shall establish close coordination with the Contracting authority for the development of all planning activities related to the project, forwarding relevant plans, procedures, etc. for review and approval, prior to putting them into force. |  |
| PMGE0190 | The Contractor shall prepare at least the following Project Plans at the appropriate stage in the project for review and approval by the Contracting authority: - Project Management Plan (PMP), - Logistic Support Plan (LSP), - Documentation Plan (DP), - Safety Assurance Plan (SAP), - Quality Assurance Plan (QAP), - Training Plan (TP), - Installation and Commissioning Plan (IP), - FAT Plan and Procedures, - SAT Plan and Procedures. |  |

## Project Management Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMPM0010 | The Contractor shall prepare a Project Management Plan (PMP) in accordance with the requirements included in the present chapter. |  |
| PMPM0020 | A draft version of the PMP shall be provided as part of the tender documentation. An initial official version of the PMP shall be provided by Contractor within 10 working days from the signature of the contract on. |  |
| PMPM0030 | The Project Management Plan shall include at least the following: - Project scope and overview, - Project deliverables (shall also include a Documentation List), - Work Breakdown Structure (Shall define the scope of work and resources necessary to meet the Contract requirements. The work breakdown shall also include the work to be performed by the Contracting authority, e.g. participation in reviews and tests, preparation of data.), - Project organisation and responsibilities, - Master Time Schedule (Progress Chart), - Quality assurance activities to be performed in the project by the Contractor, - Configuration management activities (regarding hardware, software and documentation version changes). |  |
| PMPM0040 | Any change to the PMP or to the processes outlined in it will be subject to the formal Contracting authority’s approval. The PMP shall be kept up-to-date. |  |
| PMPM0050 | The PMP shall normally include at least the following aspects: - Management procedures and practices; - Work breakdown structure (WBS); - Master time schedule, showing dates and deliverables; - Master resource plan, showing roles and responsibilities; - Key risks that might jeopardize the project. |  |

## Documentation Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMDP0010 | A Documentation Plan shall specify the list of all documents to be delivered to the Contracting authority during contract execution. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMDP0020 | A draft version of the Documentation Plan shall be provided by the Tenderer in the tender documentation. An initial official version of the Documentation Plan shall be provided by the Contractor within 10 working days from the day of the signature of the contract on. |  |
| PMDP0030 | The Documentation Plan shall include at least the following: - Title of document, - The Contractor, - Applicability of the document (e.g. entire system, a certain subsystem or a particular COTS product, hardware documentation, software documentation, etc.), - Reference and version number of the document, - Dates of each document delivery. |  |

## Logistic Support Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMLS0010 | The Contractor shall prepare a Logistic Support Plan (or equivalent) which shall specify in more detail logistic aspects of the Contract. The Logistic Support Plan shall include: - Maintenance concept of the system, - Maintenance activities, levels and responsibilities (maintenance flows overview), - Overview of the technical documentation to be provided, especially stating the manuals with maintenance procedures, - Spare Parts List which shall meet the requirements of this Technical Specification, - Relation of maintenance activities with the skills obtained at technical training, - Tools and test equipment list to be provided and/or required, - All aspects of warranty support including a detailed description of the scope of warranty support, - Possible post-warranty support arrangements regarding the system maintenance, - Overview of recommended preventive maintenance actions. |  |
| PMLS0020 | A draft version of the Logistic Support Plan shall be provided by the Contractor within 10 working days from the day of the signature of the contract on. |  |

## Installation and Commissioning Plan

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMIM0010 | On each site where installation is going to take place, the Contractor shall prepare an Installation and Commissioning Plan comprising: - The Contractor’s scope of work, - Sub-contractors involved and their scope of work (if applicable), - The Contracting authority’s scope of work, - Tasks to be performed and the person(s) responsible for each task, - Timing of the tasks, - Documentation (e.g. instructions, specifications, drawings, interconnection diagrams and other relevant information for installation), - Other information important for the final installation. |  |
| PMIM0020 | An initial (draft) version of the Installation and Commissioning Plan shall be provided in the tender documentation. |  |
| PMIM0030 | The final Installation and Commissioning Plan plan shall be submitted at least 4 weeks and approved by the Contracting authority at least 3 weeks before the start of installation activities. |  |

## Acceptance Test Plans and Procedures

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMAP0010 | The Contractor shall provide acceptance test plans and procedures (test specifications) for Factory Acceptance and Site Acceptance, including a detailed description of the proposed test techniques and procedures to verify all equipment parameters, together with a schedule for such tests. The following documents shall be provided: - Factory Acceptance Test Plan and Procedures, - Site Acceptance Test Plan and Procedures. |  |
| PMAP0020 | These acceptance test plans and procedures (test specifications) shall at least contain the following: - A schedule of the actions to be taken in the testing of various parts of the equipment, - The forms of documentation of test results, - The condition under which the tests shall be conducted and approved, - A detailed description of the tests to be performed. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| PMAP0030 | The FAT Plan and Procedures shall be submitted to the Contracting authority at latest 4 weeks before the FAT. The document shall be reviewed by Contracting authority and shall be amended or changed if necessary, and approved by both parties 2 weeks before the FAT at the latest. |  |
| PMAP0040 | The Site Acceptance Test Plan and Procedures shall be submitted to the Contracting authority at latest 4 weeks before the SAT. The document shall be reviewed by Contracting authority and shall be amended or changed if necessary, and approved by both parties 2 weeks before the SAT at the latest. |  |

# Installation and Commissioning/Transition

## Installation

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| INST0010 | The planning of the system installation and setting up shall be developed in close cooperation with the Contracting authority. |  |
| INST0020 | The Installation and Commissioning Plan, specifying all installation and transition activities, shall be approved by the Contracting authority prior to the installation. |  |
| INST0030 | The Contractor can perform a site survey before the installation takes place in order to identify necessary works to be performed, if deemed necessary. |  |
| INST0040 | The Contracting authority will perform, prior to the installation, a complete physical check of the goods received (equipment, accessories, spare parts, documentation, etc. – all according to the Contract specification), and will notify the Contractor if the delivery does not fulfill the requirements listed in the Contract Specification. |  |
| INST0050 | The Contractor shall have full responsibility for the system installation and setting up. |  |
| INST0060 | The Contractor shall produce, procure and supply all necessary equipment, tools, etc., consumable as well as non-consumable, needed for the installation and setting-up. |  |
| INST0070 | Installation on the site shall be conducted by Contractor’s skilled staff with support of the Contracting authority’s technical staff under close Contractor’s supervision. These activities shall form On-the-Job Training of the Contracting authority’s staff that shall be carried out during the installation in accordance with the agreed conditions given in the Training Plan. |  |
| INST0080 | The installation shall take into consideration all applicable local legislation rules and procedures. |  |
| INST0090 | The Contractor shall prepare a list of staff (for non-Slovenian residents) to conduct the installation activities, as well as all necessary documents early enough in order to obtain their working permits in the Republic of Slovenia (local legislation rules and procedures). |  |
| INST0100 | The Contractor shall state in the Installation and Commissioning Plan the aspects of the installation to be included in the documentation concerning: - Co-location aspects (where applicable); - Cabling arrangements, routing, identification; - EMC compatibility, Interference, susceptibility to foreign radio frequencies; - Earthing arrangements; - Equipment mounting, cooling, etc. |  |
| INST0110 | The Contractor shall specify all the facilities and procedures the Contracting authority has to provide for test purposes. |  |

## Pre-Commissioning and Completion

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TCOM0010 | The Contractor shall issue a written notice to the Contracting authority when the installation within the site is finished and commissioning may commence. |  |
| TCOM0020 | The Contracting authority shall conduct an inspection of the site and the installation itself. |  |
| TCOM0030 | If some defects or deficiencies are found during the site inspection, the Contracting authority’s Project Manager shall notify the Contractor’s representative and list all of them in a Completion Report. The Contractor shall take necessary remedial actions at its own expense, and the procedure shall be repeated. |  |

## Commissioning and Transition

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TTRA0010 | The Contractor shall be responsible for the commissioning and transition to the new system. Transition activities shall be performed in accordance with the final Installation Plan. |  |
| TTRA0020 | Transition to the new system shall be performed in a way that the functionality of the existing equipment is not significantly endangered at any moment in order to avoid/minimise the interruption of operational service. |  |

# Testing and Acceptance

## Factory Acceptance

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TFAT0010 | Factory acceptance shall be performed to verify that the equipment fully complies with the specification requirements. Non-complying equipment shall be rejected. |  |
| TFAT0020 | Factory acceptance shall be normally witnessed by the Contracting authority's representative(s), if not decided otherwise. |  |
| TFAT0030 | If the Contracting authority's representative(s) does/do not witness the Factory acceptance testing, the Contractor shall perform Factory Acceptance Tests without the presence of the Contracting authority’s technical staff, issue a FAT Report (results and protocols) and send it to the Contracting authority via official communication procedure. |  |
| TFAT0040 | All factory acceptance tests shall be normally carried out before the system installation and on the basis of a test specification submitted to Contracting authority by the Contractor at least 4 weeks before the FAT. |  |
| TFAT0050 | The Contractor shall provide a test specification (FAT Plan and Procedures) including a detailed description of the proposed test techniques and procedures to verify all equipment parameters, conditions under which the tests shall be conducted and approved, the forms of documenting test results, together with a schedule for such tests. |  |

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TFAT0060 | All equipment and sub-units shall be fully interconnected and built up into a complete system configuration. Specific tests considered as impracticable within the given system configuration will be performed using simulated inputs/outputs or a test bench when specifically approved by the Contracting authority. |  |
| TFAT0070 | All FAT procedures shall comply with and be performed according to the applicable standard documents, recommended practices or procedures (ICAO, ETSI, ITU, EUROCAE, CEI…). |  |
| TFAT0080 | The Contracting authority reserves the right to request some further tests to be performed (which are not listed in the FAT specification) if deemed necessary. These tests shall be also noted in the FAT report. |  |
| TFAT0090 | If a failure occurs during acceptance testing, the Contractor shall take necessary remedial actions at its own expense, and all relevant tests shall be repeated unless the Contracting authority decides otherwise. |  |
| TFAT0100 | After successful completion or unsuccessful attempt of the FAT, the FAT Report (including measurements) shall be immediately prepared and issued by the Contractor. The FAT report shall be signed by both the Contractor’s and Contracting authority’s FAT representatives. |  |
| TFAT0110 | After successful completion of the FAT, the FAT Certificate shall be issued by the Contractor. The FAT Certificate shall be prepared and signed by Slovenia Control’s Project Manager not later than 7 days after FAT completion and sent to the Contractor’s Representative according to the official communication procedure. |  |

## Site Acceptance

| ID | Requirements | Compliance YES/NO – references to evidence |
| --- | --- | --- |
| TSAT0010 | The purpose of a Site Acceptance Test (Guarantee Test) is to demonstrate Compliance of the delivered equipment and proper functioning of the system after commissioning, specified in the Contract. |  |
| TSAT0020 | The SAT shall be carried out after the system installation on each site on the basis of a test specification (SAT Plan and Procedures). |  |
| TSAT0030 | The Contracting authority reserves the right to request some further tests (site specific) to be performed (which are not listed in the SAT specification) if deemed necessary. These tests shall be also noted in the Site Acceptance Report. |  |
| TSAT0040 | The Site Acceptance testing (Guarantee Test) shall be performed by the Contractor’s and Contracting authority's representatives. |  |
| TSAT0050 | If a failure occurs during acceptance testing, the Contractor shall take necessary remedial actions at its own expense, and all relevant tests will be repeated unless the Contracting authority decides otherwise. |  |
| TSAT0060 | The Contractor is responsible for providing all test equipment (hardware and/or software) necessary for the tests. The test facilities provided by the Contractor are not part of the delivery. |  |
| TSAT0070 | All delivered spare parts shall be tested in real operation during the Site Acceptance Test. |  |
| TSAT0080 | The possibility of quick replacement of line replaceable units (LRUs) shall be demonstrated on the Site Acceptance Test (as a part of the On-the-Job training). |  |
| TSAT0090 | After successful completion or unsuccessful attempt of each Site Acceptance, a Site Acceptance Report (including measurements) shall be prepared and issued by the Contractor as soon as possible, but no longer than 7 days. The Site Acceptance Report shall be signed both by the Contractor’s and Contracting authority’s Site Acceptance representatives and approved by the Contracting authority’s Site Acceptance representative appointed. |  |
| TSAT0100 | Following satisfactory completion of all Site Acceptance Tests (Guarantee Test) the Contractor shall offer the System for formal acceptance by the Contracting authority. |  |
| TSAT0110 | Authorized Slovenia Control representatives shall grant the Site Acceptance if the following conditions are satisfied: - The inventory and a complete physical check of the goods received have shown that the delivery in all aspects fulfills the requirements listed in the Contract Specification; - The system has been installed correctly; - The documentation has been supplied and is in Compliance with the Contract Specification; - Training of the Slovenia Control’s staff has been carried out in accordance with the agreed conditions; - Spare parts have been delivered according to the Contract Specification and have been tested in real operation; - Site Acceptance tests have successfully been completed; - Signed and approved Site Acceptance Report has been issued. |  |
| TSAT0120 | All Site Acceptance Report problems and observations shall be closed or a relevant action assigned and agreed. |  |
| TSAT0130 | All Certificates of Conformance and Suitability for use shall be provided by the Contractor for all deliverable items (including software). |  |
| TSAT0140 | The Site Acceptance shall be granted by issuing a Site Acceptance Certificate by the Contractor. The Site Acceptance Certificate shall be prepared and signed by Slovenia Control’s Project Manager not later than 7 days after completion of Site Acceptance and sent to the Contractor’s Representative according to the official communication procedure. |  |
| TSAT0150 | If any kind of certificate regarding the equipment will be requested by Civil Aviation Authority such document shall be provided by Contractor at no cost for Contracting Authority. |  |

# Drawings and pictures

## Block schemes

### Block scheme, site Maribor and Portorož

Drawing 1: Site Maribor and Portorož

### Block scheme, site Janče

Drawing 2: Site Janče

## Principal positioning

### Principal positioning of antennas, site Maribor

Picture 3: Antenna system at the site Maribor

### Principal positioning of antennas, site Portorož

Picture 4: Antenna system at the site Portorož

### Principal positioning of antennas, site Janče

Picture 5: Antenna system at the site Janče