Project name: Upgrading national research infrastructures – RIUM

Technical Specifications for Equipment for Processing and Monitoring the Machining and Forming Processes

- SUBJECT OF PUBLIC TENDER

LOT 3: Laser Measuring Systems for Machine Tools and Coordinate Measuring Machines

Due to the tendency towards increasing accuracy, the laser system would be used to check and adjust the machine tool. You could perform the alignment of the individual parts and measure the flatness and straightness of the individual guides. This would ensure the smallest possible deviations in the manufacture of the product. Periodic assessment of the condition of the machine will ensure continuous and maximum machine performance.

- TECHNICAL REQUIEREMENTS

Delivery deadline:

LOT 3: The supplier must deliver the equipment that is the subject of this public procurement and carry out all activities for the successful implementation of item 3.1 of the technical specifications at the location University of Maribor, Faculty of Mechanical Engineering, Smetanova ulica 17, 2000 Maribor, no later than 60 days from the entry into force of the contract.

Warranty period:

LOT 3: For the supplied equipment, the supplier provides at least 1 (one) year warranty for a flawless technical operation.

Lot 3: Laser Measuring Systems for Machine Tools and Coordinate Measuring Machines

| Item number | Number of pieces | Required |
| --- | --- | --- |
| 3.1 | 1 | - Three-channel laser-interferometer; at least 40MHz sampling rate per channel, parallel processing; stabilized DFB diode laser (1510nm -1560nm); working distance: up to 1000mm; position measurement of travelling targets: up to 1 m/s; USB interface; Pilot-laser for simple aligning with red wavelength (650 nm). - Windows interface, (LabVIEW driver and Windows DLL). - Environmental module sensors for temperature, pressure and humidity. - 3 x sensor head: working distance up to 650 mm or more. - Mounting components for interferometers. - Maintenance and repair support. - Technical support. |
| 3.2 | 1 | Delivery and installation: - delivery of equipment to the address Smetanova 17, 2000 Maribor, laboratory C101. |
| 3.3 | 1 | Warranty: - at least 1 year. |
| 3.4 | 1 | Delivery time: - Maximal 60 days from the entry into force of the contract. |

The operation is co-financed by the Republic of Slovenia, the Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is implemented under the Operational Program for the Implementation of European Cohesion Policy 2014-2020 under Priority Axis 1: "International competitiveness of research, innovation and technological development in line with smart specialization for greater competitiveness and greening of the economy", priority investment 1.1 "Strengthening infrastructure for research and innovation and capacity to develop excellence in this field, as well as the promotion of competent centers, especially those of European importance ", specific objective 1.1.1" Effective use of research infrastructure and development of knowledge/competences for better national and international cooperation in the triangle knowledge ”.