Project title: Upgrading National Research Infrastructures – RIUM

COMPLIANCE DECLARATION WITH MINIMUM TECHNICAL REQUIREMENTS OF THE OFFERED EQUIPMENT – LOT 6

Name of the tenderer:

Address of the tenderer:

Subject of the public procurement: Purchase of Advanced Equipment for Development of Special Optical Fibers

Lot 6: The preform processing system

Designation of the public procurement: 302/41 - RIUM56-FERI 03.1/2021

INSTRUCTIONS:

- This Annex No. 2 must be completed and signed. The tenderer must indicate the actual technical specifications of the equipment offered in line with all requirements.

- The tenderer shall upload this Annex No. 2 to the e-JN information system in the section “Other Annexes”.

We declare that the offered equipment fully meets the minimum technical requirements of the client for the public procurement titled “Purchase of Advanced Equipment for Development of Special Optical Fibers” - Lot 6, and that we are ready to realize the order according to the following specification:

|  | EQUIPMENT |  |  |  |
| --- | --- | --- | --- | --- |
| No. | REQUIREMENTS | OFFERED |  | DATASHEET |
| 1 | Basic machine requirements: |  |  |  |
|  | - Work area: - Traverse distances (X/Y/Z): minimal 650/520/475 mm. | Traverse distances (X/Y/Z): _____ /_____/_____ mm |  |  |
|  | - Feed rate (X/Y/Z): ≥ 40 m/min. | Feed rate (X/Y/Z): _____________ m/min |  |  |
|  | - Table load: ≥ 300 kg. | Table load: _____________ kg |  |  |
|  | - Swivel/rotary table for simultaneous machining. | - YES | - NO |  |
|  | - Swivel range: minimal from -35° to +110°. | Swivel range: from ________° to _________° |  |  |
|  | - Direct drive C-axis for circular grinding operations. | - YES | - NO |  |
|  | - Table diameter surface: ≥ 650 mm. | Table diameter surface: ________ mm |  |  |
|  | - Table torque: ≤ 300 U/min. | Table torque: ________ U/min |  |  |
|  | - Torque max.: - ≥ 500 Nm at ≥ 100 U/min, - ≥ 200 Nm at ≥ 200 U/min, - ≥ 100 Nm at ≥ 300 U/min | - YES | - NO |  |
|  | - Cabin: - Half-cabin with sliding door and roof (closed) - Machine lamp: minimal 2 LED lamp (planonlight), minimal 33 Watt | - YES | - NO |  |
|  | - Operating hour meter: Meter at the electric control cabinet for „control voltage ON“ and “Program in operation” | - YES | - NO |  |
|  | - Central lubrication: automatic minimal lubrication for roller guides and ball screw spindles | - YES | - NO |  |
|  | Main drive for conventional milling: |  |  |  |
|  | - Drive type: direct drive. | - YES | - NO |  |
|  | - Speed: 20 - 15,000 min-1. | - YES | - NO |  |
|  | - Motor spindle: HSK-A63/DIN 69893-1. | - YES | - NO |  |
|  | - Bearing: O-arrangement. | - YES | - NO |  |
|  | - Tool magazine: ≥ 25 places HSK-A63 | - YES | - NO |  |
|  | - Drive capacity: minimal 21 kW / 28 hp (40% duty cycle); minimal 16 kW / 21.5 hp (100% duty cycle). | - YES | - NO |  |
|  | - Spindle torque: minimal 111 Nm / 82 ft lbs (40% duty cycle); minimal 85 Nm / 63 ft lbs (100% duty cycle). | - YES | - NO |  |
|  | - Spindle cooling: active. | - YES | - NO |  |
|  | - Lubrication: oil mist. | - YES | - NO |  |
|  | - Air blast through spindle center activated by M function. | - YES | - NO |  |
|  | Ultrasonic drive and technology for hard machining: |  |  |  |
|  | - Ultrasonic technology for hard machining. | - YES | - NO |  |
|  | - Speed: maximal 15,000 min-1. | Speed: ________________ min-1 |  |  |
|  | - Transformation of the ultrasonic high frequency from the spindle to the HSK-A63 tool holder via induction. | - YES | - NO |  |
|  | - Stimulation of the Piezo elements inside the oscillation motor (generation of a vibration in Z). | - YES | - NO |  |
|  | - Generation of a defined vibration frequency at the cutting edge of the tool: 20-50 kHz. | - YES | - NO |  |
|  | - Amplitude of oscillation: >10 µm ≤ 15 µm | - YES | - NO |  |
|  | - Amplitude detection of the actual mechanical oscillation amplitude | - YES | - NO |  |
|  | - Auto tracking of ultrasonic frequency and amplitude in the machining process monitoring of the existing process forces | - YES | - NO |  |
|  | - Ultrasonic parameter detection | - YES | - NO |  |
|  | - Automatic detection of the resonance frequency in the machining process. | - YES | - NO |  |
|  | - Ultrasonic auto tuning | - YES | - NO |  |
|  | Chip disposal and coolant supply: |  |  |  |
|  | - Chip tray | - YES | - NO |  |
|  | - Chip flushing for working area | - YES | - NO |  |
|  | - Coolant ring nozzle; minimal 12-fold | - YES | - NO |  |
|  | - Inner coolant supply and sedimentation preparation system: - Coolant reprocessing plant with sedimentation tank and cooling unit based on filter system - Paper filter type: ≥ 500 mm - Sediment tank capacity: ≥ 600 l - f50 deep filtration system - Flow: minimal 20 l / min; minimal 40 bar frequency regulated - Inner coolant supply preparation system supplies other coolant consumers such as a flushing gun via valves in the machine - Cooling unit power: ≥ 8000 W | - YES | - NO |  |
|  | Preform processing system control: |  |  |  |
|  | - Comprehensive control for the entire system - Multi-Touchscreen 21,5" | - YES | - NO |  |
|  | - System and applications must simplify the operation and enable the consistent administration, documentation and visualization of order, process, and machine data | - YES | - NO |  |
|  | - System connectable with CAD / CAM and compatible to PPS / ERP | - YES | - NO |  |
|  | - Application for visualization and adaption of the ultrasonic parameters of the machine/system | - YES | - NO |  |
|  | Measuring and monitoring tools: |  |  |  |
|  | - Direct measuring system X, Y, Z | - YES | - NO |  |
|  | - Infrared touch probe: - automatic loading from the tool magazine into the work spindle - Correction of workpiece position by positioning the NC rotary table - Determination of circle center and radius of a bore or a cylinder - Zero shift - Modification of tool correction - Workpiece tolerance checks | - YES | - NO |  |
|  | - Laser tool measuring: - Fix mounted at machine column - Tool length and diameter measuring - Calibration tool with taper - Repetitive accuracy: ≤ ± 0.001 mm | - YES | - NO |  |
|  | - Tool kit for control and compensation of the cinematic accuracy of 5 axis machine configurations | - YES | - NO |  |
|  | Milling, grinding, and polishing tools: |  |  |  |
|  | - Special diamond tools for fused silica material processing: - Diamond milling cutter for spot drilling: diameter (D): 1.2 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond drills: diameters (D): 2 mm, 3 mm, 5 mm (length 30 x D); Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece of each diameter - Diamond drill: diameter (D): 4 mm (length 30 x D); Spindle: HSK63-S; Tool holder: ER16; 2 (two) pieces - Diamond grinding wheel: type 6A9, diameter: ≥ 45 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond grinding wheel: type 6A9, diameter: ≥ 60 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond milling cutters: diameters (D): 4 mm, 6 mm, 8 mm (length 3 x D); type 2A2W; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece of each diameter | - YES | - NO |  |
|  | Technical support: |  |  |  |
|  | - The supplier must provide technical support for a period of 1 (one) year from the day of signature of the handover record | - YES | - NO |  |
|  | - Technical support must include consulting on setting parameters, use of the machine, configuring the system, and assisting in the implementation of individual processes, etc | - YES | - NO |  |
|  | - Technical support can be provided through remote access to the preform processing system | - YES | - NO |  |
|  | Acceptance criteria: a prerequisite for the signature of the handover record is also: |  |  |  |
|  | - Demonstration of drilling a hole with diameter of 4 mm and depth of 300 mm in a fused silica rod | - YES | - NO |  |
|  | - Demonstration of milling a flat surface on a fused silica rod | - YES | - NO |  |
|  | - Demonstration of milling a simple square block with fine finish (polished) sides on a fused silica rod | - YES | - NO |  |
|  | Delivery of the preform processing system into the facilities: |  |  |  |
|  | - The system will be delivered into the building through a door with dimensions (W x H): 2400 mm x 2650 mm | - YES | - NO |  |

| Place and date: | Seal: | Authorized signature: |
| --- | --- | --- |

The operation is co-financed by the Republic of Slovenia, Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is carried out under priority axis 1: ''International competitiveness of research, innovation and technological development, in line with smart specialization for increased competitiveness and greening of the economy'', priority investment 1.1 ''Strengthening R&D infrastructure and capacity to develop excellence in this and the promotion of competent centers, especially those of European importance'', specific objective 1.1.1 ''Efficient use of research infrastructure and development of knowledge / competences for better national and international cooperation in the knowledge triangle'' under the Operational Program for implementation of the European cohesion policy 2014-2020.

The tenderer must indicate the actual technical specifications of the equipment offered in accordance with all requirements.

The tenderer must give the name of the attached catalogue or brochure of the equipment manufacturer and the page number(s), together with the relevant technical specification. In exceptional cases, the tenderer may also provide a link to the equipment manufacturer's website with the relevant technical specification. Where a link to the online document is provided, the tenderer shall indicate a page number, indicating the relevant technical specification.