SPECIFICATIONS

| Contracting Authority | National Laboratory of Health, Environment and Food Prvomajska ulica 1 2000 Maribor |
| --- | --- |
| Public contract reference number | 01-OP220107 |
| Subject matter of the public contract | Molecular detection of SARS-CoV-2 in human samples |

1. TYPE, CHARACTERISTICS, QUALITY AND APPEARANCE OF THE PUBLIC CONTRACT SUBJECT/TENDER

1. The method involves a RT-PCR in real time for at least two target genes by a preliminary extraction of nucleic acids without sample pooling.

2. Samples for which the method has been validated/verified comprise samples from nasopharynx and throat and nose swabs. The Contracting Authority can send samples in liquid media (VTM or UTM) as well as dry swabs.

3. If the method detects mutations characteristic of the Omicron variant, it should be noted on the test result that it is possible this variant of the virus is present.

4. Reagents used for the test shall have a CE-IVD mark.

5. In the case of an in-house method, the latter is validated by the laboratory. The entire validation documentation shall be submitted in the tender.

6. In the case of a test provided by a commercial contractor, the method is verified by the laboratory. The entire verification documentation shall be submitted in the tender.

7. Given the validation results, the method has at least 99% specificity and capable of detecting at least 180,000 copies of SARS-CoV-2/mL.

8. The test result is stated either as POSITIVE or NEGATIVE. CT values for target genes are also indicated.

9. At 14.00 hours the contractor shall verify whether the Contracting Authority has samples ready at locations intended for taking of the samples. The contractor may take over these samples by 16.00 hours. The contractor shall provide the on-line results no later than 48 hours of taking over the samples at the locations. When the results are released, the Contracting Authority shall be informed either on-line or otherwise – e.g. in agreed electronic form that the results for a specific series of tests have been completed.

10. The test result shall be issued in writing as well.

11. The contractor shall take over the samples at several locations, i.e. at the addresses of the National Laboratory of Health, Environment and Food in Ljubljana, Maribor, Celje, Koper, Kranj, Novo mesto, Nova Gorica, Murska Sobota.

12. The remainders of the samples with proven SARS CoV-2 will be stored by the contractor to return them to the Contracting Authority for sequencing. Otherwise, the contractor shall not be allowed to use the remainders of the samples without the consent of the Contracting Authority. The remainders of negative samples shall be destroyed.

13. The maximum admissible price, including the transportation costs, is EUR 40.00 per sample.

14. The contractor shall ensure at least 3,000 samples in maximum 48 hours from the taking over of the samples with the Contracting Authority utilising this quota only if testing capacities at the National Laboratory of Health, Environment and Food are exhausted. The minimum number of samples to be ordered by the Contracting Authority shall be 1000 samples per day.

15. The contractor shall ensure the personal data protection in accordance with the General Data Protection Regulation (GDPR).

I, the tenderer’s representative/authorised signatory hereby declare that the offered services fully comply with the above description.

In     		 , on

Name and surname: