Project title: Upgrading of national research infrastructures - RIUM

DECLARATION ON MEETING THE MINIMUM TECHNICAL REQUIREMENTS OF THE CLIENT AND SPECIFICATIONS OF THE EQUIPMENT OFFERED

Tenderer’s name:

Tenderer’s address:

Subject of the public Contract: »Purchase of numerically controlled machine tools for cutting and forming processes«

Procurement code: 302/12 – RIUM70-P2-FS09-1/2022

| Name and type of equipment offered: |  |  |
| --- | --- | --- |
| Is the provider himself the manufacturer of the offered equipment? | YES ☐ | NO ☐* |
| *If your answer is NO, please write the manufacturer/s of the equipment offered (name, address): |  |  |

INSTRUCTION:

- This annex no. 2 shall be completed and signed. The bidder must indicate whether the offered equipment meets (or exceeds) the required technical specifications of the offered equipment according to all requirements of the client.

- The bidder shall load this Annex no. 2 in Word and PDF format (including attached evidence of compliance with technical requirements) in the e-JN information system in the section "Other annexes".

We declare that the offered equipment fully meets the minimum technical requirements of the contracting authority for the subject of the public procurement entitled "Purchase of numerically controlled machine tools for cutting and forming processes" Lot1: Flexible manufacturing cell for machining of prismatic workpieces by machining and forming processes, and that we are ready to realize the public procurement according to the following specification:

| EQUIPMENT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item nuber | Number of pieces |  | REQUIRED |  |  |  | OFFERED |  | PROOF |
| 1.1 |  | 1 |  | ### FLEXIBLE MANUFACTURING CELL FOR MACHINING OF PRISMATIC WORKPIECES BY MACHINING AND FORMING PROCESSES |  |  |  |  |  |
|  |  |  |  | Simultaneous control for machining in five working axes (translational axes X, Y, Z and rotary axes B, C). | YES NO |  |  |  |  |
|  |  |  |  | One-piece machine structure made of grey cast iron. | YES NO |  |  |  |  |
|  |  |  |  | Working space (linear axes): - X: at least 600 mm. - Y: at least 500 mm. - Z: at least 400 mm. | YES NO |  |  |  |  |
|  |  |  |  | Controlled rotary axes: - B: from -35° to 110° - C: at least 360° | YES NO |  |  |  |  |
|  |  |  |  | Swivel rotary table at least 600 mm in diameter, for simultaneous 5 axes machining | YES NO |  |  |  |  |
|  |  |  |  | CNC control (Siemens Sinumerik 840D-equivalent or better) with following hardware: - 32-bit processor with decentral bus concept. - Communication interfaces (MPI, industrial ethernet, profibus, ASI-bus). - Program memory on CF card that can be used in standard with EES. - Programmable logic controller (PLC) with specification equivalent to (or better) AS 317-2DP with Profibus. - CNC keyboard, control panel. - At least 50 GB memory for CNC programming. - Ethernet interface inside machine control cabinet: Fast Ethernet at least 10/100 BaseT. - TCP/IP network protocol. - Interface (at least USB 2.0) on command station. - Multi-touch screen. | YES NO |  |  |  |  |
|  |  |  |  | HSK-A63 spindle, with spindle speed at least 18.000 min-1. | YES NO |  |  |  |  |
|  |  |  |  | At least 120 Nm main spindle torque. | YES NO |  |  |  |  |
|  |  |  |  | Feed drives in geometric tolerance class IT1 | YES NO |  |  |  |  |
|  |  |  |  | At least 40 m/min feed rate for linear axes. | YES NO |  |  |  |  |
|  |  |  |  | Positioning repeatability less than 0.003 mm. | YES NO |  |  |  |  |
|  |  |  |  | Tool magazine: at least 30 pockets HSK-A63. | YES NO |  |  |  |  |
|  |  |  |  | Cooling: - By air blast trough spindle centre, - by coolant trough spindle centre, - coolant pressure at least 20 bar. | YES NO |  |  |  |  |
|  |  |  |  | Active cooling for main spindle, drives and table. | YES NO |  |  |  |  |
|  |  |  |  | Chip removal by chip conveyor. | YES NO |  |  |  |  |
|  |  |  |  | Integrated tool measuring device. | YES NO |  |  |  |  |
|  |  |  |  | Infrared touch probe for probing of workpieces. | YES NO |  |  |  |  |
|  |  |  |  | System for control and compensation of the cinematic accuracy. System for control and compensation of the cinematic accuracy. | YES NO |  |  |  |  |
|  |  |  |  | Hydraulic clamping system for workpiece clamping, with the possibility of using "zero" clamping. | YES NO |  |  |  |  |
|  |  |  |  | Postprocessor with 3D machine simulation for Siemens NX software already owned and used by the customer. | YES NO |  |  |  |  |
|  |  |  |  | CAD model of the machine, used for machine simulation. | YES NO |  |  |  |  |
|  |  |  |  | Instructions in English or Slovenian language in printed or electronic form. | YES NO |  |  |  |  |
| 1.2 |  | 1 |  | SCREW COMPRESSOR |  |  |  |  |  |
|  |  |  |  | Output power at least 7 kW. |  | YES NO |  |  |  |
|  |  |  |  | Nominal pressure at least 10 bar. |  | YES NO |  |  |  |
|  |  |  |  | Volume flow at least 1 m3/min. |  | YES NO |  |  |  |
|  |  |  |  | Integrated pressure tank at least 250 l. |  | YES NO |  |  |  |
|  |  |  |  | Water-oil separator. |  | YES NO |  |  |  |
|  |  |  |  | Instructions in English or Slovenian language in printed or electronic form. |  | YES NO |  |  |  |
| 1.3 |  | 1 |  | HYDRAULIC CLAMPING SYSTEM ADAPTED FOR MACHINE |  |  |  |  |  |
|  |  |  |  | Hydraulic clamp at least 100 mm wide, with base and "V" jaws, complete with base plate adapted for "zero" clamping (equivalent to HILMA Z 100 H or better). |  | YES NO |  |  |  |
| 1.4 |  | 1 |  | SET OF CLAMPING UNITS (MASTERS), ADAPTORS AND ACCESSORIES FOR SPINDLE HSK-A63 ACCORDING TO DIN 69893 / ISO 12164, FOR TOOL CLAMPING: |  |  |  |  |  |
|  |  |  |  | Master, CaptoTM, HSK-A63, C5, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Extension C5-C5, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Collet chuck C5-ER32, 1 |  | YES NO |  |  |  |
|  |  |  |  | Master, typ Weldon-12, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master, typ Weldon-16, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master, typ Weldon-20, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master, typ Weldon-25, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master, typ Weldon-32, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master, hydro-expansion, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 3 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 4 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 5 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 6 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 8 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Adaptor for hydro-expansion, 10 mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for collet chuck ER25, HSK-A63-ER25, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for collet chuck ER32, HSK-A63-ER32, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for collet chuck ER40, HSK-A63-ER40, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for collet chuck ER16, HSK-A63-ER16, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for collet chuck ER20, HSK-A63-ER20, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Set of collets ER16, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Set of collets ER20, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Set of collets ER25, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Set of collets ER32, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Set of collets ER40, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 16, with inbus screw, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 22, with inbus screw, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 27, with inbus screw, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 32, with inbus screw, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 16, with cross, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 22, with cross, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 27, with cross, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for milling head 32, with cross, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master with drilling head, HSK-A63, 2.5-16.0, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone, HSK-A63-MK1, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone, HSK-A63-MK2, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone, HSK-A63-MK3, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone, HSK-A63-MK4, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone and screw, HSK-A63-MK1, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone and screw, HSK-A63-MK2, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | kos Master with Morse cone and screw, HSK-A63-MK3, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master with Morse cone and screw, HSK-A63-MK4, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 6, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 8, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 10, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 12, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 16, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 20, HSK-A63, 2 pieces. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 25, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master shrink-fit 32, HSK-A63, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Extension, shrink-fit 3, 3mm/20mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Extension, shrink-fit 4, 4mm/20mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Extension, shrink-fit 5, 5mm/20mm, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for threads with axial compensation, HSK-A63, M3-M12, 1 piece. |  | YES NO |  |  |  |
|  |  |  |  | Master for threads with axial compensation, HSK-A63, M6-M20, 1 piece. |  | YES NO |  |  |  |
| 1.5 |  | 1 |  | Delivery and installation: - Delivery to the address Smetanova 17, 2000 Maribor, - delivery to the laboratory C-101, - installation of the machine on the place of use and - test start of the machine. |  | YES NO |  |  |  |
| 1.6 |  | 1 |  | Training: At least 5 days for 8 hours for five participants at the user's location. |  | YES NO |  |  |  |
| 1.7 |  | 1 |  | Warranty: 18 month. |  | YES NO |  |  |  |
| 1.8 |  | 1 |  | Time of delivery: Delivery no later than 90 days after the effective date of the contract. |  | YES NO |  |  |  |

|  | Stamp: |
| --- | --- |
| Place and date: |  | Signature of the responsible person: |

The operation is co-financed by the Republic of Slovenia, the Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. Direct validation of the operation is carried out under the Operational Programme for the Implementation of the European Cohesion Policy 2014-2020 under Priority Axis 1: "International competitiveness of research, innovation and technological development in line with smart specialisation for greater competitiveness and greening of the economy", investment priority 1.1. infrastructure for research and innovation and capacity to develop excellence in this field, as well as the promotion of competent centres, especially those of European importance ", specific objective 1.1.1 Effective use of research infrastructure and development of knowledge / competences for better national and international cooperation in the knowledge triangle”.

The bidder must indicate whether the offered equipment meets (or exceeds) the required technical specifications of the offered equipment according to all requirements of the client.

The tenderer must state the name of the attached catalog or prospectus of the equipment manufacturer and the page number indicating the relevant technical specification. Exceptionally, the tenderer may also provide a link to the equipment manufacturer's website stating the relevant technical specifications. In the case of a link to an online document, the tenderer must provide a page number indicating the relevant technical specification. The bidder shall attach to this Annex 2, along with the bid, evidence of compliance with the minimum technical requirements and, in the case of a link to the website, ensure their adequacy and activity in word form.