Project name: Upgrading national research infrastructures RIUM

302/12 – RIUM70-P2-FS09-1/2022

Technical Specifications for »Purchase of numerically controlled machine tools for cutting and forming processes«

- SUBJECT OF PUBLIC TENDER

The public tender includes components for the construction of a pilot smart factory that will serve as a platform for research in this field. It includes machine tools for processing metal and non-metal workpieces. All control units on individual units (machines) enable integration with existing equipment for servicing and transport. It enables together with the integration of  information systems  the development, implementation, testing and optimization of machining processes. The purpose of the system is to implement and develop smart manufacturing systems.

The bidder must provide for the equipment (without additional costs for the client):

- equipment, as derived from the technical specifications, must be new, packaged and delivered to the location;

- all offered equipment is compatible and interconnected;

- that all required equipment can be connected into a single, flawlessly functioning system, in accordance with technical specifications;

- to eliminate any errors that may arise due to possible, incompatibility or inadequate connection to a single, properly functioning system;

- to deliver compete equipment according to the specification at the same time,

- installation and connection of all components of equipment by the supplier at the location of University of Maribor Faculty of Mechanical Engineering, Smetanova ulica 17, 2000 Maribor in laboratory C-101.

- installation at the same time as delivery to perform or provide a demonstration of machining of at least one part and submit of instructions for handling of equipment in at least Slovene and English.

The tender must include all the material required for installation and commissioning, even if it is not explicitly stated in the technical requirements. The price of the equipment must be given in euros and include all the elements of which it consists (equipment by item and accompanying services, discounts, transport costs, labor costs, delivery and installation, installation costs, start-up and demonstration, insurance, training costs, instructions, etc.) or all costs related to the implementation of the public procurement in question.

- TECHNICAL REQUIEREMENTS

Minimum technical requirements for individual sets follows. The tender must meet or exceed all minimum technical requirements.

LOT 1: Flexible manufacturing cell for machining of prismatic workpieces by machining and forming processes

The set comprises a flexible machining cell for machining prismatic workpieces by cutting and forming processes with the configuration below. The machining cell includes all the equipment for performing 5-axis simultaneous machining with milling and incremental forming with real-time diagnostics of the machining process. The processing cell enables integration into the smart factory system according to the principles of Industry 4.0.

| Item number | Number of pieces | Required |
| --- | --- | --- |
| MAIN EQUIPMENT |  |  |
| 1.1 | 1 | Flexible machining cell for machining prismatic workpieces by cutting (milling, drilling, reaming, threading) and forming processes (incremental forming): - Simultaneous control for machining in five working axes (translational axes X, Y, Z and rotary axes B, C). - One-piece machine structure made of grey cast iron. - Working space (linear axes): - X: at least 600 mm. - Y: at least 500 mm. - Z: at least 400 mm. - Controlled rotary axes: - B: from -35° to 110° - C: at least 360° - Swivel rotary table at least 600 mm in diameter, for simultaneous 5 axes machining. - CNC control (Siemens Sinumerik 840D-equivalent or better) with following hardware: - 32-bit processor with decentral bus concept. - Communication interfaces (MPI, industrial ethernet, Profibus, ASI-bus). - Program memory on CF card that can be used in standard with EES. - Programmable logic controller (PLC) with specification equivalent to (or better) AS 317-2DP with Profibus. - CNC keyboard, control panel. - At least 50 GB memory for CNC programming. - Ethernet interface inside machine control cabinet: Fast Ethernet at least 10/100 BaseT. - TCP/IP network protocol. - Interface (at least USB 2.0) on command station. - Multi-touch screen. - HSK-A63 spindle, with spindle speed at least 18.000 min-1. - At least 120 Nm main spindle torque. - Feed drives in geometric tolerance class IT1. - At least 40 m/min feed rate for linear axes. - Positioning repeatability less than 0.003 mm. - Tool magazine: at least 30 pockets HSK-A63. - Cooling: - By air blast trough spindle centre, - by coolant trough spindle centre, - coolant pressure at least 20 bar. - Active cooling for main spindle, drives and table. - Chip removal by chip conveyor. - Integrated tool measuring device. - Infrared touch probe for probing of workpieces. - System for control and compensation of the cinematic accuracy. - Hydraulic clamping system for workpiece clamping, with the possibility of using "zero" clamping. - Postprocessor with 3D machine simulation for Siemens NX software already owned and used by the customer. - CAD model of the machine, used for machine simulation. - Instructions in English or Slovenian language in printed or electronic form. |
| SUPPORT EQUIPMENT |  |  |
| 1.2 | 1 | Screw compressor: - Output power at least 7 kW. - Nominal pressure at least 10 bar. - Volume flow at least 1 m3/min. - Integrated pressure tank at least 250 l. - Water-oil separator. - Instructions in English or Slovenian language in printed or electronic form. |
| 1.3 | 1 | Hydraulic clamping system adapted for machine (1.1): - Hydraulic clamp at least 100 mm wide, with base and "V" jaws, complete with base plate adapted for "zero" clamping (equivalent to HILMA Z 100 H or better). |
| 1.4 | 1 | Set of clamping units (masters), adaptors and accessories for spindle HSK-A63 according to DIN 69893 / ISO 12164, for tool clamping: - Master, CaptoTM, HSK-A63, C5, 1 piece. - Extension C5-C5, 1 piece. - Collet chuck C5-ER32, 1 - Master, typ Weldon-12, HSK-A63, 1 piece. - Master, typ Weldon-16, HSK-A63, 1 piece. - Master, typ Weldon-20, HSK-A63, 1 piece. - Master, typ Weldon-25, HSK-A63, 1 piece. - Master, typ Weldon-32, HSK-A63, 1 piece. - Master, hydro-expansion, HSK-A63, 1 piece. - Adaptor for hydro-expansion, 3 mm, 1 piece. - Adaptor for hydro-expansion, 4 mm, 1 piece. - Adaptor for hydro-expansion, 5 mm, 1 piece. - Adaptor for hydro-expansion, 6 mm, 1 piece. - Adaptor for hydro-expansion, 8 mm, 1 piece. - Adaptor for hydro-expansion, 10 mm, 1 piece. - Master for collet chuck ER25, HSK-A63-ER25, 1 piece. - Master for collet chuck ER32, HSK-A63-ER32, 1 piece. - Master for collet chuck ER40, HSK-A63-ER40, 1 piece. - Master for collet chuck ER16, HSK-A63-ER16, 1 piece. - Master for collet chuck ER20, HSK-A63-ER20, 1 piece. - Set of collets ER16, 1 piece. - Set of collets ER20, 1 piece. - Set of collets ER25, 1 piece. - Set of collets ER32, 1 piece. - Set of collets ER40, 1 piece. - Master for milling head 16, with inbus screw, HSK-A63, 1 piece. - Master for milling head 22, with inbus screw, HSK-A63, 1 piece. - Master for milling head 27, with inbus screw, HSK-A63, 1 piece. - Master for milling head 32, with inbus screw, HSK-A63, 1 piece. - Master for milling head 16, with cross, HSK-A63, 1 piece. - Master for milling head 22, with cross, HSK-A63, 1 piece. - Master for milling head 27, with cross, HSK-A63, 1 piece. - Master for milling head 32, with cross, HSK-A63, 1 piece. - Master with drilling head, HSK-A63, 2.5-16.0, 2 pieces. - Master with Morse cone, HSK-A63-MK1, 2 pieces. - Master with Morse cone, HSK-A63-MK2, 2 pieces. - Master with Morse cone, HSK-A63-MK3, 2 pieces. - Master with Morse cone, HSK-A63-MK4, 2 pieces. - Master with Morse cone and screw, HSK-A63-MK1, 1 piece. - Master with Morse cone and screw, HSK-A63-MK2, 1 piece. - kos Master with Morse cone and screw, HSK-A63-MK3, 1 piece. - Master with Morse cone and screw, HSK-A63-MK4, 1 piece. - Master shrink-fit 6, HSK-A63, 1 piece. - Master shrink-fit 8, HSK-A63, 1 piece. - Master shrink-fit 10, HSK-A63, 1 piece. - Master shrink-fit 12, HSK-A63, 1 piece. - Master shrink-fit 16, HSK-A63, 1 piece. - Master shrink-fit 20, HSK-A63, 2 pieces. - Master shrink-fit 25, HSK-A63, 1 piece. - Master shrink-fit 32, HSK-A63, 1 piece. - Extension, shrink-fit 3, 3mm/20mm, 1 piece. - Extension, shrink-fit 4, 4mm/20mm, 1 piece. - Extension, shrink-fit 5, 5mm/20mm, 1 piece. - Master for threads with axial compensation, HSK-A63, M3-M12, 1 piece. - Master for threads with axial compensation, HSK-A63, M6-M20, 1 piece. |
| Supply and installation, training, warranty, delivery time for main and support equipment |  |  |
| 1.5 | 1 | Delivery and installation: - Delivery to the address Smetanova 17, 2000 Maribor, - delivery to the laboratory C-101, - installation of the machine on the place of use and - test start of the machine. |
| 1.6 | 1 | Training: - At least 5 days for 8 hours for five participants at the user's location. |
| 1.7 | 1 | Warranty: - 18 month. |
| 1.8 | 1 | Time of delivery: - Delivery no later than 90 days after the effective date of the contract. |

The operation is co-financed by the Republic of Slovenia, the Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is implemented under the Operational Program for the Implementation of European Cohesion Policy 2014-2020 under Priority Axis 1: "International competitiveness of research, innovation and technological development in line with smart specialization for greater competitiveness and greening of the economy", priority investment 1.1 "Strengthening infrastructure for research and innovation and capacity to develop excellence in this field, as well as the promotion of competent centers, especially those of European importance ", specific objective 1.1.1" Effective use of research infrastructure and development of knowledge/competences for better national and international cooperation in the triangle knowledge ”.

302/12 – RIUM70-P2-FS09-1/2022