Project title: Upgrading National Research Infrastructures – RIUM

COMPLIANCE DECLARATION WITH MINIMUM TECHNICAL REQUIREMENTS OF THE OFFERED EQUIPMENT – LOT 1

Name of the tenderer:

Address of the tenderer:

Subject of the public procurement: Purchase of Advanced Equipment for Development of Special Optical Fibers – Lot 1: Systems for the operational control of the fiber drawing tower

Designation of the public procurement: 302/36 – RIUM80-FERI 03.1-P41-46/2022

| Is the tenderer also the manufacturer of the offered equipment? | YES ☐ | NO ☐* |
| --- | --- | --- |
| *If the answer is NO, enter the equipment manufacturer's name and address: |  |  |

INSTRUCTIONS:

- This Annex No. 2 must be completed and signed. The tenderer must indicate the actual technical specifications of the equipment offered in line with all requirements.

- The tenderer shall upload this Annex No. 2 to the e-JN information system in the section “Other Attachments”.

We declare that the offered equipment fully meets the minimum technical requirements of the client for the public procurement titled "Purchase of Advanced Equipment for Development of Special Optical Fibers – Lot 1", and that we are ready to realize the order according to the following specification:

|  | EQUIPMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | REQUIREMENTS | OFFERED |  |  |  | DATASHEET |
| 1 | - Measurement of bare fiber temperature close to furnace exit (high temperature): |  |  |  |  |  |
|  | - Temperature range: Tmin ≤ 500 and Tmax ≥ 1500 °C | - YES |  | - NO |  |  |
|  | - EtherCAT communication module | - YES | - NO |  |  |  |
|  | - Fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm | - YES | - NO |  |  |  |
|  | - Accuracy: ≤ ±1 °C | - YES |  | - NO |  |  |
|  | - Measuring rate: ≥ 100 measurements/sec | - YES |  | - NO |  |  |
|  | - Measurement of bare fiber diameter with airline detection close to furnace exit: |  |  |  |  |  |
|  | - Glass fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm | - YES | - NO |  |  |  |
|  | - Accuracy: ≤ 0.05 µm | - YES |  | - NO |  |  |
|  | - Repeatability: ≤ 0.02 µm | - YES |  | - NO |  |  |
|  | - Number of measurement axis: ≥ 2 | - YES |  | - NO |  |  |
|  | - Measuring rate: ≥ 2500 measurements/sec | - YES |  | - NO |  |  |
|  | - Airline detection: smallest detectable airline ≤ 0.5 µm | - YES |  | - NO |  |  |
|  | - EtherCAT communication module | - YES |  | - NO |  |  |
|  | - Air pressure cleaning system | - YES |  | - NO |  |  |
|  | - Measurement of bare fiber temperature prior to primary coating application (low temperature): |  |  |  |  |  |
|  | - Temperature range: Tmin ≤ 40 and Tmax ≥ 120°C | - YES | - NO |  |  |  |
|  | - Fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm | - YES | - NO |  |  |  |
|  | - Accuracy: ≤ ±1 °C | - YES | - NO |  |  |  |
|  | - Measuring rate: ≥ 100 measurements/sec | - YES | - NO |  |  |  |
|  | - EtherCAT communication module | - YES | - NO |  |  |  |
|  | - Measurement of bare fiber diameter prior to primary coating application: |  |  |  |  |  |
|  | - Glass fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm | - YES | - NO |  |  |  |
|  | - Accuracy: ≤ 0.05 µm | - YES | - NO |  |  |  |
|  | - Repeatability: ≤ 0.02 µm | - YES | - NO |  |  |  |
|  | - Number of measurement axis: ≥2 | - YES |  | - NO |  |  |
|  | - Measuring rate: ≥ 2500 measurements/şec | - YES |  | - NO |  |  |
|  | - EtherCAT communication module | - YES |  | - NO |  |  |
|  | - Air pressure cleaning system | - YES |  | - NO |  |  |
|  | - Measurement of glass rod diameter up to 18mm: |  |  |  |  |  |
|  | - Glass rod diameter measurement range: - OD_min ≤ 0.2 mm - OD_max ≥ 18 mm | - YES |  | - NO |  |  |
|  | - Number of measurement axis: ≥2 | - YES |  | - NO |  |  |
|  | - Accuracy: ≤ ± 0.2 µm | - YES |  | - NO |  |  |
|  | - Repeatability: ≤ ± 0.1 µm | - YES |  | - NO |  |  |
|  | - Exposure time: ≤ 0.2 µs | - YES |  | - NO |  |  |
|  | - Measuring rate: ≥ 5000 measurements/sec/axis | - YES |  | - NO |  |  |
|  | - Measurement of coated fiber diameter after application of primary coating: |  |  |  |  |  |
|  | - Coated fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm | - YES |  | - NO |  |  |
|  | - Accuracy: ≤ 0.05 µm | - YES |  | - NO |  |  |
|  | - Repeatability: ≤ 0.02 µm | - YES |  | - NO |  |  |
|  | - Number of measurement axis: ≥2 | - YES |  | - NO |  |  |
|  | - Measuring rate: ≥ 2500 measurements/sec | - YES |  | - NO |  |  |
|  | - EtherCAT communication module | - YES |  | - NO |  |  |
|  | - Air pressure cleaning system | - YES |  | - NO |  |  |
|  | - Measurement of coated fiber diameter after application of secondary coating: |  |  |  |  |  |
|  | - Coated fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm | - YES | - NO |  |  |  |
|  | - Accuracy: ≤ 0.05 µm | - YES | - NO |  |  |  |
|  | - Repeatability: ≤ 0.02 µm | - YES | - NO |  |  |  |
|  | - Number of measurement axis: ≥2 | - YES | - NO |  |  |  |
|  | - Measuring rate: ≥ 2500 measurements/sec | - YES | - NO |  |  |  |
|  | - EtherCAT communication module | - YES | - NO |  |  |  |
|  | - Air pressure cleaning system | - YES | - NO |  |  |  |
|  | - Fiber lump detector after application of secondary coating: |  |  |  |  |  |
|  | - Coated optical fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm | - YES |  |  | - NO |  |
|  | - Min. Detectable Fault Height: ≤ 5 µm | - YES |  |  | - NO |  |
|  | - Min. Fault Length: ≤ 50 µm | - YES |  |  | - NO |  |
|  | - Speed of a drawing fiber: - speed_min ≤ 1 m/min - speed_max ≥ 3000 m/min | - YES |  |  | - NO |  |
|  | - Processor system control unit shall include/facilitate: |  |  |  |  |  |
|  | - Touch screen monitor. | - YES | - NO |  |  |  |
|  | - Display with GUI allows demonstration of following production and products parameters: - Diameter - Ovality - Tension - Spinning - Concentricity - Temperature - Airlines - Vibration frequency - Optical fiber position with scatter plot presentation - Trend and statistics - Number of lumps/neckdowns | - YES | - NO |  |  |  |
|  | - EtherCAT communication module for sending all above mentioned data to customer’s central control sys | - YES | - NO |  |  |  |
|  | - Data Storage: HDD, USB memory stick or network | - YES | - NO |  |  |  |
|  | Acceptance criteria: |  |  |  |  |  |
|  | Delivery time: Until 31 March 2023 | - YES | - NO |  |  |  |
|  | Warranty At least full 18 Month warranty | - YES | - NO |  |  |  |

| Place and date: | Stamp: | Authorized signature: |
| --- | --- | --- |

The operation is co-financed by the Republic of Slovenia, Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is carried out under priority axis 1 of the Operational Program for Implementation of the European Cohesion Policy 2014–2020: “International competitiveness of research, innovation and technological development, in line with smart specialization for increased competitiveness and greening of the economy”, priority investment 1.1 “Strengthening R&D infrastructure and capacity to develop excellence in this and the promotion of competent centers, especially those of European importance”, specific objective 1.1.1 “Efficient use of research infrastructure and development of knowledge/competences for better national and international cooperation in the knowledge triangle”.

The tenderer must indicate the actual technical specifications of the equipment offered in accordance with all requirements.

The tenderer must give the name of the attached catalogue or brochure of the equipment manufacturer and the page number(s), together with the relevant technical specification. In exceptional cases, the tenderer may also provide a link to the equipment manufacturer's website with the relevant technical specification. Where a link to the online document is provided, the tenderer shall indicate a page number, indicating the relevant technical specification.