Naziv projekta: Nadgradnja nacionalnih raziskovalnih infrastruktur – RIUM

Project title: Upgrading National Research Infrastructures – RIUM

Tehnične specifikacije napredne opreme za razvoj posebnih optičnih vlaken

Technical specifications of advanced equipment for the development of special optical fibers

PREDMET JAVNEGA NAROČILA

- SUBJECT OF THE PUBLIC PROCUREMENT

| Predmet javnega naročila je nabava napredne opreme za razvoj posebnih optičnih vlaken. Oprema zajeta v tem javnem naročilu mora omogočati izdelavo (raziskave in razvoj) posebnih in naprednih optičnih vlaken iz silicijevega stekla. Oprema mora omogočati delo na področju ključnih fotonskih tehnologij, ki sodijo med ključne omogočitvene tehnologije (KET). Predmet javnega naročila vsebuje ključne sisteme za razvoj in izdelavo, kot so: - Oprema za vlečni stolp - Sistem za obdelavo surovcev Oprema za vlečni stolp mora vsebovati instrumente, potrebne za merjenje in spremljanje procesa vlečenja optičnih vlaken. Za zagotovitev učinkovite proizvodnje optičnih vlaken s konstantno kakovostjo optičnih vlaken z vnaprej določenim zunanjim premerom, debelino prevleke in krožnostjo, morajo instrumenti zagotavljati merilno in krmilno tehnologijo v vseh ključnih fazah procesa od peči do navijalnika. Sistem za obdelavo surovcev mora omogočati posebno obdelavo vlakenskih preformov. To na primer pomeni natančno vrtanje lukenj s premerom v razponu do nekaj milimetrov in do globine nekaj centimetrov, z visoko točnostjo Vsak posamezni sklop opreme lahko dostavi različni ponudnik, pri čemer lahko ponudniki ponudbe predložijo za en sklop, za več sklopov ali za vse sklope. Ponudniki morajo podati ponudbo, ki vključuje vse postavke opreme, ki je zahtevana v posameznem sklopu. Naročnik ne bo obravnaval ponudb, ki ne ponujajo vseh postavk opreme posameznega sklopa in bo takšne ponudbe zavrnil kot nedopustne. Lokacija dobave in vnos opreme: Univerza v Mariboru, Fakulteta za elektrotehniko, računalništvo in informatiko, Valvasorjeva ulica 75, 2000 Maribor, Slovenija. Naročnik bo ponudnikom omogočil ogled prostora na lokaciji vnosa in namestitve opreme, skladno s točko 11 Navodil ponudnikom. Rok dobave: Kot velja za posamezni sklop. Usposabljanje: Kot velja za posamezni sklop. Garancijski rok: Kot velja za posamezni sklop. Za vso opremo znotraj posameznega sklopa velja, da mora zanjo v času garancijskega obdobja proizvajalec opreme zagotavljati dostop do vseh posodobitev programske opreme brez dodatnih stroškov. Vsi popravki, servisni paketi ter nadgradnje in podpora morajo biti od proizvajalca strojne oziroma programske opreme posameznega sklopa. V času garancijskega obdobja mora ponudnik za vso opremo iz posameznega sklopa nuditi tehnično podporo. Tehnična podpora mora vključevati svetovanje pri nastavitvi parametrov, konfiguracijo sistema iz posameznega sklopa in pomoč pri izvedbi posameznih procesov. Tehnična podpora se lahko izvaja preko oddaljenega dostopa do sistemov iz posameznega sklopa. Ponudnik mora za opremo iz posameznega sklopa zagotoviti: - da so vsi ponujeni moduli/komponente združljivi in medsebojno povezljivi, - da je mogoče vse zahtevane module/komponente povezati v enoten in brezhibno delujoč sistem v skladu s tehničnimi specifikacijami, - odpraviti vse napake, ki bi se pojavile zaradi morebitne medsebojne nezdružljivosti ali nepovezljivosti oz. zaradi neustrezne povezave v enoten, brezhibno delujoč sistem. V ponudbi za posamezen sklop mora biti zajeti ves droben material, ki je potreben za priključitev naprav, tudi če ni izrecno naveden. | The subject of the public procurement is advanced equipment for the development of special optical fibers. The equipment covered in this procurement shall enable the production (research and development) of special and advanced silica optical fibers. The equipment facilitates work in the field of key photonic technologies, which belong to Key enabling technologies (KETs). The subject of the public procurement contains crucial systems for development and production, for example: - Fiber drawing tower equipment - Preform processing system The drawing tower equipment shall contain the instruments necessary to measure and monitor the optical fiber drawing process. To ensure efficient production of optical fibers with a constant quality with a predetermined outer diameter, coating thickness and circularity, the instruments must provide measuring and control technology at all key stages of the process from the furnace to the winder. The preform processing system shall allow special processing of fiber preforms, for example precision drilling of holes with high accuracy with a diameter ranging up to a few millimeters and to a depth of a few centimeters. Each lot of the procurement can be provided by different supplier wherein supplier can provide quotation for individual lot, several lots, or all lots. Suppliers must provide quotation which includes all demands of the equipment required in individual part. Client will not consider tenders which does not offer all the items of equipment of individual lot. Such tenders will be rejected as inadmissible. Equipment delivery and placement: University of Maribor, Faculty of Electrical Engineering and Computer Science, Laboratory for Electro-Optics and Sensor Systems, Valvasorjeva ulica 75, 2000 Maribor, Slovenia. The contracting authority will enable the tenderers to inspect the premises on site in line with Point 11 of the Instructions to tenderers. Delivery time: As applicable to the individual lot. Training: As applicable to the individual lot. The warranty period: As applicable to the individual lot. The equipment manufacturer for each lot is obliged to provide access to all software updates at no additional cost during warranty period. All patches, service packs, upgrades and support must be provided by the hardware or software manufacturer of each lot. During the warranty period the supplier for each lot must provide technical support. Technical support must include consulting on setting parameters, use of the machine, configuring the system, and assisting in the implementation of individual processes, etc. Technical support can be provided through remote access for each lot. The provider of the equipment from each lot must guarantee: - that all modules/components offered are compatible and interconnected, - that all required modules/components can be integrated into a single and seamlessly functioning system in line with technical specifications, - to eliminate all errors resulting from possible incompatibility or mismatch, or from improper connection to a single, seamlessly functioning system. The offer of each lot must include all the fine material, necessary for connecting the devices, even if it is not explicitly stated |
| --- | --- |

TEHNIČNE ZAHTEVE

- TECHNICAL SPECIFICATIONS

| V spodnjih tabelah so za vsak sklop navedene karakteristike opreme, ki jih zahteva naročnik. Ponudba mora izpolnjevati ali presegati vse minimalne tehnične zahteve. Predmet javnega naročila sta 2 (dva) sklopa, in sicer: Sklop 1: Oprema za operativno vodenje vlečnega stolpa Sklop 2: Sistem za obdelavo surovcev | Tables bellow list the characteristics of equipment for each lot, as required by the customer. The tender must meet or exceed all minimum technical specifications. Public tender consists of 2 (two) lots including: Lot 1: Systems for the operational control of the fiber drawing tower Lot 2: The preform processing system |  |
| --- | --- | --- |
| Sklop 1: Oprema za operativno vodenje vlečnega stolpa Univerza v Mariboru naroča instrumente, potrebne za merjenje in spremljanje procesa vlečenja optičnih vlaken, da bi zagotovili učinkovito proizvodnjo optičnih vlaken s konstantno kakovostjo optičnih vlaken z vnaprej določenim zunanjim premerom, debelino prevleke in krožnostjo. Instrumenti morajo zagotavljati merilno in krmilno tehnologijo v vseh ključnih fazah procesa od peči do navijalnika: - Meritev temperature golega vlakna takoj na izhodu iz peči - Meritev premera golega vlakna z detekcijo vzdolžnih mehurčkov, v bližini izhoda iz peči - Meritev temperature golega vlakna pred nanosom primarne zaščite (nizke temperature) - Meritev premera golega vlakna pred nanosom primarne zaščite - Meritev steklenega surovca do premera vsaj 18mm - Meritev premera oplaščenega optičnega vlakna takoj po nanosu primarne zaščite - Meritev premera oplaščenega optičnega vlakna po drugem nanosu - Detekcija mehurčkov v vlaknu po drugem nanosu zaščite Surove podatke iz vseh naprav je potrebno poslati v centralni nadzorni sistem stolpa za vlečenje prek komunikacijskega protokola EtherCAT, da se zagotovi stalen nadzor postopka vlečenja vlaken. V primeru, da lahko instrumentacija zagotavlja dodatne informacije za spremljanje procesa z uporabo algoritmov ponudnika, mora ponudba vsebovati samostojen procesorski sistem, ki lahko ovrednoti, shrani in posreduje vse izračunane parametre (eliptičnost, napetost vlaken itd.). Instrumenti morajo vključevati tudi komunikacijsko opremo, vključno s komunikacijskim vmesnikom in kabli za povezovanje naprav v celoten sistem. Celoten sistem naj bo združljiv s slovenskim električnim standardom - 230 VAC. Če je predvideno združevanje večih instrumentov skupaj, mora ponudba vsebovati potrebne adapterje/vmesnike, da prihranimo čim več prostora na vlečni liniji za vlakna. Ponudba naj vsebuje tudi osnovne tehnične risbe vseh instrumentov za preverjanje montaže na okvir vlečnega stolpa. Vsi ponujeni instrumenti morajo vsebovati zaščitno ohišje/pokrove za preprečevanje poškodb v primeru razlitja materialov za prevleke optičnih vlaken. V nadaljevanju so podane minimalne tehnične zahteve. Ponudba mora izpolnjevati ali presegati vse minimalne tehnične zahteve. | Lot 1: Systems for the operational control of the fiber drawing tower The University of Maribor is seeking the instrumentation needed for measuring and monitoring the optical fiber drawing process to ensure fiber production efficiency, and constantly high quality of optical fibers with predefined outer diameter, coating thickness, and circularity. Instrumentation shall provide measuring and control technology at all crucial process phases from the furnace to the capstan: - Measurement of bare fiber temperature close to furnace exit - Measurement of bare fiber diameter with airline detection close to furnace exit. - Measurement of bare fiber temperature prior to primary coating application (low temperature). - Measurement of bare fiber diameter prior to primary coating application. - Measurement of glass rod diameter up to 18mm. - Measurement of coated fiber diameter after application of primary coating. - Measurement of coated fiber diameter after application of secondary coating. - Fiber lump detector after application of secondary coating. Raw data from all devices has to be sent to draw tower’s central control system via EtherCAT communication protocol to ensures a continuous control of the fiber draw process. In case the instrumentation can provide additional information for process monitoring using tenderer’s own algorithms the offer has to include a stand-alone processor system that can evaluate, store and forward all calculated parameters (ellipticity, fiber tension, etc.). Instrumentation shall also include communication equipment including interface modulus, cables to connect devices together in working system. The whole system should be compatible with the Slovenian electrical standard - 230 VAC. If multiple instruments can be stacked together, the offer should include necessary adapters to save as much space on the fiber draw line as possible. The offer should also include basic technical drawings of all instruments in order to check the mounting on fiber draw tower frame. All offered devices should include protective housing/covers to prevent damage in case of optical fiber coating spill. Minimum technical requirements are given below. The offer must match or exceeded all minimum technical requirements. |  |
| Št. postavke; Število kosov No. of pieces; ZAHTEVANO; REQUIREMENTS. 1; 1; Minimalne tehnične zahteve: - Meritev temperature golega vlakna takoj na izhodu iz peči (visoka temperature): - Temperaturno področje: Tmin ≤ 500 in Tmax ≥ 1500 °C - EtherCAT communikacijski modul - Premeri vlaken v področju: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Točnost: ≤ ±1 °C - Hitrost zajemanja: ≥ 100 meritev/sec - EtherCAT komunikacijski modul - Meritev premera golega vlakna z detekcijo vzdolžnih mehurčkov, v bližini izhoda iz peči: - Premeri golih vlaken v področju: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Točnost: ≤ 0.05 µm - Ponovljivost: ≤ 0.02 µm - Število osi merjenja: ≥ 2 - Hitrost zajemanja: ≥ 2500 meritev/sec - Detekcija vzdolžnih mehurčkov: najmanjši vzdolžni mehurček ≤ 0.5 µm - EtherCAT komunikacijski modul - Čiščenje s stisnjenim zrakom - Meritev temperature golega vlakna pred nanosom primarne zaščite (nizke temperature): - Temperaturno področje: Tmin ≤ 40 in Tmax ≥ 120 °C - EtherCAT komunikacijski modul - Premeri vlaken v področju: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Točnost: ≤ ±1 °C - Hitrost zajemanja: ≥ 100 meritev/sec - EtherCAT komunikacijski modul - Meritev premera golega vlakna pred nanosom primarne zaščite: - Premeri steklenih vlaken v področju: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Točnost: ≤ 0.05 µm - Ponovljivost: ≤ 0.02 µm - Število osi merjenja: ≥ 2 - Hitrost zajemanja: ≥ 2500 meritev/sec - EtherCAT komunikacijski modul - Čiščenje s stisnjenim zrakom - Meritev steklenega surovca do premera vsaj 18mm: - Področje premerov surovca: - OD_min ≤ 0.2 mm - OD_max ≥ 18 mm - Število osi merjenja: ≥ 2 - Točnost: ≤ ± 0.2 µm - Ponovljivost: ≤ ± 0.1 µm - Čas izpostavljenosti: ≤ 0.2 µs - Hitrost zajemanja: ≥ 5000 meritev/sec/axis - Meritev premera oplaščenega optičnega vlakna takoj po nanosu primarne zaščite: - Premeri oplaščenih vlaken v področju: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Točnost: ≤ 0.05 µm - Ponovljivost: ≤ 0.02 µm - Število osi merjenja: ≥ 2 - Hitrost zajemanja: ≥ 2500 meritev/sec - EtherCAT komunikacijski modul - Čiščenje s stisnjenim zrakom - Meritev premera oplaščenega optičnega vlakna po drugem nanosu: - Premeri oplaščenih vlaken v področju: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Točnost: ≤ 0.05 µm - Ponovljivost: ≤ 0.02 µm - Število osi merjenja: ≥ 2 - Hitrost zajemanja: ≥ 2500 meritev/sec - EtherCAT komunikacijski modul - Čiščenje s stisnjenim zrakom - Detekcija mehurčkov v vlaknu po drugem nanosu zaščite: - Premeri oplaščenih vlaken v področju: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Min. zaznava višine mehurčka: ≤ 5 µm - Min. zaznava dolžine mehurcka: ≤ 50 µm - Hitrost vlečenja vlakna v področju: - hitrost_min ≤ 1 m/min - hitrost_max ≥ 3000 m/min - Kontrolna procesna enota mora vsebovati: - Zaslon na dotik. - Zaslon z uporabniškim vmesnikom mora podpirati prikazovanje naslednih procesnih parametrov: - Premer - Ovalnost - Mehansko napetost - Nihanje - Krožnost - Temperaturo - Vzdolžnih zračnih mehurčkov - Frekvenco vibriranja - Položaj optičnega vlakna z izrisom sipanja v grafikonu - Trendi in statistike - Število mehurčkov/zožitev - EtherCAT komunikacijski modul za pošiljanje vseh zgoraj navedenih podatkov na zunanji kontrolni sistem (ločena enota kupca) - Shranjevanje podatkov: HDD, USB spominski modul ali omrežje Kraj dobave: - Univerza v Mariboru, Fakulteta za elektrotehniko, računalništvo in informatiko, Laboratorij za elektrooptične in senzorske sisteme, Valvasorjeva ulica 75, 2000 Maribor, Slovenija Rok dobave: - Do 31. 3. 2023 Usposabljanje: - Ni potrebno Garancija - 18 mesecev od dneva podpisa prevzemnega zapisnika; Minimal technical requirements: - Measurement of bare fiber temperature close to furnace exit (high temperature): - Temperature range: Tmin ≤ 500 and Tmax ≥ 1500 °C - EtherCAT communication module - Fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Accuracy: ≤ ±1 °C - Measuring rate: ≥ 100 measurements/sec - EtherCAT communication module - Measurement of bare fiber diameter with airline detection close to furnace exit: - Glass fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Accuracy: ≤ 0.05 µm - Repeatability: ≤ 0.02 µm - Number of measurement axis: ≥ 2 - Measuring rate: ≥ 2500 measurements/sec - Airline detection: smallest detectable airline ≤ 0.5 µm - EtherCAT communication module - Air pressure cleaning system - Measurement of bare fiber temperature prior to primary coating application (low temperature): - Temperature range: Tmin ≤ 40 and Tmax ≥ 120°C - EtherCAT communication module - Fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Accuracy: ≤ ±1 °C - Measuring rate: ≥ 100 measurements/sec - EtherCAT communication module - Measurement of bare fiber diameter prior to primary coating application: - Glass fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Accuracy: ≤ 0.05 µm - Repeatability: ≤ 0.02 µm - Number of measurement axis: ≥ 2 - Measuring rate: ≥ 2500 measurements/sec - EtherCAT communication module - Air pressure cleaning system - Measurement of glass rod diameter up to 18mm: - Glass rod diameter measurement range: - OD_min ≤ 0.2 mm - OD_max ≥ 18 mm - Number of measurement axis: ≥ 2 - Accuracy: ≤ ± 0.2 µm - Repeatability: ≤ ± 0.1 µm - Exposure time: ≤ 0.2 µs - Measuring rate: ≥ 5000 measurements/sec/axis - Measurement of coated fiber diameter after application of primary coating: - Coated fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Accuracy: ≤ 0.05 µm - Repeatability: ≤ 0.02 µm - Number of measurement axis: ≥ 2 - Measuring rate: ≥ 2500 measurements/sec - EtherCAT communication module - Air pressure cleaning system - Measurement of coated fiber diameter after application of secondary coating: - Coated fiber diameter measurement range: - OD_min ≤ 50 µm - OD_max ≥ 500 µm - Accuracy: ≤ 0.05 µm - Repeatability: ≤ 0.02 µm - Number of measurement axis: ≥ 2 - Measuring rate: ≥ 2500 measurements/sec - EtherCAT communication module - Air pressure cleaning system - Fiber lump detector after application of secondary coating: - Coated optical fiber diameter range: - OD_min ≤ 100 µm - OD_max ≥ 500 µm - Min. Detectable Fault Height: ≤ 5 µm - Min. Fault Length: ≤ 50 µm - Speed of a drawing fiber: - speed_min ≤ 1 m/min - speed_max ≥ 3000 m/min - Processor system control unit shall include/facilitate: - Touch screen monitor. - Display with GUI allows demonstration of following production and products parameters: - Diameter - Ovality - Tension - Spinning - Concentricity - Temperature - Airlines - Vibration frequency - Optical fiber position with scatter plot presentation - Trend and statistics - Number of lumps/neckdowns - EtherCAT communication module for sending all above mentioned data to customer’s central control system - Data Storage: HDD, USB memory stick or network Equipment delivery: - University of Maribor, Faculty of Electrical Engineering and Computer Science, Laboratory for Electro-Optics and Sensor Systems, Valvasorjeva ulica 75, 2000 Maribor, Slovenia. Delivery time: - Until 31 March 2023 Training: - Not necessary Warranty - Full 18 Month warranty from the day of signature of the handover record. |
| Sklop 2: Sistem za obdelavo surovcev (preformov) Univerza v Mariboru naroča sistem za obdelovanje preformov, ki temelji na ultrazvočni tehnologiji. Ultrazvočna tehnologija mora omogočati natančno obdelavo kompleksnih geometrij obdelovancev v zahtevnih visokotehnoloških materialih, kot so keramika, kvarčno steklo, korund, volframov karbid ali celo kompoziti. Sistem se bo uporabljal predvsem za posebno obdelavo vlakenskih preformov. To na primer pomeni natančno vrtanje lukenj s premerom v razponu do nekaj milimetrov in do globine nekaj centimetrov, z visoko točnostjo. Zato mora biti sistem opremljen z ustreznim paketom za obdelavo stekla. Sistem za obdelavo preformov mora podpirati 5-osno delovanje. Sistem za obdelavo preformov mora imeti naslednje lastnosti: - Ultrazvočna tehnologija za rezkanje/ vrtanje/ brušenje - 5-osna miza V nadaljevanju so podane minimalne tehnične zahteve. Ponudba mora izpolnjevati ali presegati vse minimalne tehnične zahteve. |  | Lot 2: The preform processing system The University of Maribor is purchasing a versatile preform processing system, based on ultrasonic technology. The ultrasonic technology must allow precise processing of complex workpiece geometries in demanding high-tech materials such as ceramics, glass, corundum, tungsten carbide or even composites. The system shall be primarily in use for specialty machining of optical fiber preforms. This means for example, precise drilling of holes with diameters in the range of few microns, with high precision. Thus, the system must be equipped with an adequate glass machining package. The preform processing system must support 5-axes operation. The preform processing system must incorporate the following properties: - Ultrasonic technology for milling/ drilling/grinding - 5-axis simultaneous table Minimum technical requirements are given below. The offer must match or exceed all minimum technical requirements. |
| Številka postavke Item No.; Število kosov No. of pieces; ZAHTEVANO; REQUIRED. 1; 1; Minimalne tehnične zahteve: Osnovne strojne zahteve: - Delovno področje - Premik po oseh (X/Y/Z): ≥ 650/520/475 mm - Hitrost podajanja (X/Y/Z): ≥ 40 m/min - Obremenitev delovne mize: ≥ 300 kg - Vrtljiva miza za sočasno obdelavo - Območje vrtenja: vsaj med -35° in +110° - C-os z neposrednim pogonom za krožno brušenje - Premer mize: ≥ 650 mm - Navor motorja mize: ≤ 300 U/min - Maksimalni navor: - ≥ 500 Nm pri ≥ 100 U/min, - ≥ 200 Nm pri ≥ 200 U/min, - ≥ 100 Nm pri ≥ 300 U/min - Kabina: - Izvedba polovične kabine s streho in drsnimi vrati - Osvetlitev: ≥ 2 LED luči v stropni izvedbi; minimalna moč 33 W - Števec obratovanja: Vgrajen števec v krmilni omarici za prikaz napetosti in stanja delovanja sistema - Osrednje mazanje sistema: avtomatsko mazanje za vodila miz in vretena Glavni pogon vretena za standardno struženje: - Tip pogona: direktni prenos - Hitrost vrtenja: med vključno 20 in 15,000 min-1 - Motor vretena: HSK-A63/DIN 69893-1 - Uležajenje: okrogla postavitev - Menjalec orodja: ≥ 25 mest HSK-A63 - Moč pogona: ≥ 21 kW / 28 km (obratovalni cikel 40 %); ≥ 16 kW / 21.5 km (obratovalni cikel 100 %) - Navor vretena: ≥ 111 Nm / 82 ft lbs (obratovalni cikel 40 %); ≥ 85 Nm / 63 ft lbs (obratovalni cikel 100 %) - Hlajenje vretena: aktivna izvedba - Mazanje: oljno pod tlakom (meglica) - Zračno pihanje skozi središče vretena, aktivirano preko funkcije M Pogon vretena za ultrazvočno struženje: - Ultrazvočni princip struženja za obdelavo trdih materialov - Hitrost vrtenja: ≤ 15,000 min-1 - Indukcijska pretvorba ultrazvočnih visokih frekvenc iz vretena na nosilec orodja HSK-A63 - Vzbujanje piezo elementov, vgrajenih znotraj motorja za generiranje oscilacij (za generiranje vibracij in pomikov po Z-osi sistema) - Sistem za vzbujanje vibracij na rezalnem robu orodja v frekvenčnem pasu: med vključno 10 in 50 kHz - Amplituda oscilacij: >10 µm ≤ 15 µm - Sistem za detekcijo dejanske magnitude mehanske oscilacije - Sistem za avtomatsko sledenje ultrazvočne frekvence in amplitude med potekom procesa - Detekcija parametrov ultrazvočnega sistema - Avtomatska detekcija resonančne frekvence med izvajanjem procesa - Avtomatsko samo nastavljanje ultrazvočnega sistema Obvladovanje odrezkov in emulzije: - Pladenj za odrezke - Izpiranje odrezkov iz delovnega območja - Upogljiva šoba za dovod emulzije; upogljiva na vsaj 12 mestih - Sistem za pripravo emulzije in upravljanje z odrezki: - Zbiralnik za odrezke s filtrskim sistemom ter hladilnim sistemom - Sistem s papirnimi filtri, dolžina: ≥ 500 mm - Volumen zbiralnika za odrezke: ≥ 600 l - Globoko dodatno filtriranje s f50 sistemom filtrov - Pretok: ≥ 20 l/min; tlak ≥ 40 bar; frekvenčno reguliran pretok - Sistem za pripravo emulzije napaja podporne porabnike, kot je pištola za izpiranje znotraj kabine - Moč hladilnega sistema: ≥ 8000 W Programsko orodje za nadzor sistema za obdelavo preformov: - Programsko orodje mora omogočati napredni nadzor sistema - Nadzor in krmiljenje preko zaslona na dotik; diagonala: ≥ 21,5" - Programsko orodje mora zagotavljati enostavnost uporabe, omogočati nadzor in administracijo; omogočati izdelavo dokumentacije in vizualizacije procesa - Programsko orodje mora biti združljivo s CAD / CAM sistemi in PPS / ERP sistemi - Programsko orodje mora omogočati vizualizacijo in prilagoditve parametrov procesa za ultrazvočno obdelavo Orodja za meritev in nadzor: - Neposredni merilni sistem za X, Y in Z - Infrardeča sonda na dotik: - Avtomatsko nalaganje iz menjalca orodja v delovno vreteno - Korekcija obdelovanca z pozicioniranjem delovne mize - Možnost določanja centra in polmera kroga izvrtine ali valja obdelovanca - Ničelni premik - Možnost popravkov parametrov nastavljenih orodij - Možnost preverjanja toleranc obdelovanca - Laserska meritev orodja: - Fiksno postavljeno na steber pogona - Možnost meritve dolžine in premera orodja - Možnost umerjanja koničnega orodja - Ponovljivost: ≤ ± 0.001 mm - Orodje za nadzor in kompenzacijo konfiguracije kinematskih parametrov/točnosti pri 5-osnem delovanju Orodje za rezkanje, struženje, brušenje in poliranje: - Posebna diamantna orodja, namenjena za obdelavo materialov - Diamantni rezkalnik za točkovno vrtanje: premer (D): 1.2 mm; za vreteno HSK63-S; vpenjalni trn: ER16; 1 (en) kos - Diamantni svedri: s premeri (D): 2 mm, 3 mm, 5 mm (dolžina 30 x D); za vreteno: HSK63-S; vpenjalni trn ER16; 1 (en) kos od vsakega premera - Diamantni sveder: premer (D): 4 mm (dolžina 30 x D); za vreteno: HSK63-S; vpenjalni trn ER16; 2 (dva) kosa - Diamantno brusilno kolo: tip 6A9, premer: ≥ 45 mm; vreteno: HSK63-S; vpenjalni trn: ER16; 1 (en) kos - Diamantno brusilno kolo: tip 6A9, premer: ≥ 60 mm; vreteno: HSK63-S; vpenjalni trn: ER16; 1 (en) kos - Diamantni rezkarji: premeri (D): 4 mm, 6 mm, 8 mm (dolžina 3 x D) ; tip 2A2W; vreteno: HSK63-S; vpenjalni trn: ER16; 1 (en) kos od vsakega premera Tehnična podpora: - Ponudnik mora nuditi tehnično podporo v obdobju 1 (enega) leta od dneva podpisa prevzemnega zapisnika - Tehnična podpora mora vključevati svetovanje pri nastavitvi parametrov, konfiguraciji sistema in pomoč pri izvedbi posameznih procesov. - Tehnična podpora se lahko izvaja preko oddaljenega dostopa do sistema za obdelavo preformov. Zahteve za izpolnitev pogojev ob sprejemu: predpogoj za podpis prevzemnega zapisnika je med drugim tudi: - Demonstracija in izdelava luknje s premerom 4 mm in globine 300 mm v okroglo palico iz kvarčnega stekla. - Demonstracija in izdelava ravne ploskve v prečni ravnini okrogle palice iz kvarčnega stekla. - Demonstracija in izdelava preprostega bloka s fino poliranimi stranicami na vrhu okrogle palice iz kvarčnega stekla. Vnos sistema za obdelavo preformov v objekt: - Vnos sistema v objekt bo izveden skozi vrata z dimenzijami (Š x V): 2400 mm x 2650 mm Rok dobave: - Do 20. 3. 2023 Garancijski rok: - Najmanj 1 (eno) leto od dneva podpisa prevzemnega zapisnika. Celotna 1-letna garancija vključuje letni servis. Usposabljanje: - Dobavitelj mora izvesti osnovno začetno usposabljanje, ki vključuje strojne in programske module na dobavljenem predmetu pogodbe za najmanj 1 (eno) osebo uporabnika v obsegu 1 (enega) delovnega dneva.; Minimum technical requirements: Basic machine requirements: - Work area - Traverse distances (X/Y/Z): minimal 650/520/475 mm - Feed rate (X/Y/Z): ≥ 40 m/min - Table load: ≥ 300 kg - Swivel/rotary table for simultaneous machining - Swivel range: minimal from -35° to +110° - Direct drive C-axis for circular grinding operations - Table diameter surface: ≥ 650 mm - Table torque: ≤ 300 U/min - Torque max.: - ≥ 500 Nm at ≥ 100 U/min, - ≥ 200 Nm at ≥ 200 U/min, - ≥ 100 Nm at ≥ 300 U/min - Cabin: - Half-cabin with sliding door and roof (closed) - Machine lamp: minimal 2 LED lamp (planonlight), minimal 33 Watt - Operating hour meter: Meter at the electric control cabinet for „control voltage ON“ and “Program in operation” - Central lubrication: automatic minimal lubrication for roller guides and ball screw spindles Main drive for conventional milling: - Drive type: direct drive - Speed: 20 - 15,000 min-1 - Motor spindle: HSK-A63/DIN 69893-1 - Bearing: O-arrangement - Tool magazine: ≥ 25 places HSK-A63 - Drive capacity: minimal 21 kW / 28 hp (40% duty cycle); minimal 16 kW / 21.5 hp (100% duty cycle) - Spindle torque: minimal 111 Nm / 82 ft lbs (40% duty cycle); minimal 85 Nm / 63 ft lbs (100% duty cycle) - Spindle cooling: active - Lubrication: oil mist - Air blast through spindle center activated by M function Ultrasonic drive and technology for hard machining: - Ultrasonic technology for hard machining - Speed: maximal 15,000 min-1 - Transformation of the ultrasonic high frequency from the spindle to the HSK-A63 tool holder via induction - Stimulation of the Piezo elements inside the oscillation motor (generation of a vibration in Z) - Generation of a defined vibration frequency at the cutting edge of the tool: 20-50 kHz - Amplitude of oscillation: >10 µm ≤ 15 µm - Amplitude detection of the actual mechanical oscillation amplitude - Auto tracking of ultrasonic frequency and amplitude in the machining process monitoring of the existing process forces - Ultrasonic parameter detection - Automatic detection of the resonance frequency in the machining process - Ultrasonic auto tuning Chip disposal and coolant supply: - Chip tray - Chip flushing for working area - Coolant ring nozzle; minimal 12-fold - Inner coolant supply and sedimentation preparation system: - Coolant reprocessing plant with sedimentation tank and cooling unit based on filter system - Paper filter type: ≥ 500 mm - Sediment tank capacity: ≥ 600 l - f50 deep filtration system - Flow: minimal 20 l / min; minimal 40 bar frequency regulated - Inner coolant supply preparation system supplies other coolant consumers such as a flushing gun via valves in the machine - Cooling unit power: ≥ 8000 W Preform processing system control: - Comprehensive control for the entire system - Multi-Touchscreen 21,5" - System and applications must simplify the operation and enable the consistent administration, documentation and visualization of order, process, and machine data. - System connectable with CAD / CAM and compatible to PPS / ERP - Application for visualization and adaption of the ultrasonic parameters of the machine/system Measuring and monitoring tools: - Direct measuring system X, Y, Z - Infrared touch probe: - automatic loading from the tool magazine into the work spindle - Correction of workpiece position by positioning the NC rotary table - Determination of circle center and radius of a bore or a cylinder - Zero shift - Modification of tool correction - Workpiece tolerance checks - Laser tool measuring: - Fix mounted at machine column - Tool length and diameter measuring - Calibration tool with taper - Repetitive accuracy: ≤ ± 0.001 mm - Tool kit for control and compensation of the cinematic accuracy of 5 axis machine configurations Milling, grinding, and polishing tools: - Special diamond tools for fused silica material processing: - Diamond milling cutter for spot drilling: diameter (D): 1.2 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond drills: diameters (D): 2 mm, 3 mm, 5 mm (length 30 x D); Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece of each diameter - Diamond drill: diameter (D): 4 mm (length 30 x D); Spindle: HSK63-S; Tool holder: ER16; 2 (two) pieces - Diamond grinding wheel: type 6A9, diameter: ≥ 45 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond grinding wheel: type 6A9, diameter: ≥ 60 mm; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece - Diamond milling cutters: diameters (D): 4 mm, 6 mm, 8 mm (length 3 x D); type 2A2W; Spindle: HSK63-S; Tool holder: ER16; 1 (one) piece of each diameter Technical support: - The supplier must provide technical support for a period of 1 (one) year from the day of signature of the handover record. - Technical support must include consulting on setting parameters, use of the machine, configuring the system, and assisting in the implementation of individual processes, etc. - Technical support can be provided through remote access to the preform processing system. Acceptance criteria: a prerequisite for the signature of the handover record is also: - Demonstration of drilling a hole with diameter of 4 mm and depth of 300 mm in a fused silica rod. - Demonstration of milling a flat surface on a fused silica rod. - Demonstration of milling a simple square block with fine finish (polished) sides on a fused silica rod. Delivery of the preform processing system into the facilities: - The system will be delivered into the building through a door with dimensions (W x H): 2400 mm x 2650 mm Delivery time: - Until 20 March 2023 The warranty period: - At least 1 (one) year from the day of signature of the handover record, including annual service. Training: - The supplier must carry out basic initial training, which includes hardware and software modules on the delivered subject of the contract for at least 1 (one) person within the scope of 1 (one) working day.. |

Operacijo sofinancirata Republika Slovenija, Ministrstvo za izobraževanje, znanost in šport in Evropska unija iz Evropskega sklada za regionalni razvoj. Operacija se izvaja v okviru Operativnega programa za izvajanje evropske kohezijske politike 2014-2020 v okviru prednostne osi 1: »Mednarodna konkurenčnost raziskav, inovacij in tehnološkega razvoja v skladu s pametno specializacijo za večjo konkurenčnost in ozelenitev gospodarstva«, prednostne naložbe 1.1 »Krepitev infrastrukture za raziskave in inovacije ter zmogljivosti za razvoj odličnosti na tem področju, pa tudi spodbujanje pristojnih centrov, zlasti takšnih, ki so evropskega pomena«, specifičnega cilja 1.1.1 »Učinkovita uporaba raziskovalne infrastrukture ter razvoj znanja/kompetenc za boljše nacionalno in mednarodno sodelovanje v trikotniku znanja«.

The operation is co-financed by the the Republic of Slovenia, Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is carried out under priority axis 1: ''International competitiveness of research, innovation and technological development, in line with smart specialization for increased competitiveness and greening of the economy'', priority investment 1.1 ''Strengthening R&D infrastructure and capacity to develop excellence in this and the promotion of competent centers, especially those of European importance'', specific objective 1.1.1 ''Efficient use of research infrastructure and development of knowledge / competences for better national and international cooperation in the knowledge triangle'' under the Operational Program for implementation of the European cohesion policy 2014-2020.