Project name: Upgrading national research infrastructures RIUM

No. 302/46 – RIUM84-P12-FS09-1/2022

Technical Specifications for “Purchase of flexible manufacturing cell for machining of prismatic workpieces by machining and forming processes”

- SUBJECT-MATTER OF PUBLIC PROCUREMENT

The public procurement comprises a flexible machining cell for machining prismatic workpieces by cutting and forming processes with the configuration below. The machining cell includes all the equipment for performing 5-axis simultaneous machining with milling and incremental forming. The processing cell enables integration into the smart factory system according to the principles of Industry 4.0.

- TECHNICAL REQUIEREMENTS

Minimum technical requirements for individual lots are provided below. The tenders must meet or exceed all minimum technical requirements.

Regarding the equipment, the tenderer has to provide the following (without additional costs for the Contracting Authority):

- equipment as follows from Technical Specifications, has to be new, packed and delivered to the specified place of the User, at the same time and completely;

- adequate transport of equipment from the manufacturer/supplier to the placement location, transport insurance, unloading, placement to the C101 Laboratory of the User (ground floor, Smetanova 17, 2000 Maribor);

- placement, installation and connection of all elements or units of equipment at the location University of Maribor, Faculty of Mechanical Engineering, Smetanova ulica 17, 2000 Maribor, in Laboratory C101

- connecting all units that require a connection to the existent installations/systems for their operation;

- compatibility and interconnectivity of all units of the offered equipment;

- rectification of all faults that would occur due to a possible mutual incompatibility, non-interconnectivity or inadequate connection into a single, flawlessly functioning system;

- test start-up in line with the Technical Specifications, with all consumables that are/may be required for operation;

- demonstration with training which includes all hardware and software units on the supplied subject-matter of the contract, if training is required in an individual lot;

- handing over manufacturer’s technical documentation and instructions for handling the equipment in electronic and printed form for hardware and software as well as user programmes, all of them at least in English.

The tender has to include all small/supporting equipment needed for the installation and start-up, even if it is not explicitly stated in the technical requirements.

The equipment price has to be given in euro and include all elements of which it is composed (equipment per items and accompanying services, guarantee period, any discounts, transport costs, costs of labour, delivery and assembly, placement, start-up and demonstration of operation, insurance, training, instructions, etc.) or all costs related to the implementation of the concerned public contract.

The tender value is the specified DDP of the Contracting Authority (Incoterms 2010 – Delivery Duty Paid).

The tenderers have to submit a tender for all items of the equipment within the particular lot. The Contracting Authority shall not consider tenders relating only to a part of the equipment or shall identify such tenders as inadmissible.

Flexible manufacturing cell for machining of prismatic workpieces by machining and forming processes

The lot comprises a flexible machining cell for machining prismatic workpieces by cutting and forming processes with the configuration below. The machining cell includes all the equipment for performing 5-axis simultaneous machining with milling and incremental forming. The processing cell enables integration into the smart factory system according to the principles of smart manufacturing.

Requirements regarding supply, training, guarantee period and delivery deadline:

Supply and placement:

- delivery of equipment to the address Smetanova 17, 2000 Maribor, Laboratory C-101,

- placing and connecting the machine to the place of use and

- test start-up of the machine.

Education/training:

- On the machine and working with the machine 5 days for 8 hours for at least 5 participants at the location of the User.

Guarantee period:

- At least 18 months from the signing of the handover record.

Delivery deadline:

- 120 days from the date of entry into force of the contract.

| Item number | Number of pieces | Required |
| --- | --- | --- |
| MAIN EQUIPMENT |  |  |
| 1.1 | 1 | Flexible machining cell for machining prismatic workpieces by cutting (milling, drilling, reaming, threading) and forming processes (incremental forming): - Simultaneous control for machining in five working axes (3 translational axes X, Y, Z and 2 rotary axes A, B or C). - Working space (linear axes): - X: at least 600 mm. - Y: at least 450 mm. - Z: at least 400 mm. - Controlled rotary axes: - Tilting table: at least 145° - Rotary table: at least 360° - Swivel rotary table at least 500 mm in diameter, for simultaneous 5 axes machining. - CNC control (Siemens Sinumerik 840D-equivalent or better) with following hardware: - 32-bit processor with decentral bus concept. - Communication interfaces (MPI, industrial ethernet, Profibus, ASI-bus). - Program memory on CF card that can be used in standard with EES. - Programmable logic controller (PLC) with specification equivalent to (or better) AS 317-2DP with Profibus. - CNC keyboard, control panel. - 15 MB memory on control, and at least 4 GB on removable media. - Ethernet interface inside machine control cabinet: Fast Ethernet at least 10/100 BaseT. - TCP/IP network protocol. - Interface (at least USB 2.0) on the command station. - Multi-touch screen. - HSK-A63 spindle, with spindle speed at least 16.000 min-1. - At least 100 Nm main spindle torque. - At least 35 m/min feed rate for linear axes. - Tool magazine: at least 30 pockets HSK-A63. - Cooling: - By air blast trough spindle centre, - by coolant trough spindle centre, - coolant pressure at least 20 bar. - Active cooling for main spindle, drives and table. - Chip removal by chip conveyor. - Integrated tool measuring device. - Measurement system for probing of workpiece position. - Support for hydraulic clamping system. - CAD model of the machine used for machine simulation. - Instructions in English, German, or Slovenian language in printed or electronic form. |
| SUPPORT EQUIPMENT |  |  |
| 1.2 | 1 | Screw compressor: - Output power of at least 7 kW. - Nominal pressure at least 10 bar. - Volume flow at least 1 m3/min. - Integrated pressure tank at least 250 l. - Water-oil separator. - Instructions in English or Slovenian language in printed or electronic form. |
| 1.3 | 1 | Hydraulic clamping system adapted for the machine (1.1): - Hydraulic clamp at least 100 mm wide, with base and "V" jaws, complete with base plate adapted for "zero" clamping (equivalent to HILMA Z 100 H or better). |
| 1.4 | 1 | Set of clamping units (masters), adaptors and accessories for spindle HSK-A63 according to DIN 69893 / ISO 12164, for tool clamping: - Master, CaptoTM, HSK-A63, C5, 1 piece. - Extension C5-C5, 1 piece. - Collet chuck C5-ER32, 1 - Master, typ Weldon-12, HSK-A63, 1 piece. - Master, typ Weldon-16, HSK-A63, 1 piece. - Master, typ Weldon-20, HSK-A63, 1 piece. - Master, typ Weldon-25, HSK-A63, 1 piece. - Master, typ Weldon-32, HSK-A63, 1 piece. - Master, hydro-expansion, HSK-A63, 1 piece. - Adaptor for hydro-expansion, 3 mm, 1 piece. - Adaptor for hydro-expansion, 4 mm, 1 piece. - Adaptor for hydro-expansion, 5 mm, 1 piece. - Adaptor for hydro-expansion, 6 mm, 1 piece. - Adaptor for hydro-expansion, 8 mm, 1 piece. - Adaptor for hydro-expansion, 10 mm, 1 piece. - Master for collet chuck ER25, HSK-A63-ER25, 1 piece. - Master for collet chuck ER32, HSK-A63-ER32, 1 piece. - Master for collet chuck ER40, HSK-A63-ER40, 1 piece. - Master for collet chuck ER16, HSK-A63-ER16, 1 piece. - Master for collet chuck ER20, HSK-A63-ER20, 1 piece. - Set of collets ER16, 1 piece. - Set of collets ER20, 1 piece. - Set of collets ER25, 1 piece. - Set of collets ER32, 1 piece. - Set of collets ER40, 1 piece. - Master for milling head 16, with inbus screw, HSK-A63, 1 piece. - Master for milling head 22, with inbus screw, HSK-A63, 1 piece. - Master for milling head 27, with inbus screw, HSK-A63, 1 piece. - Master for milling head 32, with inbus screw, HSK-A63, 1 piece. - Master for milling head 16, with cross, HSK-A63, 1 piece. - Master for milling head 22, with cross, HSK-A63, 1 piece. - Master for milling head 27, with cross, HSK-A63, 1 piece. - Master for milling head 32, with cross, HSK-A63, 1 piece. - Master with drilling head, HSK-A63, 2.5-16.0, 2 pieces. - Master with Morse cone, HSK-A63-MK1, 2 pieces. - Master with Morse cone, HSK-A63-MK2, 2 pieces. - Master with Morse cone, HSK-A63-MK3, 2 pieces. - Master with Morse cone, HSK-A63-MK4, 2 pieces. - Master with Morse cone and screw, HSK-A63-MK1, 1 piece. - Master with Morse cone and screw, HSK-A63-MK2, 1 piece. - kos Master with Morse cone and screw, HSK-A63-MK3, 1 piece. - Master with Morse cone and screw, HSK-A63-MK4, 1 piece. - Master shrink-fit 6, HSK-A63, 1 piece. - Master shrink-fit 8, HSK-A63, 1 piece. - Master shrink-fit 10, HSK-A63, 1 piece. - Master shrink-fit 12, HSK-A63, 1 piece. - Master shrink-fit 16, HSK-A63, 1 piece. - Master shrink-fit 20, HSK-A63, 2 pieces. - Master shrink-fit 25, HSK-A63, 1 piece. - Master shrink-fit 32, HSK-A63, 1 piece. - Extension, shrink-fit 3, 3mm/20mm, 1 piece. - Extension, shrink-fit 4, 4mm/20mm, 1 piece. - Extension, shrink-fit 5, 5mm/20mm, 1 piece. - Master for threads with axial compensation, HSK-A63, M3-M12, 1 piece. - Master for threads with axial compensation, HSK-A63, M6-M20, 1 piece. |

The operation is co-financed by the Republic of Slovenia, the Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is implemented under the Operational Program for the Implementation of European Cohesion Policy 2014-2020 under Priority Axis 1: "International competitiveness of research, innovation and technological development in line with smart specialization for greater competitiveness and greening of the economy", priority investment 1.1 "Strengthening infrastructure for research and innovation and capacity to develop excellence in this field, as well as the promotion of competent centers, especially those of European importance ", specific objective 1.1.1" Effective use of research infrastructure and development of knowledge/competences for better national and international cooperation in the triangle knowledge ”.

302/46 – RIUM84-P12-FS09-1/2022