Project title: Upgrading National Research Infrastructures – RIUM

COMPLIANCE DECLARATION WITH MINIMUM TECHNICAL REQUIREMENTS

OF THE OFFERED EQUIPMENT

Name of the tenderer:

Address of the tenderer:

Subject of the public procurement: Equipment for refining and reshaping semiconductor, photonic and other slices – Lot 1

LOT 1: Photolithographic system

Designation of the public procurement:  302/43 - RIUM83-FERI03.5_P20_20/2022

| Is the tenderer also the manufacturer of the offered equipment? | YES ☐ | NO ☐* |
| --- | --- | --- |
| *If the answer is NO, enter the equipment manufacturer's name and address: |  |  |

INSTRUCTIONS:

- This Annex No. 2 must be completed and signed. The tenderer must mark whether the offered equipment is meeting (or exceeding) the required technical specifications of the offered equipment according to all Contracting Authority’s requirements.

- The tenderer uploads this Attachment No. 2 in Word and PDF format (including the enclosed means of proof for meeting all technical requirements) to the e-JN information system, section “Other attachments”.

We declare that the offered equipment fully meets the minimum technical requirements of the client for the public procurement titled " Equipment for refining and reshaping semiconductor, photonic and other slices – Lot 1", and that we are ready to realize the order according to the following specification:

| EQUIPMENT ☐ |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | REQUIREMENTS | OFFERED |  | DATASHEET |
| 1 | Photolithographic system |  |  |  |
|  | 1.st: Spin coater for applying a thin layer |  |  |  |
|  | - Maximum dimensions (WxDxH): 300 x 455 x 300 mm | ☐ YES | ☐ NO |  |
|  | - Spinner module for wafers up to 150 mm round or up to 100 mm square substrates | ☐ YES | ☐ NO |  |
|  | - Vacuum chuck for standard wafers up to 150 mm | ☐ YES | ☐ NO |  |
|  | - Vacuum chuck for pieces of small dimensions for sample size diameter 12 mm or larger | ☐ YES | ☐ NO |  |
|  | - Controlling system with GUI and colour touchscreen | ☐ YES | ☐ NO |  |
|  | - Rugged, transparent lid hinge | ☐ YES | ☐ NO |  |
|  | - Process bowl | ☐ YES | ☐ NO |  |
|  | - Cover open interlock | ☐ YES | ☐ NO |  |
|  | - Speed: programable speed setting from at least 5 rpm to at least 10000 rpm (+/- 1 rpm) | ☐ YES | ☐ NO |  |
|  | - Acceleration: ≥ 25000 rpm/s | ☐ YES | ☐ NO |  |
|  | - Spinning time: Programmable from at least 1s to at least 9999 s | ☐ YES | ☐ NO |  |
|  | - Program storage for recepies: At least 150 recipes | ☐ YES | ☐ NO |  |
|  | - External connection of USB drive in the controller | ☐ YES | ☐ NO |  |
|  | - Drain connection | ☐ YES | ☐ NO |  |
|  | - Vacuum Pump (Oilfree) | ☐ YES | ☐ NO |  |
|  | - Central syringe holder with at least triple connectors | ☐ YES | ☐ NO |  |
|  | - Standard syringe holders | ☐ YES | ☐ NO |  |
|  | 2.nd: Spin coater for development |  |  |  |
|  | - Maximum dimensions (WxDxH): 300 x 455 x 300 mm | ☐ YES | ☐ NO |  |
|  | - Spinner module for wafers up to 150 mm round or up to 100 mm square substrates | ☐ YES | ☐ NO |  |
|  | - Vacuum chuck for standard wafers up to 150 mm | ☐ YES | ☐NO |  |
|  | - Vacuum chuck for pieces of small dimensions for sample size diameter 12 mm or larger | ☐ YES | ☐NO |  |
|  | - Controlling system with GUI and colour touchscreen | ☐ YES | ☐ NO |  |
|  | - Rugged, transparent lid hinge | ☐ YES | ☐ NO |  |
|  | - Process bowl | ☐ YES | ☐ NO |  |
|  | - Cover open interlock | ☐ YES | ☐ NO |  |
|  | - Speed: programable speed setting from at least 5 rpm to at least 10000 rpm (+/- 1 rpm) | ☐ YES | ☐ NO |  |
|  | - Acceleration: ≥ 25000 rpm/s | ☐ YES | ☐ NO |  |
|  | - Spinning time: Programmable from at least 1s to at least 9999 s | ☐ YES | ☐ NO |  |
|  | - Program storage for recepies: At least 150 recipes | ☐ YES | ☐ NO |  |
|  | - External connection of USB drive in the controller | ☐ YES | ☐NO |  |
|  | - Drain connection | ☐ YES | ☐ NO |  |
|  | - Vacuum Pump (Oilfree) | ☐ YES | ☐ NO |  |
|  | - Central syringe holder with at least triple connectors | ☐ YES | ☐ NO |  |
|  | 3.rd: Hot plate |  |  |  |
|  | - Maximum dimensions (WxDxH): - 320x450x200 mm | ☐ YES | ☐ NO |  |
|  | - Table top hot plate for wafers up to 150 mm minimum | ☐ YES | ☐ NO |  |
|  | - Hinged lid | ☐ YES | ☐ NO |  |
|  | - Lifting pins for substrate handling | ☐ YES | ☐ NO |  |
|  | - Proximity pins | ☐ YES | ☐ NO |  |
|  | - Vacuum bake holders | ☐ YES | ☐ NO |  |
|  | - Temperature range: 50 ≥ T ≥ 230 °C | ☐ YES | ☐ NO |  |
|  | - Temperature uniformity: Max.: +/- 0.5 °C | ☐ YES | ☐ NO |  |
|  | - Number of recipes: ≥ 10 | ☐ YES | ☐ NO |  |
|  | - 230 VAC power supply with EU power | ☐ YES | ☐ NO |  |

| Place and date: | Stamp: | Authorized signature: |
| --- | --- | --- |

The operation is co-financed by the Republic of Slovenia, Ministry of Education, Science and Sport and the European Union from the European Regional Development Fund. The operation is carried out under priority axis 1 of the Operational Program for Implementation of the European Cohesion Policy 2014–2020: “International competitiveness of research, innovation and technological development, in line with smart specialization for increased competitiveness and greening of the economy”, priority investment 1.1 “Strengthening R&D infrastructure and capacity to develop excellence in this and the promotion of competent centers, especially those of European importance”, specific objective 1.1.1 “Efficient use of research infrastructure and development of knowledge/competences for better national and international cooperation in the knowledge triangle”.

The tenderer must mark whether the offered equipment is meeting (or exceeding) the required technical specifications of the offered equipment according to all Contracting Authority’s requirements.

The tenderer must give the name of the attached catalogue or brochure of the equipment manufacturer and the page number(s), together with the relevant technical specification. In exceptional cases, the tenderer may also provide a link to the equipment manufacturer's website with the relevant technical specification. Where a link to the online document is provided, the tenderer shall indicate a page number, indicating the relevant technical specification.