SPECIFICATIONS

| Contracting Authority | Inštitut za kovinske materiale in tehnologije Lepi pot 11 1000 Ljubljana |
| --- | --- |
| Public contract reference number | JN 2024/5 |
| Subject matter of the public contract | Naprava za selektivno lasersko taljenje kovinskih materialov z uporabo umetne inteligence za in situ spreminjanje mikrostrukture - Device for laser powder bed fusion for metallic materials using artificial intelligence for in situ microstructure modification. |

TYPE, CHARACTERISTICS, QUALITY AND APPEARANCE OF THE PUBLIC CONTRACT SUBJECT/TENDER

Device for laser powder bed fusion for metallic materials using artificial intelligence for in situ microstructure modification.

- Build space: at least Ø 150 mm x H 150 mm (without preheating) - Possibility of build space reduction: Ø 55 mm x H 190 mm (without preheating) - 3D Scan unit for focus variation during printing - The preheating temperature of the base plate: up to at least 1200 °C - Two laser sources: - first laser at least 400 W single-mode fiber laser - second laser switchable at least 1000 W / 3000 W laser with single-mode / multi-mode fiber laser - Process monitoring on-axis high-speed pyrometer - Air- water chiller 15 kW - Possibility of inline power control as later upgrades (Close loop by measuring emissions and controlling the laser power within at least 80 kHz sampling rate) - 4 digital I/O signals (24 V) and 2 analogues I/O signals 0-10 V with ability to address these signals for external sensor or actuator integration - Script based build job definition with ability to specify input-specific if-loops as condition-based job definition - Full access to all process parameters