DAB TRANSMITTERS

TECHNICAL REQUIREMENTS

AND

SPECIFICATIONS

FOR PUBLIC TENDER

No. JN-B1060

1	Introduction	4

2	General	5

2.1	CE and EMC	5

2.2	Instructions for operation and maintenance	5

2.3	Noise	5

2.4	Internal Parts	5

2.5	External Parts	6

2.6	Transportation	6

2.7	Physical Dimensions	6

2.8	Connectors	6

2.9	Protection loop - interlock	6

2.10	Local interface	6

2.11	Remote interface	6

2.11.1	Web GUI	7

2.11.2	SNMP interface	7

2.12	Event log	7

2.13	Warranty and after warranty remote support	7

3	Technology	8

3.1	Exciter	8

3.1.1	Input Signal	8

3.1.2	Pre-correction	8

3.1.2.1	Manual pre-correction	8

3.1.2.2	Adaptive pre-correction	8

3.1.3	Transmission Mode	8

3.1.4	TII	9

3.1.5	Test Signal/PRBS	9

3.2	Working Frequency	9

3.3	Synchronization	9

3.3.1	Frequency and time reference	9

3.3.2	Stability	9

3.3.3	Internal Reference	9

3.3.4	External Reference	9

3.3.5	Behavior in case of GPS Failure	10

3.3.6	Dynamic Network Compensation Delay	10

3.3.7	Static Transmitter Compensation Delay	10

3.3.8	Transmitter Offset Delay:	10

3.4	Transmitter power	10

3.4.1	Output power	10

3.4.2	Nominal power	10

3.4.3	Power consumption	10

3.4.4	Stability	11

3.4.5	Return Loss	11

3.4.6	Efficiency	11

3.5	Output stage	11

3.6	Measurement/test points	11

3.6.1	RF test points	11

3.6.2	Isolation points	11

4	Quality of operation	12

4.1	Oscillator phase noise	12

4.2	Shoulder attenuation	12

4.3	Spectrum mask	12

4.4	Spurious emissions	13

4.5	Crest factor	13

4.6	Modulation error ratio / MER	13

5	Mains voltage	13

5.1	Connection to the mains supply	13

5.2	Switch-on	13

5.3	Uninterruptible Power Supply	13

5.3.1	Built-in power supply	13

5.3.2	External power supply	13

6	Output channel band-pass filter	14

7	Cooling system	14

7.1	Liquid Cooling	14

7.2	Air Cooling	14

8	Control and monitoring	14

8.1	Event log	15

8.2	Local interface	16

8.3	Remote interface	17

8.3.1	Web GUI	17

8.3.2	SNMP/MIB	17

# Introduction

DAB transmitter is device that broadcasts DAB+ signal in accordance with:

- Standard ETSI EN 300401 V1.4.1.

- Technical Report ETSI TR 101496

The incoming EDI data stream is modulated into a DAB+ COFDM signal suitable for feeding one or more power amplifier stages.

Transmitter can consist of several functional stages combined into logical units:

- Exciter forms a DAB+ COFDM signal from input EDI stream and modulates it on to the required carrier frequency.

- GPS receiver provides reference timing signals.

- The power amplifier stage amplifies the RF signal to a required level.

- Band-pass channel filter eliminates all unwanted signal components.

- The cooling system enables the operation of a transmitter in a proper temperature range by cooling the appropriate number of transmitter units.

- The control unit enables control and supervision of the whole transmitter.

Picture 1: DAB+ Transmitter

# General

## CE and EMC

All equipment must meet technical requirements and standards in force in the Republic of Slovenia and related to electrical power supplies (230V / 400 V, 50 Hz), grounding and safety measures against electrical current strike:

- EN 50160,

- EN 60950,

- EN 61140 and

- Power factor - cos phi ≥ 0,95.

Guidance TSG-N-003 2008 – Safety actions against lightning must be fulfilled.

All offered equipment must also fulfill following Slovenian EMC standards:

- Technical EMC norm (ur.l.RS št. 132/06),

- EN 50081-1,

- EN 50082-2,

- EN 55022 and

- EN 55024.

## Instructions for operation and maintenance

Technical documentation needed for user’s daily work and maintenance must be in Slovene or English language. For technical documentation in electronic form Acrobat PDF format must be used. The documentation must reflect the current state of the equipment at the time of delivery. Documentation must include a user manual and service manual. By using this information, the user must be able to repair the equipment.

For each device it is required to provide one copy of documentation in printed and electronic form. If there are 2 or more devices of the same type, it is required to provide documentation in an according number of copies (not more than 3) in printed and electronic form for each type of device.

The technical documentation must comprise of at least these items:

- Description of operation of the device and individual stages,

- Description of procedures for setting parameters and retuning of the device,

- Detailed schematic design of all stages of the device,

- List and description of all service exercises as required by the producer,

- Guidelines for repair and exchange of parts,

- Description of program functions and error messages,

- Protocols of all accessible control and supervisory interfaces,

- All system and administrative passwords,

- Declaration of all dangerous substances used and guidance for handling and removal.

The labels on the equipment must be in Slovene or English language.

## Noise

The noise level at nominal operation must not exceed 65 dBA measured at the height of 1,5 m and at 1 m distance from the device.

## Internal Parts

Parts of the device that are to be installed inside the building and will not be exposed to the weather conditions must operate under the following conditions:

- Temperature range: +5 °C ÷ 45 °C

- Relative humidity: <90%

- Input air temperature: <35 °C

all devices must meet the quality requirements in the temperature range from +10 °C to +35 °C at 1200 m above sea level.

## External Parts

Parts of the device, which are intended to be used outdoor, and thus exposed to the weather conditions, must operate under the following conditions:

- Temperature range: -30 °C ÷ 45 °C

- Relative humidity: 8 ÷ 100%

- Wind speed: 50 m/s

- Sun radiation power flux density: 1000 W/m2

## Transportation

During the transportation a device in non-working state must withstand:

- Temperature range: -40 °C ÷ 55 °C

- Relative humidity: 8 ÷ 100%

## Physical Dimensions

The depth of the whole device without the external cooling unit must be in a standard 19'' cabinet. The depth must not exceed 1m.

If a modular design of the transmitter is required is required and transmitter units are mounted in a cabinet, all spaces on the front side that are not completed must be closed with blind covers.

## Connectors

Only the following connector types can be used:

- BNC 75 Ω or 50 Ω,

- N,

- SMA, SMB, TNC,

- DIN 7/16,

- 7/8”, 1 5/8”,1/8" EIA,

- RJ45

For devices up to 200 W output power output RF connectors must be of type N-female or type DIN 7/16-female.

## Protection loop - interlock

The device must be equipped with a connector for protective loop (interlock) functionality.

If the protection loop is opened the transmitter must be automatically switched off.

## Local interface

For fast device state recognition light indication according to following key must be used:

- Green – normal operation,

- Yellow – warning (abnormal operation without interruption of service),

- Red – fault (interruption of service).

The operator must be able to manage all parameters of the device by use of the local interface, which consists of a display and keyboard.

## Remote interface

Remote control is based on IP over 10/100BaseT Ethernet network. Therefore, the device must have a LAN interface with RJ-45 connector according to the ISO 8877 specification.

The solutions to access the remote interface via the data network RJ45 connectors with separate VLAN is also allowed.

It must be possible to reset the IP, username and password in one of the following ways:

- Hidden switch or button on the device or

- Local interface or

- Serial interface RS232 and open terminal access without username and password or

- Remote interface.

It must be impossible to damage the device through the remote interface.

The remote control must have a web interface and a SNMP interface.

### Web GUI

The web user interface is the main remote interface. The single window web interface must provide complete monitoring and control over the device and all attached sub-devices. It must be possible to make software upgrades though the Web-GUI.

A two-level user interface with username and password must be implemented.

The web interface must allow simultaneous access for multiple users - 3 or more.

Transfer of each web site must be completed in less than 4 seconds.

Process data and status information inside the web interface must be refreshed in regular intervals (up to 15 seconds) with the possibility to disable or change.

It must be possible to save and restore device configuration by a single click.

An action history for logging of user action must be implemented.

The web interface must enable browsing, clearing and saving of the device log. In the case of large logs, pagination with not more than 40 events per page must be implemented.

It must be possible to easily (one-click) export the whole device log in CSV format.

The web interface must be fully functional in the web browsers compatible to Mozilla Firefox 10 and Microsoft Internet Explorer 10 running on Linux or Windows operating system.

### SNMP interface

The UDP based SNMP agent for monitoring and control must allow complete control over the device. All needed MIB structures in ASN.1 format must be included. The SNMP must send messages (traps) on failure (urgentAlarmTrap), warning (WarningTrap) and other informational events (InfoTrap).

At the end of failure or warning the device must send an appropriate message (endofAlarmTrap). The web interface must allow changing at least following SNMP agent functionalities:

- At least four IP addresses for sending (Trap) notifications,

- Disable/enable of notification sending for various events (Alarm, Warning and Info).

The functions GET, SET, WALK and TRAP must be supported.

## Event log

The device must track all events/warnings/alarms in an event log. Each log entry must be equipped with the date and time of event beginning and end of event. The event log must be organized as a circular buffer. If the event log buffer is full new entries must overwrite the oldest.

Event timestamps must be based on local time. The local time must be possible to set manually and automatically through synchronization via:

- GPS reference or

- Network time protocol – NTP.

The log must allow storage of more than 1000 entries. Entries in the log must be listed in reverse order, youngest first. By analyzing the event log, it must be possible to find the cause of the failure.

The event log must not be deleted in case of power failure or when device is switched off.

## Warranty and after warranty remote support

Remote access for support/maintenance will be enabled over the internet. For this purpose, access to an intermediate computer in the internal network of the subscriber will be enabled and will allow access to each device via a web browser (Mozilla Firefox).

This will be implemented by a VNC session to a local computer. From this computer web access to devices will be possible.

# Technology

## Exciter

The exciter first modulates the input stream (EDI) into a COFDM signal which is further transposed to the required RF output channel. The output signal is then ready for the amplification in the power stages.

### Input Signal

The exciter must have two 100Base-TX (or 1000Base-T) RJ-45 EDI inputs.

Both EDI inputs must correspond to following specifications:

- ETSI TS 102693 (EDI Packet Resend Functionality is not required) and

- ETSI TS 102821.

Used data structure must comply with ETI (LI) data stream with PFT layer as described in ETSI EN 300799.

Data RJ45 network connector must comply with the ISO 8877 specification. Via the mentioned connectors exciter must allow receiving of packets based on VLAN tags according to standard IEEE 802.1Q.

The transmitter must be switched off immediately if at least one of the following conditions occurs:

- No transport stream,

- Presence of synchronization failures,

- Discrepancies between the Time Stamp (TIST) data and the setup in the transmitter.

When the error is gone the transmitter must be switched on automatically without manual intervention.

If the functionality for automatic switchover between inputs is activated and in case the above-mentioned error appears the transmitter must switch from the input A (main signal) to the input B (backup signal). When the failure disappears the transmitter must, after an adjustable hysteresis (min. 10 min) switch back to input A. In case of failure on input B the transmitter must immediately switch to input A. A possible toggling between the two inputs must be prevented.

Switching between inputs must be performed via an interface for local and remote control.

### Pre-correction

#### Manual pre-correction

If manual pre-correction methods for the correction of the non-linear and linear distortions are used, the corresponding pre-correction tables must be saved in the device. It must be possible to save at least two such tables:

- At nominal output power

- At -3 dB of nominal output power

Pre-correction tables must be saved in the device at time of delivery. It must be possible to disable the pre-correction.

#### Adaptive pre-correction

If automatic linear and non-linear adaptive pre-correction is required, it must provide following modes of automatic optimization of the transmitter performance:

- Single,

- Continuous,

- Monitoring.

Digital adaptive precorrection improves the modulation error ratio (MER) and shoulder attenuation.

Input level signal for adaptive precorrection must be in the 0 dBm +/- 5 dB range.

### Transmission Mode

The exciter must be able to operate in the single-frequency (MFN) and multi-frequency (SFN) network.

The following settings must be possible to change:

- DAB Mode,

- Offset Delay and

- Transmitter identification parameters of the transmitter (TII).

Offset Delay and Transmitter identification parameters must be possible to change through:

- Remote – via ETI,

- Local – via Web interface.

### TII

To avoid any incompatibility with other DAB transmitters within SFN cell, exciter must allow setting identification parameters of the transmitter - Transmitter Identification Information (TII).

### Test Signal/PRBS

The exciter must be able to generate a PRBS test signal undependable from the input signal in accordance with the Technical Specification ETSI TS 300799 annex G.

## Working Frequency

The nominal center frequency is the signal middle frequency. Channel bandwidth is 1,536 MHz. The device must be adjustable in the range of 250 kHz in steps of 1 Hz.

## Synchronization

### Frequency and time reference

Table 1: Frequency and time reference

| Reference signal | Requirements |
| --- | --- |
| 10 MHz | Frequency accuracy < 1×10E-9 |
| 1 pps | Time Deviation < 250 ns |

### Stability

The center frequency must be synchronized to the internal 10 MHz reference signal. When it is not synchronized to the external reference signal the center frequency accuracy must be 10E-7 per year.

For the operation in an SFN network the transmitter must have a built-in SFN synchronization unit. The transmitter must then meet the requirements for frequency stability and timing. The transmitter must evaluate the time stamp (TIST) in the EDI stream. If the transmitter for whatever reason cannot meet the bit synchronization and the frequency synchronization, then the transmitter must be switched off immediately.

### Internal Reference

The source of internal reference is a built-in GPS receiver unit. The receiver must have additional reference output to feed additional transmitters with the reference signal. Output must have the same characteristics as valid for the external reference input.

For the GPS antenna the following connector is required: 50 Ω, BNC/SMA, asymmetric.

### External Reference

The transmitter must have an external 10 MHz reference signal input. It must be able to synchronize to this external signal.

Specifications for reference signal input interface:

- Connector: 50 Ω, BNC/SMA, asymmetric,

- Signal: sinusoidal or square,

- Level: 0,5 to 2,5 Vrms.

### Behavior in case of GPS Failure

When a GPS failure occurs the transmitter performance must not be impaired. When the GPS receiver is locked the local oscillator must be synchronized with GPS reference. The switching between locked and unlocked GPS receiver must not influence the transmitter operation.

The transmitter must be automatically switched off when the requirements for the timing of dynamic delay compensation are not met. There must be a possibility to deactivate this functionality manually or remotely.

If the transmitter operates with failed GPS receiver the transmitter must not reach the limit value sooner than in 12 hours.

The status of GPS receiver, the number of satellites and the signal level must be accessible by the control/management interface of the transmitter.

### Dynamic Network Compensation Delay

The transmitter must have an automatic dynamic compensation of a signal propagation delay which enables to compensate the delay difference of the signal path to the transmitter. This compensation must compensate for delays of up to 1 second.

A timestamp (TIST) contained in the ETI data stream serves as a reference. The present time is delivered by a GPS receiver at the transmitter.

### Static Transmitter Compensation Delay

The transmitter must have a static transmitter compensation delay which compensates the difference of the transmitter processing delay between transmitters from various manufacturers. The transmitter static delay must be adjustable via web interface in 1 us steps up to the value of 1 s.

The signal delay between the transmitter input (input transport stream) and the transmitter output because of signal processing inside the transmitter without any additional static or dynamic delay for the compensation of propagation and network optimization must be specified in the transmitter documentation.

### Transmitter Offset Delay:

For the network SFN optimization the transmitter must have settings for transmitter offset delay. This must be adjustable via:

- Remote – via ETI data stream (MNSC signaling),

- Local – via Web interface.

The transmitter offset delay must be adjustable in 1 us steps up to the value of 2000 us.

## Transmitter power

### Output power

Output power is the effective output power of a DAB+ signal that is measured at the output of a device by a thermal power measurement method.

If a transmitter with an output filter is required, the output power is measured after the output filter.

In the case where the output filter is not required and is located in the channel combiner the measurement is performed on the monitoring terminal of channel combiner.

It must be possible to reduce the output power in steps of 0,1 dB up to -6 dB in reference to nominal power. By reducing the power all quality requirements must be fulfilled. It is allowed to change the non-linear pre-correction.

### Nominal power

Nominal power is the maximum output power at which the device meets all the required technical quality criteria.

### Power consumption

The power required by the device to achieve nominal output power. This must include the power consumption of all sub-stages of the transmitter (output stage, cooling, M&C, …).

### Stability

The tolerance of output power must be less than 0,8 dB in the temperature range of +5 do +45 °C and 5% tolerance of mains voltage. This stability must be achieved at the latest 2 min. after switching it on.

### Return Loss

Output impedance of device must be 50 Ω. The device must operate up to VSWR 1,3 (18 dB). At VSWR up to 1,7 (12 dB) the output power may be reduced up to 6 dB. If the VSWR deteriorates (< 13 dB) and after VSWR status is checked 3 times, the final stage must be switched off. Short or open circuit on the output of the device must not damage the device.

### Efficiency

Efficiency is the ratio of nominal power to power consumption. Measurement report of output power and power consumption of individual sub-stages must be included in the documentation.

## Output stage

If a compact design of the transmitter is required, the PA unit and exciter must be integrated into a common housing.

If a modular design of the transmitter is required, in order to increase operational reliability, the final PA stage can contain multiple PA units.

A safe exchange of PA units during operation must be possible. During this time a reduction of output power is allowed. After replacement, the transmitter must meet all the required quality criteria.

Shutdown or failure of individual PA units may result in reduced output power, but not interruption of operation. The control system of the transmitter must recognize, log and indicate this event.

The device must be able to work continuously even if individual PA units fail. Regardless of this the cooling system must operate normally.

## Measurement/test points

The device must be equipped with test points with various signals. It must be possible to make measurements without influence on the operation of the transmitter.

### RF test points

Following test points must be provided and must not be used for transmitter internal purposes:

- On output of the exciter (in configuration above 200 W).

- On each individual PA unit if multiple PA units are used.

- Test points for forward and reflected power between output stage and channel filter (in configuration above 200 W).

- If a transmitter with an output filter is required, test points for forward and reflected power must be located at the output of channel filter.

All test points must be implemented as directional couplers according to the specification:

- Output level: -10 do +10 dBm,

- Source impedance: 50 Ω.

The test point on the device output must fulfill:

- Precision: 0,15 dB,

- Directivity: 26 dB,

### Isolation points

In the modular design there must be freely accessible separation points between individual stages of the transmitter where separation between the stages can be achieved.

# Quality of operation

## Oscillator phase noise

Phase noise of all oscillators inside the device must fulfill the following phase noise mask with and without connected external frequency reference. Measurement reports of phase noise are part of transmitter documentation:

Table 2: Oscillator phase noise

| Relative frequency | Relative level |
| --- | --- |
| 10 Hz | ≤ -55 dBc/Hz |
| 100 Hz | ≤ -85 dBc/Hz |
| 1 kHz | ≤ -85 dBc/Hz |
| 10 kHz | ≤ -95 dBc/Hz |
| 100 kHz | ≤ -112 dBc/Hz |
| 1 MHz | ≤ -130 dBc/Hz |

Picture 7: Phase noise mask

## Shoulder attenuation

Shoulder attenuation - measured on output stage before channel filter according to technical specification ETSI EN 302077 must be greater than 37 dB.

## Spectrum mask

If an output filter is required, the output signal at nominal output power must meet following requirements:

- Critical mask (Case 1) or

- Non-critical mask (Case 2).

The measurement procedure must be according to guideline and instruction.

## Spurious emissions

Emissions on frequencies outside the operating channel are unwanted and must not exceed limit values for spurious emissions in guideline 1.

## Crest factor

Voltage peaks at nominal output power must not exceed 13 dB (CCDF) above effective voltage. Crest factor must be specified for signal after output stage. The measurement duration must also be specified.

## Modulation error ratio / MER

Measurement report of a calibrated instrument of type R&S ETL is part of the device documentation.

Modulation error ratio (MER) of the transmitter must be ≥ 32 dB, measured at the output of the transmitter.

# Mains voltage

According to the device nominal power the connection to mains supply must be carried out in one or three phases. If nominal mains input voltage of 230 V / 400 V, 50 Hz vary in the range of +10% to -14%, the output power of the transmitter must not deviate by more than 10%. Power factor value: cos phi ≥ 0,95.

If a modular design of the transmitter is required, it must have separate power connections to the exciter and power amplifier(s).

## Connection to the mains supply

If a modular design of the transmitter is required, it must be equipped with a main disconnect switch and automatic circuit breakers for individual units.

## Switch-on

The warm-up time after power-on or mains failure (> 1s) must not be longer than:

- 45 sec. in the multi frequency network (MFN),

- 45 sec. in the single frequency network (SFN) with use of reference signals (1pps, 10MHz),

- 10 min. in the single frequency network (SFN) with use of GPS receiver.

Within this time, deviations of the central frequency may deviate by up to± 1*10-7.

## Uninterruptible Power Supply

### Built-in power supply

If uninterruptible power supply is required it must ensure smooth operation of exciter, GPS receiver and control unit.

Uninterruptible power supply must provide at least 20 minutes of autonomous operation.

States and events of uninterruptible power supply must be collected in the device control unit and must be accessible through the user interface.

### External power supply

If configuration with external uninterruptible power supply is required device must allow connection of an external uninterruptible power source, which separately supplies exciter, GPS receiver and control unit.

The connection point for external uninterruptible power supply must be clearly marked and connection scheme must be attached to the documentation.

# Output channel band-pass filter

If the output channel band-pass filter is required, the device must include filter with wiring cables and must be an integral part of the device. It must be adapted to the nominal power of the device and to the loads that occur during any power or voltage peaks. The filter must be tunable over the entire frequency band. A measurement log of the transmission characteristics of the operating channel in the range for the spurious, out-of band transmission and harmonic waves (VSWR, amplitude and phase frequency response, typical temperature behavior) and tuning instructions must be provided.

# Cooling system

Cooling system of DAB+ devices can be performed with liquid or air cooling.

Heat dissipation into the room of the entire device with all its components must not exceed:

Table 3: Heat dissipation

| Volume of the device | Allowed dissipation |
| --- | --- |
| 0,5-2 m3 | 1,5 kW/m3 |
| 2-5 m3 | 1 kW/m3 |
| 5-10 m3 | 0,75 kW/m3 |

## Liquid Cooling

Liquid cooling must be carried out through two pumps. Each of those pumps must be able to provide the necessary flow of coolant. Both pumps can work in change-over mode, so that one is always in reserve and full redundancy is achieved.

Piping between the cooling system and heat exchanger must be made from metal. It is allowed to use short piece of non-metallic pipe for vibration compensation. All piping must be carried out in such a way that in case of leakage water does not run on to the device and its electrical components. The cooling system and the valves must be mounted inside the building and only heat exchanger must be mounted outside. When the device is not working, coolant must not freeze until the temperature of -45 ° C.

At maximum allowed outside permissible temperature of medium must not exceed 45 °C.

## Air Cooling

If air cooling with ducting is required, cooling must be carried out through a fan, which is installed inside the device. Heat dissipation from the device into the room must be as low as possible. Air ducting must be used for air intake and outlet. It must be possible to mount the air intake/outlet at the bottom or top of the device.

If stand-alone air cooling is required, cooling must be carried out through a built-in fan, which is installed inside the device and blows hot air into the surrounding area.

# Control and monitoring

Control and monitoring of device must be provided via an interface for local and remote control.

The control unit must have a built-in Linux operating system.

All device settings must be stored in the control unit and be protected against power failure. The device must be able to store two complete configurations including pre-correction settings.

In case of a wrong command, the device must not be damaged.

## Event log

The control unit of the device must record all events/warnings/alarms in an event log (Table 4).

States 1 & 2 show the messages for each event.

Table4:.Event log

| Event Name | State 1 | State 2 | Error Description |
| --- | --- | --- | --- |
| Forward Power Low | Start | End | low forward power (adjustable threshold) |
| Forward Power Fault | Start | End | loss of forward power |
| Reflected Power Fault | Start | End | high reflected power |
| High Temperature | Start | End | over-temperature |
| No EDI | Start | End | missing EDI signal |
| GPS Fault | Start | End | GPS reception fault |
| Sync Fault | Start | End | loss of reference signal (1pps, 10MHz) |
| TIST Fault | Start | End | timestamp error |
| Modulator Fault | Start | End | modulator fault |
| Control Unit Fault | Start | End | control unit fault |
| PA Module Fault | Start | End | power amplifier module fault |
| PreA Module Fault | Start | End | preamplifier module fault |
| PS Module Fault | Start | End | power supply module fault |
| Interlock Open | Start | End | open interlock loop |
| Cooling Fault | Start | End | cooling system fault |
| Tx Fault | Start | End | transmitter fault |
| Manual/Automatic Mode | To M | To A | change-over between manual and automatic mode |
| Local/Remote Mode | To L | To R | change-over between local and remote mode |
| AC power Fault | Start | End | loss of mains input voltage |

Testing of all possible events is an integral part of the measurement protocol.

## Local interface

States stated in Table 5 must be shown on the first page of a display device of local interface.

Table 5: Local interface

States

current forward power (after output filter)

current reflected power

device temperature

EDI fault

TIST errors

interlock state

state of the carrier muting

status of the EDI data stream inputs

status of the frequency and time synchronization

local/remote mode

Alarms

low forward power (adjustable threshold)

high reflected power

cooling system fault

over-temperature

Control functions

device switch-on/switch-off

all device parameters

The local interface must allow full management of the device.

The local interface must offer simple change-over between manual and automatic mode (not more than three button presses).

## Remote interface

### Web GUI

After login and password, the main window must appear. At least parameters stated in Table 6 must be shown in the main window of web GUI.

### SNMP/MIB

At least states and values from Table 6 must be supported via the SNMP GET.

At least control commands from Table 6 must be supported via the SNMP SET.

SNMP/MIB interface must support SNMP TRAP functionality (Table 6). Traps must be sent automatically according to events described under the Event log. When there is a transition from a bad state to a good state corresponding "all clear" trap must be sent.

Table 6: Remote interface

Web GUI

forward and reflected power level

device temperature

status of EDI inputs

local/remote mode

device location

device type

transmission settings and parameters

synchronization settings and parameters

pre-correction settings and parameters (mer, shoulders)

output RF settings and parameters

PA settings and parameters (temperature, voltages, currents) * if compact design is used

external PA settings and parameters (temperature, voltages, currents) * if modular design is used

power supply parameters

log of present faults – log of all alarms and warnings

SNMP/MIB (GET)

general alarm

states described under Event log

current forward power (after output filter)

current reflected power

device temperature

SNMP/MIB (SET)

transmitter switch-on/switch-off

reset after a failure

SNMP/MIB (TRAP)

states described under Event log

all clear

ETSI EN 302077 - Electromagnetic compatibility and Radio spectrum Matters (ERM)

R&S Application Note: DAB Transmitter Measurements for Acceptance, Commissioning and Maintenance