NABAVA OPREME: LIOFILIZATOR Z DODATNO OPREMO

| Zaporedna številka | Zahtevane strokovne in tehnične lastnosti | Opis je v katalogu na strani |
| --- | --- | --- |
|  | Naprava za sušenje v zamrznjenem stanju za pripravo vzorcev za analize in za sušenje manjših količin preparatov/aktivnih snovi za pripravo funkcionalnih živil/dopolnil. |  |
|  | Kompakten, visoko zmogljiv laboratorijski liofilizator, ki zavzame malo prostora. |  |
|  | Sušilna komora neposredno nad kondenzatorjem ledu. |  |
|  | Ledena kondenzatorska komora z visoko kakovostjo tuljave iz nerjavečega jekla in notranji kondenzator. |  |
|  | Enostavno dostopen kondenzator ledu in popoln vizualni nadzor. |  |
|  | Hitro odmrzovanje z vročim plinom in čiščenje/dekontaminacija, s priloženim odtočnim ventilom. |  |
|  | Digitalno prikazana temperatura ledenega kondenzatorja s posrednim določanjem temperature vzorca. |  |
|  | Modularna zasnova z možnostjo nadgradnje. |  |
| Zaporedna številka | Paket naj vključuje LIOFILIZATOR z | Opis je v katalogu na strani |
| 1. | Maksimalna kapaciteta vsaj 2,5 kg. |  |
| 2. | Zmogljivostjo vsaj 2 kg/24 h. |  |
| 3. | Temperatura do -55 °C. |  |
| 4. | Volumen posode vsaj 3,5 l. |  |
| 5. | Dimenzije liofilizatorja: približno 345x315x460 mm (višina x dolžina x širina). |  |

Tehnične specifikacije in kriteriji za nabavo:

| Zaporedna številka | Paket naj vključuje DODATNO OPREMO | Opis je v katalogu na strani |
| --- | --- | --- |
| 1. | Vakuumska črpalka RZ-2.5, 2x10-3 mbar. |  |
| 2. | Izpušni filter za vakuumsko črpalko. |  |
| 3. | Vakuumska cev. |  |
| 4. | Vakuumski senzor. |  |
| 5. | Osnovna plošča. |  |
| 6. | 3 police Ø 200 mm. |  |
| 7. | Akrilna posoda z 6-8 konektorji. |  |
| 8. | Gumijasti ventili (6-8 kosov). |  |
| 9. | Ventil za elektromagnetni pritisk. |  |

| 1. | Dobavitelj in servisna podpora v RS. |  |
| --- | --- | --- |
| 2. | Ob predaji opreme priložen po 1 izvod navodil za uporabo in vzdrževanje, garancijski list in ostala dokumentacija. Dokumentacija je v celoti v angleškem ali slovenskem jeziku.. |  |
| 3. | Dobavni rok: najkasneje do 15. 12. 2024 |  |
| 4. | Garancija: 2 leti.** |  |
| 5. | Usposabljanje ob dobavi oz. po vzpostavitvi aparature, v fizični obliki in v slovenskem jeziku. Na osnovnem nivoju oz. za običajne aplikacije, nadalje pa dostopnost za pomoč/tehnično podporo na daljavo, v primeru potrebe. |  |
| 6. | Dodatna zahteva: dobavljivost rezervnih delov, potrošnega materiala (npr. olje za vakuumsko črpalko) vsaj še 10 let po dobavi. |  |
| 7. | Proizvajalec je dolžan predložiti izjavo o skladnosti. |  |

Druge zahteve:

** Z dnem dostave opreme v prostore naročnika oprema preide v last naročnika. O dobavi se sestavi prevzemni zapisnik. Naročnik si pridržuje pravico, da montažo in zagon ter edukacijo uporabnikov naroči po poteku določenega časovnega obdobja po dobavi opreme. Dobavitelj bo moral na pisni poziv naročnika montirati in zagnati opremo ter izvesti edukacijo na rok, določen v pozivu.

Po izvedeni montaži, zagonu ter edukaciji se sestavi zapisnika o montaži in zagonu opreme. Garancijsko obdobje prične teči naslednji dan po izdelavi zapisnika o montaži in zagonu opreme.

| Serial Number | Required Professional and Technical Features | Description in the catalog on page |
| --- | --- | --- |
| 1. | A freeze-drying device for preparing samples for analysis and for drying small quantities of preparations/active substances for the preparation of functional foods/supplements. |  |
| 2. | Compact, high-performance laboratory freeze dryer that occupies minimal space. |  |
| 3. | Drying chamber directly above the ice condenser. |  |
| 4. | Ice condensing chamber with high-quality stainless steel coil and internal condenser. |  |
| 5. | Easily accessible ice condenser and complete visual control. |  |
| 6. | Rapid defrosting with hot gas and cleaning/decontamination, with included drain valve. |  |
| 7. | Digitally displayed temperature of the ice condenser with indirect determination of sample temperature. |  |
| 8. | Modular design with upgradeability options. |  |
| Serial Number | The package should include a Freeze Dryer with | Description in the catalog on page |
| 1. | Maximum capacity of at least 2.5 kg. |  |
| 2. | Capacity of at least 2 kg/24 h. |  |
| 3. | Temperature down to -55°C. |  |
| 4. | Container volume of at least 3.5 l. |  |
| 5. | Dimensions of the freeze dryer: approximately 345x315x460 mm (height x length x width). |  |

PURCHASE OD DEVICE: A FREEZE DRYER WITH ADDITIONAL EQUIPMENT

Technical Specifications and Procurement Criteria:

| Serial Number | The package should include ADDITIONAL EQUIPMENT | Description in the catalog on page |
| --- | --- | --- |
| 1. | Vacuum pump RZ-2.5, 2x10-3 mbar. |  |
| 2. | Exhaust filter for vacuum pump. |  |
| 3. | Vacuum tube. |  |
| 4. | Vacuum sensor. |  |
| 5. | Base plate. |  |
| 6. | 3 shelves Ø 200 mm. |  |
| 7. | Acrylic container with 6-8 connectors. |  |
| 8. | Rubber valves (6-8 pieces). |  |
| 9. | Electromagnetic pressure valve. |  |

Other requirements:

| 1. | Supplier and service support in Repubic of Slovenia. |  |
| --- | --- | --- |
| 2. | User manuals, maintenance instructions, warranty, and other documentation each in 1 copy are provided upon delivery of equipment. The provided documentation is in Slovenian or English language. |  |
| 3. | Delivery deadline: no later than 15th of December 2024 |  |
| 4. | Warranty: 2 years.** |  |
| 5. | Training upon delivery or installation of the equipment, in physical form and in Slovenian language. Training on basic level or for common applications and with continued access for assistance/technical support remotely, if needed. |  |
| 6. | Additional requirement: availability of spare parts, consumables (e.g., vacuum pump oil) for at least 10 years after delivery. |  |
| 7. | The manufacturer must provide a declaration of conformity. |  |

** Upon delivery of the equipment to the customer's premises, the equipment becomes the property of the customer. A delivery report will be prepared. The customer reserves the right to schedule the installation, commissioning, and user training for a period after the delivery of the equipment. The supplier will be required to install and commission the equipment, as well as conduct the training, within the timeframe specified in the written request from the customer.

After the installation, commissioning and training, a report on the installation and commissioning of the equipment will be prepared. The warranty period will commence the day after the report on the installation and commissioning of the equipment is completed.