NABAVA NAPRAVE: SIMULATOR VISOKE ZVESTOBE Z NAPREDNEJŠIMI SAMOODZIVNIMI FUNKCIJAMI

Tehnične specifikacije in kriteriji za nabavo:

| Zaporedna številka | Zahtevane strokovne in tehnične lastnosti | Opis je v katalogu na strani |
| --- | --- | --- |
|  | Oprema predstavlja simulator visoke zvestobe, ki je anatomsko korekten in se uporablja za kompleksne scenarije pri simulacijskem usposabljanju zdravstvenih poklicev. |  |
|  | Simulator omogoča izvajanje večine intervencij, s katerimi se lahko sooča zdravstveni delavec v kliničnem okolju. |  |
|  | Gre za visoko tehnološko opremo, ki je najboljši približek realnega pacienta. |  |
|  | Simulator omogoča naprednejše samoodzivne funkcije na intervencije zdravstvenih delavcev – tehničnega vodje. |  |
|  | Simulator predstavlja celotno telo odraslega pacienta z realno višino, težo in razmerji delov telesa. |  |
|  | Simulator je kompatibilen s SimPad in LLEAP (Leardal learning application). |  |
|  | Simulator omogoča spreminjanje kože obraza, za spreminjanje videza simuliranega pacienta. |  |
|  | Simulator je brezžičen in se lahko povezuje na obstoječa brezžična računalniška omrežja. |  |
|  | Simulator ima notranje električno napajanje. |  |
|  | Omogoča izvajanje vseh temeljnih intervencij v zdravstveni negi (administracijo zdravil (npr. peroralno prek cevi NG, IV, IM), izvedbo osebne higiene, oskrba ran, rokovanje s pacientom, urinsko kateterizacijo, oksigenacijo in perfuzijo, jemanje venske krvi, vstavljanje perifernih venskih katetrov, oskrbo dihalne poti, reanimacijo, auskultacijo dihanja, srčnih tonov in peristaltike, merjenje perifernega tlaka, tipanje pulzov na vseh konvencionalnih mestih, ...). |  |
|  | Simulator ima popolnoma gibljive sklepe, sedi brez dodatne podpore in je anatomsko korekten. |  |
|  | Omogoča brezžično krmiljenje vitalnih funkcij (npr. dihanje, srčni ritem in pulz, tlak, frekvenca mežikanja, komunikacija, ...). |  |
|  | Omogoča predvajanje vnaprej posnetih zvokov (npr. kašljanje, bruhanje, kričanje, ...) in živega govora. |  |
|  | Omogoča predvajanje prednaloženih scenarijev. |  |
|  | Simulator je čvrsto izdelan in je namenjen za delovanje v različnih vrstah okolij. |  |
|  | Simulator omogoča zamenjavo obrazne kože z drugačnimi moduli. |  |
|  | Simulator je kompatibilen s simulatorjem ventilatorja ASL 5000. |  |
|  | Oprema omogoča podporo na daljavo, zbiranje podatkov in napovedovanje vzdrževanja. |  |
|  | Oprema se samodejno odzove na več kot 150 farmakoloških in medicinskih pripomočkih z vgrajeno tehnologijo RFID. |  |

Druge zahteve:

| 1. | Ponudba vključuje enkratno usposabljanje osebja za uporabo simulatorja, v slovenskem ali angleškem jeziku. |  |
| --- | --- | --- |
| 2. | Zagotavljanje rezervnih delov, nadgradnje programske opreme, servisa za obdobje najmanj 10 let. |  |
| 3. | Oprema je v garanciji vsaj 1 leto od začetka uporabe. ** |  |
| 4. | Dobavni rok: najkasneje do 15. 12. 2024 |  |
| 5. | Ob predaji opreme priložen po 1 izvod navodil za uporabo in vzdrževanje, garancijski list in ostala dokumentacija. Dokumentacija je v slovenskem ali angleškem jeziku |  |

** Z dnem dostave opreme v prostore naročnika oprema preide v last naročnika. O dobavi se sestavi prevzemni zapisnik. Naročnik si pridržuje pravico, da montažo in zagon ter edukacijo uporabnikov naroči po poteku določenega časovnega obdobja po dobavi opreme. Dobavitelj bo moral na pisni poziv naročnika montirati in zagnati opremo ter izvesti edukacijo na rok, določen v pozivu.

Po izvedeni montaži, zagonu ter edukaciji se sestavi zapisnika o montaži in zagonu opreme. Garancijsko obdobje prične teči naslednji dan po izdelavi zapisnika o montaži in zagonu opreme.

PURCHASE OF DEVICE: HIGH-FIDELITY SIMULATOR WITH ADVANCED SELF-RESPONSIVE

Technical specifications and procurement criteria:

| Serial Number | Required Professional and Technical Features for HIGH-FIDELITY SIMULATOR WITH ADVANCED SELF-RESPONSIVE FUNCTIONS | Description in the catalog on page |
| --- | --- | --- |
|  | The equipment is an anatomically correct, high-fidelity simulator used for complex scenarios in simulation training for the health professions. |  |
|  | The simulator allows the implementation of most of the interventions that a healthcare professional may face in a clinical setting. |  |
|  | It is a high-tech piece of equipment that is the best approximation of a real patient. |  |
|  | The simulator allows for more advanced self-reactive functions to the interventions of the health professional - the technical manager. |  |
|  | The simulator represents the entire body of an adult patient with realistic height, weight, and body proportions. |  |
|  | The simulator is compatible with SimPad and LLEAP (Laerdal Learning Application). |  |
|  | The simulator allows changing of the facial skin for altering the appearance of the simulated patient. |  |
|  | The simulator is wireless and can connect to existing wireless computer networks. |  |
|  | The simulator has internal electrical power supply. |  |
|  | Enables performing all fundamental nursing interventions (administration of medications (e.g., orally via NG tube, IV, IM), personal hygiene, wound care, patient handling, urinary catheterization, oxygenation and perfusion, venous blood sampling, insertion of peripheral venous catheters, airway management, resuscitation, auscultation of breathing, heart sounds, and peristalsis, measuring peripheral blood pressure, palpating pulses at all conventional sites, etc.). |  |
|  | The simulator has fully movable joints, sits upright without additional support, and is anatomically correct. |  |
|  | Allows wireless control of vital functions (e.g., breathing, heart rate and pulse, pressure, blink frequency, communication, etc.). |  |
|  | Allows playback of pre-recorded sounds (e.g., coughing, vomiting, screaming, ...) and live speech. |  |
|  | Allows playback of pre-loaded scenarios. |  |
|  | The simulator is robustly constructed and designed for operation in various types of environments. |  |
|  | The simulator allows replacement of facial skin with different modules. |  |
|  | The simulator is compatible with the ASL 5000 ventilator simulator. |  |
|  | The equipment enables remote support, data collection, and maintenance forecasting. |  |
|  | The equipment automatically responds to over 150 pharmacological and medical devices with built-in RFID technology. |  |

Other requirements:

| 1. | The offer includes one-time staff training for using the simulator, in English or Slovenian. |  |
| --- | --- | --- |
| 2. | Ensures the supply of spare parts, software upgrades, and service for a period of at least 10 years. |  |
| 3. | The equipment is under warranty for at least 1 year from the start of use. ** |  |
| 4. | Delivery deadline: no later than 15th of December 2024 |  |
| 5. | User manuals, maintenance instructions, warranty, and other documentation are provided upon delivery of equipment, each in one copy and in Slovenian or English. |  |

** Upon delivery of the equipment to the customer's premises, the equipment becomes the property of the customer. A delivery report will be prepared. The customer reserves the right to schedule the installation, commissioning, and user training for a period after the delivery of the equipment. The supplier will be required to install and commission the equipment, as well as conduct the training, within the timeframe specified in the written request from the customer.

After the installation, commissioning and training, a report on the installation and commissioning of the equipment will be prepared. The warranty period will commence the day after the report on the installation and commissioning of the equipment is completed.

NABAVA NAPRAVE: SIMULATOR VISOKE ZVESTOBE

Tehnične specifikacije in kriteriji za nabavo:

| Zaporedna številka | Zahtevane strokovne in tehnične lastnosti | Opis je v katalogu na strani |
| --- | --- | --- |
|  | Oprema predstavlja simulator visoke zvestobe, ki je anatomsko korekten in se uporablja za kompleksne scenarije pri simulacijskem usposabljanju zdravstvenih poklicev. |  |
|  | Simulator omogoča izvajanje večine intervencij, s katerimi se lahko sooča zdravstveni delavec v kliničnem okolju. |  |
|  | Gre za visoko tehnološko opremo, ki je najboljši približek realnega pacienta. |  |
|  | Simulator je kompatibilen s SimPad in LLEAP (Leardal learning application). |  |
|  | Na simulatorju je mogoče spreminjanje modulov moški, ženska, starostnik. |  |
|  | Simulator je brezžičen in se lahko povezuje na obstoječa brezžična računalniška omrežja. |  |
|  | Simulator ima notranje električno napajanje. |  |
|  | Omogoča izvajanje vseh temeljnih intervencij v zdravstveni negi (administracijo zdravil (npr. peroralno prek cevi NG, IV, IM), izvedbo osebne higiene, oskrba ran, rokovanje s pacientom, urinsko kateterizacijo, oksigenacijo in perfuzijo, jemanje venske krvi, vstavljanje perifernih venskih katetrov, oskrbo dihalne poti, reanimacijo, auskultacijo dihanja, srčnih tonov in peristaltike, merjenje perifernega tlaka, tipanje pulzov na vseh konvencionalnih mestih, ...). |  |
|  | Simulator ima popolnoma gibljive sklepe, sedi brez dodatne podpore in je anatomsko korekten. |  |
|  | Omogoča krmiljenje vitalnih funkcij (npr. dihanje, srčni ritem in pulz, tlak, frekvenca mežikanja, komunikacija, ...). |  |
|  | Omogoča predvajanje vnaprej posnetih zvokov (npr. kašljanje, bruhanje, kričanje, ...) in živega govora. |  |
|  | Omogoča predvajanje prednaloženih scenarijev. |  |
|  | Ima notranji rezervoar s prostornino vsaj 450 ml tekočine z možnostjo uporabe dodatnega zunanjega rezervoarja. |  |

Druge zahteve:

| 1. | Ponudba vključuje enkratno osnovno in celovito usposabljanje osebja za uporabo simulatorja, ki bo potekalo v slovenskem ali angleškem jeziku na mestu uporabe simulatorja. |  |
| --- | --- | --- |
| 2. | Zagotavljanje možnosti dobave rezervnih delov, nadgradnje programske opreme, dostopnosti servisa za obdobje najmanj 10 let. |  |
| 3. | Ob predaji opreme priložen po 1 izvod navodil za uporabo in vzdrževanje, garancijski list in ostala dokumentacija, v slovenskem ali angleškem jeziku. |  |
| 4. | Dobavni rok: najkasneje do 15. 12. 2024 |  |
| 5. | Oprema je v garanciji vsaj 1 leto od začetka uporabe. ** |  |

** Z dnem dostave opreme v prostore naročnika oprema preide v last naročnika. O dobavi se sestavi prevzemni zapisnik. Naročnik si pridržuje pravico, da montažo in zagon ter edukacijo uporabnikov naroči po poteku določenega časovnega obdobja po dobavi opreme. Dobavitelj bo moral na pisni poziv naročnika montirati in zagnati opremo ter izvesti edukacijo na rok, določen v pozivu.

Po izvedeni montaži, zagonu ter edukaciji se sestavi zapisnika o montaži in zagonu opreme. Garancijsko obdobje prične teči naslednji dan po izdelavi zapisnika o montaži in zagonu opreme.

PURCHASE OF DEVICE: HIGH-FIDELITY SIMULATOR

Technical specifications and procurement criteria:

| Serial Number | Required Professional and Technical Features for High-Fidelity Simulator | Description in the catalog on page |
| --- | --- | --- |
|  | The equipment is a high-fidelity, anatomically correct simulator used for complex scenarios in simulation training for the health professions. |  |
|  | The simulator allows the implementation of most of the interventions that a healthcare professional may face in a clinical setting. |  |
|  | It is a high-tech piece of equipment that is the best approximation of a real patient. |  |
|  | The simulator is compatible with SimPad and LLEAP (Laerdal Learning Application). |  |
|  | It is possible to change modules for male, female, and elderly on the simulator. |  |
|  | The simulator is wireless and can connect to existing wireless computer networks. |  |
|  | The simulator has internal electrical power supply. |  |
|  | Enables performing all fundamental nursing interventions (administration of medications (e.g., orally via NG tube, IV, IM), personal hygiene, wound care, patient handling, urinary catheterization, oxygenation and perfusion, venous blood sampling, insertion of peripheral venous catheters, airway management, resuscitation, auscultation of breathing, heart sounds, and peristalsis, measuring peripheral blood pressure, palpating pulses at all conventional sites, etc.). |  |
|  | The simulator has fully movable joints, sits without additional support, and is anatomically correct. |  |
|  | Allows control of vital functions (e.g., breathing, heart rate and pulse, pressure, blink frequency, communication, etc.). |  |
|  | Allows playback of pre-recorded sounds (e.g., coughing, vomiting, screaming, ...) and live speech. |  |
|  | Allows playback of pre-loaded scenarios. |  |
|  | Has an internal reservoir with a volume of at least 450 ml of liquid with the option to use an additional external reservoir. |  |

Other requirements:

| 1. | The offer includes one -time basic and comprehensive staff training for using the simulator, which will take place in Slovenian or English language at the location of simulator use. |  |
| --- | --- | --- |
| 2. | Ensures the possibility of supplying spare parts, software upgrades, service availability for a period of at least 10 years. |  |
| 3. | User manuals, maintenance instructions, warranty, and other documentation are provided upon delivery of equipment, each in 1 copy and in Slovenian or English. |  |
| 4. | Delivery deadline: no later than 15th of December 2024 |  |
| 5. | The equipment is under warranty for at least 1 year from the start of use. ** |  |

** Upon delivery of the equipment to the customer's premises, the equipment becomes the property of the customer. A delivery report will be prepared. The customer reserves the right to schedule the installation, commissioning, and user training for a period after the delivery of the equipment. The supplier will be required to install and commission the equipment, as well as conduct the training, within the timeframe specified in the written request from the customer.

After the installation, commissioning and training, a report on the installation and commissioning of the equipment will be prepared. The warranty period will commence the day after the report on the installation and commissioning of the equipment is completed.