Public procurement documentation

INTRODUCTION OF THE MODERN PREDICTIVE DIAGNOSTIC CENTER BASED ON THE ADVANCED PREDICTIVE MAINTENANCE ANALYTICS TECHNOLOGIES

TECHNICAL REQUIREMENTS

table of contents

1.	Introduction	4

2.	ZVAPS System Architecture	4

3.	Main Project Objectives	7

4.	General Activities for the Implementation of the DC System	8

4.1	Activities in the Contracting Authority’s (DEM) scope of work are the following:	8

4.2	Activities in the Service Provider’s scope of work:	9

5.	Detailed Activities for the Implementation of the DC System	10

6.	Technical Requirements for the DC System Software Tools	13

7.	General IT Requirements and cybersecurity	18

8.	Deliveries and Services	20

9.	Software Delivery	21

10.	Hardware Delivery	21

11.	Testing of the Installed DC System Software	22

12.	Software Maintenance and Technical support	22

13.	Training of the Contracting Authority’s Personnel	23

14.	Project Duration	23

15.	Warranty Period	23

16.	Literature	25

Abbreviations

AI	Artificial Intelligence

APA		Advanced Predictive Analytics

APMA		Advanced Predictive Maintenance Analytics

API		Application Programing Interface

APM 		Asset Performance Management

DC		Diagnostic Center

DDE		Dashboard Design Environment

DGA		Dissolved Gas Analysis

DEM		Drava Power Plants Maribor

DL		Deep Learning

ERP 		Enterprise Resource Planning

FMEA 		Failure Mode and Effect Analysis

HPP 		Hydropower Plant

IDE		Integrated Development Environment

ISA95		International Standard for the Integration of Enterprise and Control Systems

ML		Machine Learning

OPC 		Open Platform Communications

OSA-CBM		Open System Architecture for Condition-Based Maintenance

CBM 		Condition-Based Maintenance

PMA 		Predictive Maintenance Analytics

PdM 		Predictive Maintenance

RCC		Remote Control Center

RCM 		Reliability Centered Maintenance

RDC		Remote Diagnostic Center

RPD		Remote Predictive Diagnostics

SCADA		Supervisory Control and Data acquisition

SAP PM		SAP Plant Maintenance

TA		Trend Analysis

VPN		Virtual Private Network

ZVAPS 		OPC Technology-Based Historian system at DEM premises

Introduction

DEM (Drava Power Plants Maribor) is the largest producer of electricity from renewable sources in Slovenia, generating nearly a quarter of the country's total electricity. At DEM, we recognize the critical importance of continuously updating modern digital technologies, both at the technical and business process levels.

In 2014, DEM introduced the OPC technology-based system ZVAPS to address the need for improved and simplified collection, archiving, and representation of process data for in-depth analysis of hydropower plant (HPP) operations. The ZVAPS system was designed to aggregate process data from various sources and present it in a modern, web-accessible environment. Utilizing the Proficy Historian database (GE Digital), the system collects data from different process/technical systems and associated databases across eight HPPs along the Drava River. Currently, approximately 10,000 measurements are stored in the Historian database, offering significant potential for detailed analysis of HPP operations.

The pre-project phase of ZVAPS focused on the initial concepts of HPP condition-based maintenance (CBM), heavily relying on the ISA-95 [1] and OSA-CBM [2] standards, as well as modern digital technologies such as OPC and machine learning [3] – [5]. Although the ZVAPS system was intended to extend into the CBM field, practical experience revealed that it was inadequate for implementing CBM and predictive maintenance (PdM) principles. The primary limitations were related to the system's inability to provide and organize data suitably for further processing with modern data science and artificial intelligence (AI) techniques, which are now central to Advanced Predictive Maintenance Analytics (APMA). These solutions predominantly depend on multivariate time-series forecasting models developed in machine and deep learning environments.

To overcome these limitations, the ZVAPS system requires replacement with modern digital solutions that integrate APMA technologies, serving as a technical platform for the Diagnostic center (DC). The core objective of this public procurement is to introduce APMA-based technologies to enable the DC's operation.

The following chapters detail the technical requirements for the APMA hardware and software solutions intended for the DC's operation. The latter will be built on state-of-the-art digital technologies, designed to detect critical events, and optimize the maintenance process. The introduction of such technologies must result in significant maintenance cost cuts, increased equipment availability and improved reliability.

ZVAPS System Architecture

The ZVAPS system refers to the system based on the OPC technology that is used to collect, validate, analyse, present, and centrally archive process data obtained from eight power plants belonging to the DEM company. The system is built on the Proficy Historian database (GE Digital) and accompanying tools: iFIX – the web-based SCADA, Operations Hub – the operational intelligence software and Proficy Historian Analysis – the trend analysis tool. The introduced OPC system collects and stores around 10.000 measurements in the Historian database. These data are partially obtained from real-time sources via the OPC DA protocol, specifically from the HPPs process control and other technical systems. The remaining amount of data includes around six thousand measurements obtained from different on-site measuring equipment (e.g., measurements related to state of batteries, insulation resistance of windings, etc.) and imported manually.

Fig. 1 shows the ZVAPS system’s main components and interconnected systems: the HPPs control systems, the OPC-based Historian database, the MATLAB/SIMULINK mathematical environment, the Python programming environment, the SAP ERP system, and the remote-control center (RCC).

Fig. 1: System based on the OPC technology (i.e., ZVAPS): the HPPs control systems, the OPC-based Historian database, the MATLAB/SIMULINK mathematical environment, the Python programming environment, the SAP ERP system, and the RCC.

The MATLAB/SIMULINK mathematical environment retrieves data from the Historian database using OPC HDA (historical OPC data) and OLE-DB communication protocols. This environment provides an appealing option for accessing OPC data during system simulations, allowing thus many progressive functionalities such as system identification based on the OPC data.

The ZVAPS system collects data from the following process/technical systems and corresponding databases – see Fig. 1:

The SIEMENS PCS7 system installed on the following DEM HPPs:

- HPP Vuhred,

HPP Ožbalt,

- HPP Zlatoličje and Melje dam,

- HPP Fala,

- HPP Mariborski otok,

- HPP Vuzenica,

- HPP Dravograd.

The hybrid process system based on the ABB ADVANT AC110 and AC70 controllers and the VISPRO SCADA (Visual Systems Automation GmbH), installed at the following facilities:

HPP Formin,

Markovci dam.

NOTE 2.1: The aforementioned facilities will undergo refurbishment over the next three years. Consequently, the specified hybrid system will be replaced with the Siemens PCS7 system. Therefore, the integration with the current process system is beyond the scope of the DC project and will be implemented following the completion of the HPP and dam refurbishment.

The construction monitoring system (database) based on the Proficy Historian and iFIX web-based SCADA (GE Digital) technology – the system’s Proficy Historian database can be accessed via OPC and ODBC protocols.

The DEM Remote Control Center (RCC), based on the Hitachi Network Manager EMS – the system’s PostgreSQL database can be accessed via OPC UA protocol.

The Bruel & Kjaer Vibro vibration monitoring system – the system’s data values can be read via OPC UA protocol.

MATLAB/SIMULINK mathematical environment – this environment retrieves data from the ZVAPS system database using the OPC HDA protocol and SQL-based queries (scripts).

NOTE 2.2: Where possible, the Contracting Authority must replace the current OPC DA and OPC HDA protocols with the newer OPC UA version.

The following are the main servers of the ZVAPS system, based on the GE Digital software solutions:

HIST_SERV_ZVAPS – the main server, which runs the Proficy Historian database.

WEB_SERV_ZVAPS – the server which runs the Proficy iFIX Web Space web-based SCADA.

OPHUB_SERV_ZVAPS – this server runs the Proficy Operations Hub tool, which is used to visualize dashboards for real-time situational awareness and interactive reporting.

DREAMREPORT_SERV_ZVAPS – this server runs the Dream report application, which is used to create web-based reports based on data gathered in the Historian database.

Main Project Objectives

The main project objective is to build the Advanced Predictive Maintenance Analytics-based (APMA) system, with the aim to support the operation of the Diagnostic Center (DC). The following are the typical characteristics of the APMA-based system (hereinafter the DC system):

- The DC system must be part of the Advanced Predictive Maintenance Analytics (APMA) software and thus advertised by the Service Provider. The APMA software solutions are based on the state-of-the-art artificial intelligence technologies [6], [7], such as machine and deep learning or similar. These solutions are also associated with Asset Performance Management (APM) [8] systems/technologies, either as components of these systems/technologies or marketed alongside them.

NOTE 3.1: The Contracting Authority’s position on using the aforementioned APMA-based systems aligns with the requirements specified in other public procurements for established technologies of similar complexity, such as SCADA, Historian Systems, and Energy Management Systems. The term APMA may be substituted with similar terms like Advanced Predictive Analytics (APA), Predictive Analytics (PA), Advanced Predictive Maintenance (APM), or Predictive Maintenance (PdM). The Contracting Authority will accept all systems/technologies under these terms, provided they meet the definitions and requirements for the DC system outlined in this document.

The APMA functionality will be applied to the hydropower plants (HPPs) devices with the goal to increase availability and reduce maintenance costs – i.e., the devices and systems belonging to the generators, turbines and transformers.

The DC system must support hierarchical model of device status variables, i.e., the ISA-95 equipment hierarchy model, which must be aligned with the current hierarchical structure of the SAP PM (Plant Maintenance) system. The latter is being used to manage all maintenance activities in the DEM company.

The DC system must be established as a Historian-based system, specifically designed to store time-series values of device status variables.

The DC system must be founded on data-oriented models capable of predicting future events, outages, and malfunctions of individual HPP devices or groups of devices before they occur. These models should leverage state-of-the-art artificial intelligence techniques, such as machine learning and deep learning, or similar approaches.

The DC system must enable users to adequately report, and document predicted device outages and malfunctions. This means that upon detecting each individual malfunction, the DC system web-based tool will allow specialists to detail the malfunction and assign its resolution to the corresponding DEM department. The tool must track all events related to resolution of each individual malfunction, ensuring they are properly classified within the equipment hierarchical structure. In other words, all the past events related to error and failure resolutions, along with the corresponding experiences, must be stored in a knowledge base.

The DC system must allow sharing of expertise technical comments and observations between the DC specialists and HPPs maintenance staff.

The system must be RCM ready – i.e., ready for upgrade with the RCM (Reliability Centered Maintenance) functionality.

Production portfolio expansion - The system must be ready to be expand on additional production units. In the following phases, the system will be applied on additional hydropower plants and also solar and wind power plants.

NOTE 3.2: The costs of a newly included units can only increase proportionally to the general rise in prices, if generally the functionality of the DC system remains the same.

General Activities for the Implementation of the DC System

This chapter describes in detail the general activities in the Contracting Authority’s and Service Provider’s scope of work that must be carried out to implement the DC system successfully. The detailed requirements related to characteristics of the DC system software tools and services are specified in the following chapters 5 and 6.

Activities in the Contracting Authority’s (DEM) scope of work are the following:

- Analysis of the existing HPP technical systems and identification of devices that need to be integrated into the DC system.

Preparation of data point lists (measurements) for the DC system and corresponding APMA-based predictive models. These data lists should include all available measurement data points gathered from the systems described in chapter 2 of this document. Additionally, the process lists must incorporate financial indicators (e.g., purchase value, current value) related to individual devices, available in the SAP ERP system.

Introduction and activation of the measurements data points in the HPPs process systems – i.e., in the process controllers and SCADA systems.

Preparation of the detailed functional specifications for the DC system software tools:

- Determination of the hierarchical asset structure, which defines the relationships between the highest level of equipment and subordinate components. The asset structure must follow the ISA-95 standard and align with the existing hierarchical asset structure of the SAP PM system.

- Specification of constants (e.g., device lifetime, working hours, on/off states) and time-series (e.g., trends) variables, which will be used to determine the state of a particular device or group of devices. This functionality is a part of the CBM techniques, and it is not covered by the DC system advanced predictive models – see chapter 5, Creation of the dashboards for the Mechanical Department.

- Determination of threshold values for triggering maintenance actions according to the values of the constants and time-series variables described in the previous point B. This also involves determination of the corresponding alarm messages and dashboards to visualize adequately the process related to the triggered action.

- Dashboard design – determining appearance of the DC system dashboards intended for monitoring the equipment state, providing real-time situational awareness, interactive reports, and analytics.

NOTE 4.1: The exact determination of tasks defined in points from A to D is not entirely within the Contracting Authority’s responsibility and scope of work. The Contracting Authority will determine dashboards intended for displaying real-time (SCADA) data. Dashboards developed by the Service Provider to specifically support APMA functionality must be defined by the Service Provider. However, the Service Provider's responsibility also extends to dashboards determined by the Contracting Authority. This means that if a health condition parameter is calculated or determined by the APMA predictive tools (e.g., for a pump device displayed on a dashboard determined by the Contracting Authority), the Service Provider must determine the placement of the health condition parameter on that dashboard.

The same applies to all other functionalities within the Service Provider’s scope of work when the APMA (or other specific DC system functionality) affects tasks determined in the Contracting Authority’s scope of work.

Procurement and preparation of the DC system virtual servers and installation of the corresponding operating systems – see chapter 9 and 10, Software and Hardware delivery.

Preparation, procurement, and installation of the DC system network infrastructure, as defined by the Service Provider – see chapter 10, Hardware delivery.

Coordination and cooperation in the installation process of the DC system software tools.

Factory Acceptance Tests (FAT).

Site Acceptance Tests (SAT).

Activities in the Service Provider’s scope of work:

- Adding the measurement data points and alarms in the communication servers (e.g., OPC DA/UA or other) of the process and other technical systems described in chapter 2.

Implementation and configuration of the communication mechanisms between the HPPs process and technical systems and software tools and components of the DC system – see chapter 2.

Installation and configuration of the DC system software tools and corresponding components.

Installation and configuration of the APMA-based models (e.g., template, blueprints) intended to predict the state of the HPPs devices.

Implementation of all the specified activities for implementation of the DC system and technical requirements described in this document.

Training of the DEM personnel – see chapter 13, Training of the Contracting Authority's personnel.

Factory Acceptance Tests (FAT).

Site Acceptance Tests (SAT).

Detailed Activities for the Implementation of the DC System

The purpose of this chapter is to describe the activities within the Service Provider’s scope of work in sufficient detail to ensure a thorough understanding of the Contracting Authority’s requirements for building the DC system. The requirements related to the characteristics of the DC system software tools are specified separately in chapter 6 “Technical Requirements for the DC System Software Tools”.

Fig. 2: The DEM hydropower plants on the Drava River.

The following are the main assets that must be covered with the DC system solution:

Turbines (20 turbines)

Generators (20 generators),

Transformers (19 transformers),

The detailed activities in the Service provider’s scope of work are as follows:

- Creation of the main dashboard (i.e., process picture) that schematically shows the Drava River and the overall diagnostic status of the main assemblies, such as turbines, generators, and transformers (see Fig. 2 above and Table 1 below). The overall diagnostic status will be presented using a semaphore gadget indicating different severity levels (red, yellow, and green). By clicking on a selected assembly in the main dashboard, users will be redirected to the DC system predictive analytics tool, which displays all relevant predictive maintenance parameters related to the selected assembly.

- The following list shows the DEM HPPs that must be covered with the DC system solution and included in the main dashboard:

| Hydropower plant | Units | Annual Production (GWh) | Power (MW) |
| --- | --- | --- | --- |
| Dravograd | 3 | 142 | 26,2 |
| Vuzenica | 3 | 247 | 55,6 |
| Vuhred | 3 | 297 | 72,3 |
| Ožbalt | 3 | 305 | 73,2 |
| Fala | 3 | 260 | 58 |
| Mariborski Otok | 3 | 270 | 60 |
| Zlatoličje | 2 | 577 | 136 |
| Formin (SEE NOTEs 2.1 and 5.1) | 2 | 548 | 116 |

Table 1: The list of DEM HPPs and corresponding production data.

NOTE 5.1: The HPP Formin will undergo refurbishment over the next three years. Therefore, the integration with the current process system is beyond the scope of the DC project and will be implemented following the completion of the HPP and corresponding dam refurbishment.

Creation of the transformers’ predictive models – Currently, the 20 transformers at DEM lack real-time Dissolved Gas Analysis (DGA) data acquisition systems for fault detection. However, process measurements provide oil and winding temperature data. The articles [9] – [11] demonstrate that oil temperature data can effectively predict a transformer's operational state. Consequently, the Service Provider must utilize its own DC system models to forecast and alarm the transformer's oil temperature, in this case without the obligation to identify specific fault types.

Creation of the dashboards for the Mechanical Department – The Service Provider must create dashboards that display the health status of the mechanical equipment, e.g., pumps, compressors, etc. Around 110 data points (measurements) will be collected via OPC protocol from each HPP unit (see the units in Table 1) and displayed on dashboards using various graphical symbols, e.g., trends, tables, gauges, XY plots, different chart types, etc.

Creation of the dashboards for the Construction Department – These dashboards will contain approximately 10 data points for each HPP unit, such as temperatures, water levels, and conductivities. Additionally, around 250 data points will be displayed on dashboards from sensors situated in areas between HPP locations. The data points will be collected from the construction monitoring system, which is based on the Proficy Historian database (GE Digital), and displayed using various graphical symbols, including trends, tables, gauges, XY plots, and different chart types.

Connection to the DEM Remote Control Center (RCC) (see chapter 2), utilizing the Hitachi Network Manager EMS. The Service Provider must establish a connection from the DC system to the RCC OPC UA server, and this connectivity must be verified by reading a few data points present in the RCC.

Creation of the dashboards for the Environment and Spatial Planning Department – To achieve DEM's sustainability objectives and to preserve the storage capacity for hydropower energy potential on the river Drava, it is important to monitor a variety of data reflecting sustainable environmental, social, and economic aspects of the activities. In line with the EU CSRD Directive adopted in 2023, the focus on monitoring data to identify risks, gaps and opportunities for the company is further increased. The Environment and Spatial Planning Department gathers and evaluates data for decision making and predictive modelling in the field of waste management (municipal, construction and hazardous waste volumes and their management), wastewater data, air emissions, noise emissions, energy efficiency of buildings, water and sediment quality, turbidity, suspended solids concentrations in the Drava River reservoirs and many others. The substantial amount of this data, trends, and the effectiveness of the implemented measures, would be easier to monitor through graphical displays (graph). The Environment and Spatial Planning Department is currently gathering and manually entering approximately 80 data points related to the aforementioned monitoring data into a dedicated Excel file, which serves as a database. The Service Provider is required to develop a tool for transferring data from the Excel database to the historical database of the DC system. These data points will be displayed on approximately 8 dashboards, each featuring around 10 data points, and will be represented using various graphical symbols, such as trends, tables, gauges, XY plots, and different chart types.

Fig. 3: The conceptual design of the Diagnostic center system.

- The overall number of process data points should be somewhere between 6000 to 8000.

NOTE 5.2: This chapter describes in more detail the activities in the Contracting Authority’s and Service Provider’s scope of work. However, it must be considered that the final and the most accurate description of all the necessary activities can and will be provided through functional specifications in the initial project phase. In this sense, the Service Provider must consider the possibility that the herein defined scope of work may be up to 10 percent higher or lower, which means a corresponding increase or decrease in the number of dashboards, data-points, and corresponding services.

The DC system must comply with all the requirements specified in this document. The Service Provider must provide all necessary proof that their system meets these requirements, including comprehensive technical documentation, guidelines, manuals, and other relevant materials.

Technical Requirements for the DC System Software Tools

The following are the specific technical requirements for the APMA-based system software tools and corresponding components (hereinafter the DC system):

- The DC system software must be capable of predicting equipment faults and malfunctions using state-of-the-art artificial intelligence tools, such as machine learning and deep learning [6], [7], etc. These tools form the core functionality of the APMA systems [6] – [8], often referred to as time-to-failure forecasting systems, time-to-action forecasting systems, or similar. These forecasting systems must be based on pre-prepared AI models (e.g., templates, blueprints) designed to identify different component/device failure modes using process data (measurements) included in the model structure. Based on the identified failure mode, the system must estimate the remaining time until the machine reaches its integrity limit, i.e., the machine's useful life (RUL). The AI-based model functionality must already be integrated into the Service Provider’s APMA system (DC system) and thus advertised by the Service Provider. The Contracting Authority expects the AI-based models provided by the Service Provider to comprehensively address the primary unit assemblies, specifically turbines, generators, and transformers. For turbines and generators, this includes mechanical failures (e.g., bearings), electrical failures (e.g., stator, excitation, cooling), oil failures, control mechanism failures, and other related issues. The Service Provider must provide detailed functionality descriptions of the AI-based model, including the necessary variables required to build the models.

NOTE 6.1: It must be emphasized that the required APMA models are not solely used for detecting and diagnosing subtle deviations (e.g., as digital twins) from real-time values of certain device variables. They must also include functionality for determining the failure modes of devices/components and estimating their remaining useful life (RUL).

NOTE 6.2: The Contracting Authority’s position on relying on the above described and advertised APMA-based systems [6] – [8] and the corresponding AI-based technologies is analogue to the requirements specified in other public procurements for already established technologies of similar complexity, e.g., SCADA, Historian Systems, Energy Management Systems, and similar – i.e., because of its complexity the APMA technology must be already established and thus advertised by the Service Provider. The term APMA is often substituted with other similar terms, e.g. Advanced Predictive Analytics (APA), Predictive Analytics (PA), Advanced Predictive Maintenance (APM) or simply Predictive Maintenance (PdM). The Contracting Authority will accept all the systems/technologies behind these terms as long as they are in accordance with the above concepts and with requirements for the DC system given in this document.

The system for prediction of the equipment faults and malfunctions must be user friendly and simple to use and must allow users of the DC system to create their own AI-based models. Neither the administrators and users of this system do not require in-depth knowledge of machine and deep learning or similar techniques. This means that all the user interfaces of the faults and malfunction prediction system must be built in such way that will not require the administrators and users to write program code, as it is done in the typical machine and deep learning frameworks – e.g., the Python based frameworks Keras, Tensorflow or similar. Instead, the data preprocessing, training, modelling and tunning of the AI-based models must be completely performed within the user interfaces of the faults and malfunction prediction system (i.e., DC system), and the coding must be kept to a minimum. This must be achieved through simplified interface that does not require users to know in depth details behind the neural network architecture. Furthermore, the user interfaces must also show comparison between the actual measured data and results of the AI-based models. The information about the performance and accuracy of the trained AI-based models must also be available.

The DC system must enable users to adequately report, and document predicted device outages and malfunctions. This means that upon detecting each individual malfunction, the DC system web-based tool will allow specialists to detail the malfunction and assign its resolution to the corresponding DEM department. The tool must track all events related to resolution of each individual malfunction, ensuring they are properly classified within the equipment hierarchical structure. In other words, all the past events related to error and failure resolutions, along with the corresponding experiences, must be stored in a knowledge base.

The DC system must be ready to include Reliability Centered Maintenance (RCM) functionality. The RCM allows the deployment of maintenance strategies for individual assets or a group of assets. Maintenance strategies are optimized in such a way that productivity is maintained using the most cost-effective maintenance techniques. The RCM must optimize maintenance strategies based on costs and risks and, on this basis, determine the assets that belong to time-based maintenance and the assets that belong to predictive maintenance.

NOTE 6.3: The RCM is out of scope in this project; however, it must be noted that the Contracting Authority intends to introduce the DC system that is already part of the wider APM solution and thus can be easily upgraded to include RCM.

The DC system must be founded as the historian-based system, which is referred to in certain sources as an operational historian. It should be emphasized that the operational historian differs from the enterprise historian, since it is designed purposely for industrial automation systems. Properties of process historians are described in [12] and [13].

NOTE 6.4: Currently, the Proficy Historian system (GE Digital) is installed at DEM premises. The Service Provider shall introduce their own historian system or database.

The DC system software tools must allow gathering of data from various sources, i.e., various types of so-called data collector tools must already be part of the Service Provider’s DC system. The collector tools must cover a wide range of industrial and other communication protocols, e.g.: OPC DA, OPC HDA, OPC UA, OPC A&E, OLEDB, ODBC, JDBC, etc. The Service Provider must state that the specified data protocols are already present in his version of DC system, which means that the Contracting Authority’s personnel can use these protocols and modify corresponding connections based on the prescribed instructions prepared by the Service Provider.

The DC system must contain mechanisms to create backup copies of the DC system historian data and configuration data of all the DC system software tools.

The DC system must support tools necessary to configure and import the equipment hierarchy model, as defined by the ISA-95 standard [14]. This means that prescribed procedures and related tools used for data importation must already be part of the DC system. With the help of the aforementioned procedures and tools, the service provider performs the import of the entire equipment hierarchy structure from the SAP PM (Plant Maintenance) system, which is used in the DEM company to manage all maintenance activities. When changes occur in the SAP PM system, they must be maintained and updated in the DC system. The structure of the DC system must mirror the structure of the SAP PM system.

The DC system must allow creation of groups of devices that are comparable in terms of common properties, e.g., drainage HPP systems, pumps, transformers, etc.

The SAP Business system (ERP) and SAP Plant Maintenance system is the base system for the equipment hierarchy. The DC system must support connectivity with the SAP Business system (ERP) and SAP Plant Maintenance system. The Service Provider must develop procedures and corresponding DC system interfaces (i.e., APIs) that enable data exchange between the SAP ERP/PM systems and the DC system. This ensures that the Contracting Authority’s personnel can use these APIs to read and write data to and from the SAP ERP/PM systems, following the procedures established by the Service Provider.

The DC system must allow the setting of attributes that are used to describe the predictive maintenance status of the HPP components determined in the equipment hierarchy model (see point 8). These attributes are based on the operational data and are the following:

- Constants – e.g., device lifetime, number of circuit breakers trips, etc.

- Time-series data – e.g., measurement data presented with data arrays.

- Calculations – mathematical calculations performed on the constants and time-series data.

- Warnings, notifications, alarms, and records related to a device or a group of devices.

The DC system must support the use of the calculations mentioned in point 11C within a high-level programming language, i.e., in the integrated development environment (IDE), which includes functions such as:

- Logical functions – e.g.: and, if, else, and not, or then, etc.

- Standard mathematical functions – e.g.: exp, sin, cos, log, sinh, tanh, cosh, etc.

- Statistical functions – e.g.: min, max rand, standard deviation, etc.

- Text processing functions, i.e., for manipulation of the string variables.

NOTE 6.5: The IDE must encompass a broader range of functions beyond those listed above. The scope should be as much as possible comparable to the capabilities of the modern IDEs, e.g., Python, PHP, Java, C, C++, C#, Pascal, MATLAB or similar. If certain functions are not available within the existing IDE, the DC system will enable users to create them within the programming code. In other words, the functions should not be limited to basic mathematical operations, such as e.g., addition, subtraction, multiplication, division, etc.

The DC system must allow users to define triggered events and send corresponding notifications. These events and notifications are triggered if certain limits of constants or time-series data are being exceeded (e.g., number of circuit breakers trips), or when certain calculated values achieve desired values.

The DC system must allow users to change the names of the data points (i.e., OPC tags) gathered via the OPC protocol form the HPPs process systems. The system must also allow users to change corresponding tag descriptions.

The DC system must include the dashboard design environment (DDE) in the form of the web browser-based application. The DDE allows creation of custom web dashboards to visualize the HPPs real-time and predictive maintenance data. The following are the main characteristics of the DDE:

- The DDE must allow users to create own dashboards.

- The DDE must allow users to search for process data points and visualize the time-series and other process related data.

- The users must have an option to save/bookmark displays and share them with other users. The possibility of defining displays as private must also be available.

- The DDE must be supported by the most modern browsers on various operating systems for computers, tablets, and mobile phones.

- The DDE must be user friendly and simple to use, thus allowing quick and easy creation of the process dashboards. It must contain various graphical symbols (e.g., trends, tables, gauges, XY plots, different chart types, etc.) and sophisticated graphical libraries for different industries and engineering areas (e.g., electrical, mechanical, chemical, etc.).

NOTE 6.6: The DDE can also relay on external graphical libraries if these are not already available within DDE. The procedures to import external libraries must be simple and straightforward.

- The control charts used to present the time-series data must include the following functions:

- Chart editing functionality – allows users to change chart appearance, i.e., colors, line thickness, axes attributes (e.g., position, ticks, text etc.).

- Time editing functionality – e.g., for choice of the visible time range and adding/removal of the time markers.

- Legend editing functionality.

- Displaying more graphs in one control chart – it must be possible to display several graphs with different measurement quantities in the same time interval.

- The DDE must have characteristics of the modern dashboard software solutions defined in [15] and [16]. The aforementioned guidelines must be followed as much as possible.

NOTE 6.7: The Contracting Authority’s position on following the characteristics of the modern DDEs is based on the fact that nowadays most advertised APMA/APM-based solutions have already integrated DDEs that generally follow characteristics of the modern dashboard software solutions defined in [15] and [16]. The characteristics must be generally followed. If some crucial part of functionality is missing, the system must be expandable enough to easily achieve characteristics of the modern DDEs, either through the native functionality or with the Service Provider’s support within the project timeframe.

The DC system must include Excel add-in or similar feature that allows users to access data stored in the DC system historian database from the Microsoft Excel environment.

Variables calculated by the calculations specified in point 11 must be directly importable into the DDE.

The DC system must support importation of the alarm and events generated in the HPPs SCADA systems via the OPC A&E (Alarm & Events) protocol.

The system must support access to data stored in the DC system historian from programming environments such as Python and MATLAB. The Service Provider must prepare scripts in these environments to read and write data to and from the DC system historian database.

NOTE 6.8: Such OPC and OLE-DB-based scripts are already prepared for the MATLAB environment and should be adequately modified.

The DC system must be ready to expand. System must support inclusion of all the production portfolio of the HSE group into the system, it means additional hydro, solar and wind power plants. With a possibility to create own models for hydro, solar or wind power plant equipment.

General IT Requirements and cybersecurity

The following are the IT requirements for the DC system software tools and components:

- The software must be installed on state-of-the-art versions of operating systems (OS) and web browsers. These are the following:

for servers: OS Microsoft Server 2022,

for clients (workstations): OS Microsoft Windows 11 or Microsoft Server 2022,

for clients (web browsers): Edge, last available version.

Other system software – If a database or other system software is required, the following policies must be followed:

The latest safe and patched versions should be used.

The upgrades and support of these systems must be covered by the Service Provider, unless a different agreement is reached during implementation, or the software is already supported by the Contracting Authority.

Users must log into the DC system with a domain username and password – DC system must support Microsoft Active Directory authentication. SSO (Single SignOn) mechanism should be implemented for login in the system.

The DC system software tools must follow Microsoft's security recommendations, i.e., installation of new versions of software tools (e.g., web browsers, OS) and corresponding security updates and patches. The DC system should provide compatibility with new versions of operations systems, database systems, browsers, and other important software necessary to run the system. The necessary upgrades to support new versions should be provided without additional cost.

The data collection software (so-called collectors, e.g., OPC) must allow collection and storage of all process data measurements with a time resolution of one second or less.

The DC system database must allow storage and processing of the data for a longer period – the life of the HPPs units is approx. 40 years.

Basic windows domain administration rules – There are only few domain administrator accounts. These accounts are used for carrying out maintenance tasks in domain environments such as, patching, deploying new domain controllers, etc. Other tasks must be delegated to less privileged accounts in domain. Such tasks include creating low privileged accounts, creating low privileged group policies, maintaining non domain controller servers, etc.(https://learn.microsoft.com/en-us/windows-server/identity/ad-ds/plan/security-best-practices/appendix-f--securing-domain-admins-groups-in-active-directory).

- Privileged accounts MUST have “Account is sensitive and cannot be delegated” attribute set in Active Directory. Tools such as Ping Castle will issue warning if this setting is not present on sensitive accounts. Built-in groups such as “Administrators”, “Server operators”, Account Operators”, “Print Operators” are not used. Account permissions and rights are delegated only to specific systems where accounts require permissions. Custom groups are created and used for such delegations. Every individual that is going to manage the environment must have his own individual account. Generic accounts are not permitted.

Additional windows domain administration rules – In order to maintain consistency, clarity, and security within our IT infrastructure, we have established a structured naming convention for personal accounts, administrative accounts (T0, T1, T2), and service accounts. This ensures easy identification of account roles, privileges, and purposes across the system. The following rules apply to both the SamAccountName and UPN (User Principal Name):

SamAccountName refers to the account name without the domain.

UPN includes the domain as part of the account identifier.

- External Accounts that are not used on a daily basis should have an expiration date set on them. Internal password policies should be followed and implemented.

- LAPS is used for managing local (built-in) Administrator accounts on desktops and servers. Other local accounts (accounts not managed by LAPS) are NOT permitted.

- Printer Spooler MUST be disabled on domain controllers using Group Policy.

Windows desktop and server environments – There should be NO special (not a standard best practice) needs about windows desktops and servers that will be used. This will be managed according to internal security standards and policies. Every deviation from that should be solved by the Service Provider free of charge. The following are important policies:

No 3rd party software is allowed on T0 systems to prevent any supply chain attacks.

Tools such as PingCastle and Purple Knight are regularly run in the environment. Any security vulnerabilities identified by these tools are reviewed and mitigated when possible. Service Provider should solve the security vulnerabilities which are part of their software solution.

All servers and desktops are onboard to Microsoft Defender for Endpoint AV platform.

All systems in the environment (desktops and servers) must have Sysmon deployed.

All servers and desktops are onboard to SIEM.

The only supported browsers in the environment are Edge and Chrome. All browser extensions are blocked by default using Group Policy.

To prevent unauthorized usage of the USB ports on the workstations the USB ports are disabled.

Several standard applications will be installed on the machines for various purposes, mostly related to system management and system security. They are managed by the internal IT department.

The DC system must include the state-of-the-art cyber security mechanisms that comply with the ISO/IEC 27001 standard [17] or similar standards. The DC system security mechanisms used to meet the requirements of the ISO/IEC 27001 or similar standards must be described in detail by the Service Provider in the tender documentation.

If a major security vulnerability is detected in the operation of the DC system that cannot be diminished or neutralized by using the standard network equipment features in the Contracting Authority’s ownership, then it is the Service Provider responsibility to remove the deviation without additional costs to the Contracting Authority

Deliveries and Services

The Service Provider must provide the following deliveries and services:

- Project reporting – includes reports on budget, schedule, critical dependencies, obstacles, and progress

- Supply and delivery of all software tools included in the DC system.

Installation of all software tools included in the DC system.

Implementation of all the activities and technical requirements specified in this document for the introduction of the DC system.

Factory Acceptance Tests (FAT) and certificate of FAT completion.

Site Acceptance Tests (SAT) and certificate of SAT completion.

Training of the Contracting Authority’s personnel.

Preparation and delivery of the DC system technical documentation.

Supply and delivery of licenses for all the DC system software tools for a three-year period after the installation has been completed. The Service Provider must deliver 10 licenses for administration and use of the DC system and corresponding web-based tools. It is recommended to use a concurrent licensing model, as a large number of users access the system infrequently.

NOTE 8.1: After three years, the cost of a newly concluded subscription contract can only increase proportionally to the general rise in prices, if generally the functionality of the DC system remains the same.

Software maintenance and technical support (i.e., Subscription program) for a three-year period, which starts after all the DC system software tools have been installed at the DEM premises.

Software Delivery

The DC system software tools must be delivered to the Contracting Authority onto the original media. i.e., CDs, DVDs, printed materials, USB keys, etc. These software tools include the DC system application software, corresponding components, and even operating systems if they are prepared and fundamentally modified to obtain specific functionality necessary for the implementation of the DC system. Furthermore, the Service Provider must specify all third-party software solutions used in the DC system. By this it is meant that the third-party software must already be included in the DC system solution, excluding thus the possibility to charge additional costs to the Contracting Authority separately for its implementation.

The DC system software tools and corresponding services must comply with all the requirements specified in this document. The Service Provider must provide all necessary proof that their software complies with these requirements, including comprehensive technical documentation, guidelines, manuals, and other relevant materials.

Hardware Delivery

The procurement of hardware for the DC system is not part of this documentation. This documentation applies only to the DC system software tools and corresponding services. The proper hardware will be purchased, installed, and configured by the Contracting Authority, in accordance with the Service Provider's specifications and guidelines.

The Service Provider must provide specifications for the DC system servers, workstations, IT equipment (e.g., storage, firewalls etc.) and corresponding network configuration schemes, so that the Contracting Authority can plan and order the hardware on time. The Contracting Authority and Service Provider must concur on the final equipment list, which must be verified and confirmed by the Service Provider.

Testing of the Installed DC System Software

The DC system software tools are being evaluated during Factory Acceptance Test (FAT) and Customer Acceptance Test (SAT) phases – see chapter 4. These tests will evaluate and verify the following:

Compatibility of the DC software tools with the DC system requirements described in chapters 4, 5 and 6.

Validity of all data points collected from the OPC servers of the HPPs process systems – the real-time values of all data points gathered in the DC system historian database must be compared with the values of the same data points in the process systems (these systems are described in the chapters 2 and 5).

Software Maintenance and Technical support

The Service Provider is required to deliver a comprehensive software maintenance and technical support program within the three-year subscription model. This program must include, at no additional cost, all necessary upgrades and patches to support specific updates of the operating systems, database, browsers, and other software components specifically associated with the DC system. Additionally, it must encompass all security patches and upgrades required to address any discovered security vulnerabilities.

NOTE 12.1: After three years, the cost of a newly concluded subscription contract can only increase proportionally to the general rise in prices, if generally the functionality of the DC system remains the same.

The software maintenance and technical support program should generally include and define the following:

- Service Desk Website – allows users to initiate cases to report DC system errors and malfunctions via dedicated Service Desk website. This service also allows tracking of the reported cases, thus showing the current and historical status of the errors and malfunctions.

E-mail and Telephone Hotline – support during business hours.

Remote Access Network Connection – virtual private network (VPN) or similar technology that allows remote resolution of reported cases.

Remote Service Desk technical support – DC system users must have a direct access to experienced engineers at the Service Desk, who possess in-depth knowledge and expertise of the DC system software tools and corresponding hardware and network configuration.

Software changes – the Service Provider must provide the management of upgrades, updates, releases, patches, service packs or any other software modifications, used to correct errors and potential security vulnerabilities in the DC system source code. The Contracting Authority is entitled to the aforementioned software changes. The Service Provider must take timely steps to adequately evaluate and confirm validity of the software changes and, furthermore, to inform Contracting Authority’s personnel on time about the new software changes. The DC system users must be able to install simpler software changes (e.g. patches, service packs) by themselves, in accordance with the Service Provider's guidelines. If it is not feasible for the DC system user to perform such installations themselves, the Service Provider must provide assistance without additional costs to the Contracting Authority.

The service provider must describe in detail the software maintenance and technical support program in the tender documentation.

Training of the Contracting Authority’s Personnel

The Service Provider must organize dedicated training for the Contracting Authority’s personnel for the DC system operation and maintenance. The training must be provided for at least ten employees and must cover the knowledge related to the DC system software tools described in chapters 4, 5 and 6 After successful training, the Service provider will issue the Contracting Authority’s personnel with an appropriate certificate of competence. The training must be performed by the experienced Service’s Provider personnel who possess in-depth knowledge and expertise of the DC system software tools and corresponding hardware and network configuration.

The service provider must provide details on the proposed courses in the tender documentation.

Project Duration

Based on the performed technical and professional dialogues, the Contracting Authority estimates the project duration should be one year. The Service Provider must supply the estimated project plan in the tender documentation.

Warranty Period

After successful completion of the Site Acceptance Test (SAT), the DC systems will be under warranty for one year, i.e., after certificate of competition has been issued by the Service Provider.

The defects and malfunctions encountered during the warranty period, specifically related to the software bugs in the DC system software tools and components, or caused as a consequence of the poor-quality services, must be corrected by the Service Provider without additional costs to the Contracting Authority.

Literature

- ISA95, Enterprise-Control system Integration, web-based source:

- https://www.isa.org/standards-and-publications/isa-standards/isa-standards-committees/isa95

- MIMOSA OSA-CBM, web-based source:

- https://www.mimosa.org/mimosa-osa-cbm/

- Edgar Amaya Simeón, Alberto José Alvares, A data-driven framework for predicting the remaining useful life of hydroelectric equipment, ABCM Symposium Series in Mechatronics, Vol. 5, 2012.

- MIMOSA – An Operations and Maintenance Information Open System Alliance, web-based source:

- http://www.mimosa.org/?q=node/3

- Mari Cruz Garcia, Miguel A. Sanz-Bobi, Javier del Pico, SIMAP: Intelligent System for Predictive Maintenance: Application to the health condition monitoring of a wind turbine gearbox, Computers in Industry, Volume 57, Issue 6, August 2006, Pages 552–568.

- Ayser et al., Artificial Intelligence for Predictive Maintenance Applications: Key components, Trustworthiness, and Future Trends, web-based source:

- https://www.mdpi.com/2076-3417/14/2/898 .

- Mounia Achouch et al., On Predictive Maintenace in Industry 4.0: Overview, Models, and Challenges, web-based source:

- https://www.mdpi.com/2076-3417/12/16/8081 .

- Gartner: Asset Performance Manager, web-based source:

- https://www.gartner.com/reviews/market/asset-performance-management

- Haomin Chen et al, Power transformer oil temperature prediction based on empirical mode decomposition-bidirectional long short-term memory, Sge Journals, web-based source:

- https://journals.sagepub.com/doi/10.1177/17483026231176196

- Manuel J. Jimenez-Navarro et al, A new deep learning architecture with inductive bias balance for transformer oil temperature forecasting. Springer Open, web-based source:

- https://journalofbigdata.springeropen.com/articles/10.1186/s40537-023-00745-0

- Zengxu Bian et al, Oil Temperature Prediction Method Based on Deep Learning and Digital Twins, Springer Nature Link, web-based source:

- https://link.springer.com/chapter/10.1007/978-3-031-47665-5_15

- Clarify: What are data historians and do you need one, web-based source:

- https://www.clarify.io/learn/data-historian

- Parasyn: Operational Historian vs. Enterprise Historian, web-based source:

- https://www.parasyn.com.au/article/operational-historian-vs-enterprise-historian-whats-the-difference/

- ISA-95: Equipment Hierarchy Model, web-based source:

- https://www.leanvalidation.eu/index.php?option=com_content&view=category&layout=blog&id=22&Itemid=40

- Datapin: Dashboard Software Features, web-based source:

- https://www.datapine.com/articles/best-dashboard-software-features

- Thwack: All in one Modern Dashboards, web-based source:

- https://thwack.solarwinds.com/products/hybrid-cloud-observability/b/news/posts/all-in-on-modern-dashboards

- ISO 27001:2022, Information security, cybersecurity and privacy protection, web-based source:

- https://www.iso.org/standard/27001