Technical Specifications for Public Procurement: Microgrid Experimental System

Project name: Upgrading of National Research Infrastructures 2 - RIUM 2

Tender reference:  302/1-RIUM2-FERI-8/26  »Microgrid experimental system«

SUBJECT OF THE PUBLIC PROCUREMENT

Subject of the public procurement: is the purchase of a microgrid experimental system, including transport and delivery, in accordance with the technical specifications.

Delivery deadline: maximum 70 days from the start of the validity of the contract.

Warranty and maintenance: at least two (2) years from the signing of the acceptance report.

The tenderer must ensure for the equipment (at no additional cost to the contracting authority):

- the equipment, as derived from the technical specifications, must be new, packaged and delivered to the required premises of the user;

- that all offered equipment is compatible and mutually interconnectable with the existing equipment – the dynamic test machine;

- that it is possible to connect all required equipment into a single, flawlessly functioning system (also with the existing equipment), in accordance with the technical specifications;

- that it will remedy all errors that would arise due to possible mutual incompatibility, non-interconnectivity or inadequate connection into a single, flawlessly functioning system;

- that it will deliver all equipment according to the specification as a complete set.

For all equipment, the offered price must also include:

- appropriate transport of the equipment from the supplier to the place of installation, transport insurance, unloading, and placement at the final location of the University of Maribor, Faculty of Electrical Engineering and Computer Science,

- technical documentation of the manufacturer in electronic and physical form for hardware and software.

The offer must include all small materials necessary for installation and commissioning, even if not explicitly stated in the technical requirements.

The price of the equipment must be given in euros and include all elements of which it is composed (equipment by items and accompanying services, any discounts, transport costs, labour costs, delivery and assembly, installation costs, insurance, training costs, instructions, etc.) or all costs associated with the execution of the subject public procurement.

TECHNICAL REQUIREMENTS

The Microgrid experimental system consists of several components (items), for which the tender documentation also uses the common term “equipment” or “Microgrid experimental system”.

The offer must include all components that will enable the realization of all the contracting authority’s requirements.

Tenderers must submit an offer that includes all items of equipment required in the minimum technical requirements. The contracting authority will not consider offers that do not offer all items of equipment and will reject such offers as inadmissible.

The following are the minimum technical requirements for the equipment. The offer must meet or exceed all minimum technical requirements.

| Item No. | Qty | REQUIRED |
| --- | --- | --- |
| 1.1 | 4 | Controller: - At least 24 ADC inputs (20MSPS) - ≥ 24 PWM optical outputs - FPGA core - ≥ Quad-core ARM with frequency ≥ 1.5 GHz |
| 1.2 | 4 | Code generation software for Simulink |
| 1.1 | 4 | Filter set: - ≥ 32 A ≥ 2.2 mH inductors (one per SiC half bridge) - ≥ 10 µF filtering capapacitors - Rack-mounted housing |
| 1.4 | 28 | SiC Half-bridges: - ≥ 32 A - SiC technology - Phase-leg topology - ≥ 800 V - ≥ 200 kHz |
| 1.5 | 4 | Set for connection to the electrical grid: - 3-phase installation circuit breaker 16 A - 2x 3-phase ≥ 25 A relays - Pre-charge resistors |
| 1.6 | 16 | Analog high bandwidth voltage sensor: - ≥ 500 V - ≥ 2MHz - Isolation ≥ 1kVrms - DIN-rail mounting |
| 1.7 | 12 | Analog isolated voltage sensor: - ≥ 1000 V - ≥ 100 kHz - Working voltage (basic isolation) ≥ 1.5kVrms - DIN-rail mounting |
| 1.8 | 24 | Analog high bandwidth current sensor: - ≥ 25 A - ≥ 1MHz - Working voltage (basic isolation) ≥ 1kVrms - DIN-rail mounting |
| 1.9 | 4 | Connecting cable set and integration: - Coax cables - Optical cables - RJ45 analog cables - Safety banana cables - 19’ cabinet integration and pre-wiring |
| 1.10 | 2 | Bidirectional DC source: - ≥ 800 V - ≥ 50 A - ≥ 12kW |
| 1.11 | 2 | Bidirectional DC source: - ≥ 80 V - ≥ 150 A - ≥ 5 kW |
| 1.12 | 1 | Real-time simulator: - AMD Kintex™-7 410T or better - ≥ 16 GB RAM - ≥ 1 TB NVMe SSD drive - ≥ 6 PWM - ≥ 4 SFP, ≥ 1 Gbit/s - 2 PCIe - RS485 - RS422 - USB C 3.2 - HDMI - Rx/Tx FO - Rx/Tx LVDS 2× 16-channel I/O modules |
| 1.13 | 1 | Delivery, installation and startup: - delivery of equipment to address Koroška cesta 46, 2000 Maribor, |
| 1.14 | 1 | Delivery deadline: - Delivery no later than 70 days from the day the contract enters into force. |
| 1.15 | 1 | Education/training: - About the machine/equipment and the participants’ work with it. |
| 1.16 | 1 | Warranty period: - 24 months. |

The operation is co-financed by the Republic of Slovenia, the Ministry of Education, Science and Youth, and the European Union from the European Regional Development Fund. The operation is implemented within the framework of the European Cohesion Policy Programme for the period 2021–2027 in Slovenia, under Policy Objective 1: A more competitive and smarter Slovenia by promoting innovative and smart economic transformation and regional ICT connectivity, under Priority 1: Innovative knowledge society, under Specific Objective RSO1.1: Developing and enhancing research and innovation capacities and the uptake of advanced technologies in the area of strengthening research capacities.